3GPP TSG-RAN WG4 Meeting #95-e	R4-2007769

# Online, 20th May – 06th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on L1-SINR measurement accuracy requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, the L1-SINR accuracy requirements need to be introduced for NR eMIMO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the L1-SINR accuracy requirements with the following secnarios: - CSI-RS based CMR with no dedicated IMR configured - SSB based CMR with dedicated ZP-IMR configured - CSI-RS based CMR with dedicated ZP-IMR configured - SSB based CMR with dedicated NZP-IMR configured - CSI-RS based CMR with dedicated NZP-IMR configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The L1-SINR accuracy requirements will be missing for NR eMIMO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23, 10.1.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.23	L1-SINR accuracy requirements for FR1

#### 10.1.23.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured

##### 10.1.23.1.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all CSI-RS resources of the serving cell configured for L1-SINR measurement with no dedicated IMR configured in FR1.

The accuracy requirements in Table 10.1.23.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.23.1.1-1.

Table 10.1.23.1.1-1: CSI-RS only based L1-SINR absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| ±3.5 | ±4.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C |  | -120 |  | -117 |  | -114 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  | -119.5 |  | -116.5 |  | -113.5 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  | -119 |  | -116 |  | -113 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F |  | -118.5 |  | -115.5 |  | -112.5 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G |  | -118 |  | -115 |  | -112 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H |  | -117.5 |  | -114.5 |  | -111.5 |  | N/A |  | -50 |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for CSI-RS Ês/Iot ≤ 25 dB. |  |  |  |  |  |  |  |  |

#### 10.1.23.2	L1-SINR accuracy requirements with SSB based CMR and dedicated ZP-IMR configured

##### 10.1.23.2.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all SSBs of the serving cell configured for L1-SINR measurement with dedicated ZP-IMR configured in FR1.

The accuracy requirements in Table 10.1.23.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.23.2.1-1: SSB + ZP-IMR based L1-SINR absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±3.5 | ±4.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C |  | -120 |  | -117 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  | -119.5 |  | -116.5 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  | -119 |  | -116 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F |  | -118.5 |  | -115.5 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G |  | -118 |  | -115 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H |  | -117.5 |  | -114.5 |  | N/A |  | -50 |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. |  |  |  |  |  |  |  |

#### 10.1.23.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated ZP-IMR configured

##### 10.1.23.3.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all CSI-RS resources of the serving cell configured for L1-SINR measurement with dedicated ZP-IMR configured in FR1.

The accuracy requirements in Table 10.1.23.3.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.23.3.1-1.

Table 10.1.23.3.1-1: CSI-RS + ZP-IMR based L1-SINR absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| ±3.5 | ±4.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C |  | -120 |  | -117 |  | -114 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  | -119.5 |  | -116.5 |  | -113.5 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  | -119 |  | -116 |  | -113 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F |  | -118.5 |  | -115.5 |  | -112.5 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G |  | -118 |  | -115 |  | -112 |  | N/A |  | -50 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H |  | -117.5 |  | -114.5 |  | -111.5 |  | N/A |  | -50 |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for CSI-RS Ês/Iot ≤ 25 dB. |  |  |  |  |  |  |  |  |

#### 10.1.23.4	L1-SINR accuracy requirements with SSB based CMR and dedicated NZP-IMR configured

##### 10.1.23.4.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all SSBs of the serving cell configured for L1-SINR measurement with dedicated NZP-IMR configured in FR1.

The accuracy requirements in Table 10.1.23.4.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.x for a corresponding Band for each relevant SSB and each relevant CSI-RS.

Table 10.1.23.4.1-1: SSB + NZP-IMR based L1-SINR absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB/CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| ±3.0 | ±4.0 | 0 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -120 | -117 | -114 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB and CSI-RS Ês/Iot ≤ 25 dB and SSB Ês/Iot= CSI-RS Ês/Iot. |  |  |  |  |  |  |  |  |  |  |

#### 10.1.23.5	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated NZP-IMR configured

##### 10.1.23.5.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all CSI-RS resources of the serving cell configured for L1-SINR measurement with dedicated NZP-IMR configured in FR1.

The accuracy requirements in Table 10.1.23.5.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.x for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.23.2.1-1.

Table 10.1.23.5.1-1: CSI-RS + NZP-IMR based L1-SINR absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| ±3.0 | ±4.0 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The requirements apply for CSI-RS Ês/Iot ≤ 25 dB and the Ês/Iot of CSI-RS configured as CMR is equal to theÊs/Iot of CSI-RS configured as IMR. |  |  |  |  |  |  |  |  |

10.1.24	L1-SINR accuracy requirements for FR2

#### 10.1.24.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured

##### 10.1.24.1.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all CSI-RS resources of the serving cell configured for L1-SINR measurement with no dedicated IMR configured in FR2.

The accuracy requirements in Table 10.1.24.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.24.1.1-1.

Table 10.1.24.1.1-1: CSI-RS only based L1-SINR absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±3.5 | ±4.5 | ≥-3 | Same value as CSI-RS_RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤ 25 dB. |  |  |  |  |  |  |

#### 10.1.24.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured

##### 10.1.24.2.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all SSBs of the serving cell configured for L1-SINR measurement with dedicated ZP-IMR configured in FR2.

The accuracy requirements in Table 10.1.24.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.24.2.1-1: SSB + ZP-IMR based L1-SINR absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±3.5 | ±4.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. |  |  |  |  |  |  |

#### 10.1.24.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated ZP-IMR configured

##### 10.1.24.3.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all CSI-RS resources of the serving cell configured for L1-SINR measurement with dedicated ZP-IMR configured in FR2.

The accuracy requirements in Table 10.1.24.3.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.24.3.1-1.

Table 10.1.24.3.1-1: CSI-RS + ZP-IMR L1-SINR absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±3.5 | ±4.5 | ≥-3 | Same value as CSI-RS_RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤ 25 dB. |  |  |  |  |  |  |

#### 10.1.24.4	L1-SINR accuracy requirements with SSB based CMR and dedicated NZP-IMR configured

##### 10.1.24.4.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all SSBs of the serving cell configured for L1-SINR measurement with dedicated NZP-IMR configured in FR2.

The accuracy requirements in Table 10.1.24.4.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.x for a corresponding Band for each relevant SSB and each relevant CSI-RS.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.24.4.1-1: SSB + NZP-IMR based L1-SINR absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB/CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | Minimum Io |  |  |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±3.0 | ±4.0 | ≥0 | Same value as SSB_RP in Table B.2.4.x-2, according to UE Power class, operating band and angle of arrival |  | Same value as CSI-RS_RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB and CSI-RS Ês/Iot ≤ 25 dB and SSB Ês/Iot= CSI-RS Ês/Iot. |  |  |  |  |  |  |  |  |

#### 10.1.24.5	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated NZP-IMR configured

##### 10.1.24.5.1	L1-SINR Absolute Accuracy

Unless otherwise specified, the requirements for L1-SINR absolute accuracy in this clause apply to all CSI-RS resources of the serving cell configured for L1-SINR measurement with dedicated NZP-IMR configured in FR2.

The accuracy requirements in Table 10.1.24.5.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.4.x for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.24.5.1-1.

Table 10.1.24.5.1-1: CSI-RS + NZP-IMR based L1-SINR absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |  |
| ±3.0 | ±4.0 | ≥0 | Same value as CSI-RS_RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤ 25 dB and the Ês/Iot of CSI-RS configured as CMR is equal to theÊs/Iot of CSI-RS configured as IMR. |  |  |  |  |  |  |

<End of Change 1>