3GPP TSG-RAN4 Meeting #95-e	R4-2007844

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0846 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for measurement requriements for PRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurement period requirements for PRS-RSRP need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define measurement period requirements for PRS-RSRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement period requirements for PRS-RSRP are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.8.3	DL PRS-RSRP measurements

The requirements in this clause apply if UE is requested to provide NR DL-AoD location measurements via LPP [31].

All requirements specified in this clause shall apply without DRX as well as for any DRX cycles specified in TS 38.331 [2]. All requirements specified in this clause shall apply for intra-frequency and inter-frequency PRS layers,

When the physical layer cell identities of neighbour cells together with the NR-DL-AoD Assistance Data are provided, the UE shall be able to detect and measure PRS-RSRP, specified in TS 38.215 [4], for all the PRS resources associated with all the TRPs and on all the PRS frequency layers according to the reported UE capabilities, including the reference resource, within TPRS-RSRP ms as given below:

TPRS-RSRP= NRxBeam*M/S*maxTres,Tproc*4+Tproc,

where

- NRxBeam=8 is the scaling factor for Rx beam sweeping,

- Tres is the maximum resource periodicity among all PRS resources on the PRS frequency layer,

- Tproc is the PRS processing time as indicated in UE capability reporting,

- M is the maximum number of PRS resources per slot, and S is the reported capability on the maximum of PRS resources UE can buffer and process per slot,

- The requirements apply provided that the configured PRS duration per occasion K is no larger than the reported capability on the maximum duration of PRS symbols N UE can buffer and process every Tprocms.

Editor’s Note: Requriements for multiple PRS layers are FFS

The UE physical layer shall be capable of reporting PRS-RSRP for the reference cell and all the neighbor cells within  TPRS-RSRP provided the PRS Ês/Iot for the reference resource and neighbour resources, as defined in clause (TBD: PRS-RSRP accuracy sections) are met for at least half of PRS occasions within TPRS-RSRP,

The time TPRS-RSRP starts from the first subframe of the PRS positioning occasion closest in time after both the NR-DL-AoD-RequestLocationInformation and the assistance data in the NR-DL-AoD-ProvideAssistanceData message as specified in TS 37.355 [31], are delivered to the physical layer of the UE.

The PRS-RSRP measurement accuracy for all measured resources shall be fulfilled according to the accuracy as specified in the clause (TBD: PRS-RSRP accuracy sections).

Editor’s Note: Requriements for HO case are FFS

<End of Change 1>