3GPP TSG-RAN4 Meeting #95-e	R4-2007865

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0852 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI-RS measurement capability - number of cells and beams |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CSI-RS measurement capability requirements needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify CSI-RS measurement cpability requirements in number of cells and beams. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.8.2.2, 9.8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.8	CSI-RS L3 measurements

9.8.1	Introduction

TBA

9.8.2	Intra-frequency measurements

9.8.2.1	Introduction

TBA

9.8.2.2	Requirements applicability

TBA

9.8.2.3	Number of cells and number of CSI-RS resources

9.8.2.3.1	Requirements for FR1

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	8 identified cells for which the associated SSB have been detected, and

-	32 CSI-RS resources with different resource index and/or PCI.

9.8.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	6 identified cells for which the associated SSB have been detected, and

-	32 CSI-RS resources with different resource index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band and UE is configured to report CSI-RS based measurements for PCC; or

-	PSCC when UE is configured with EN-DC with PSCC in the band and UE is configured to report CSI-RS based measurements for PSCC; or

-	One of the SCCs on which UE is configured to report CSI-RS based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with CSI-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least 2 CSI-RS resources on serving cell for each of the other intra-frequency layer(s) in the same band.

9.8.2.4	Measurement Reporting Requirements

TBA

9.8.2.5	Intra-frequency measurements without measurement gaps

TBA

9.8.2.6	Intra-frequency measurements with measurement gaps

TBA

<End of Change 1>

<Start of Change 2>

9.8.3	Inter-frequency measurements

9.8.3.1	Introduction

TBA

9.8.3.2	Requirements applicability

TBA

9.8.3.3	Number of cells and number of CSI-RS resources

9.8.3.3.1	Requirements for FR1

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	4 identified cells for which the associated SSB have been detected, and

-	24 CSI-RS resources with different resource index and/or PCI.

9.8.3.3.2	Requirements for FR2

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	4 identified cells for which the associated SSB have been detected, and

-	24 SSBs with different SSB index and/or PCI on the inter-frequency layer, and

9.8.3.4	Measurement Reporting Requirements

TBA

9.8.3.5	Inter-frequency measurements without measurement gaps

TBA

9.8.3.6	Inter-frequency measurements with measurement gaps

TBA

<End of Change 2>