3GPP TSG-RAN4 Meeting #95-e	R4-2007874

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6890 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRM requirements for PUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 agreed to define UE measurement requirements for RSRP change based TA validation. This is a new measruement requirement for UE in idle mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UE measurement requirements for RSRP change based TA validation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSRP change based TA validation may not work due to outdated RSRP measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 4.7.XX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.7.XX	Measurement for RSRP change based TA validation

The requirements in this clause apply for UE configured with PUR, and serving cell RSRP change as TA validation mechanism.

UE shall compare the RSRP level of serving cell which is obtained no earlier than T ms before the PUR where UE will transmit, with the reference RSRP of the serving cell for TA validation as specified in [Ref TBD], and T is

- min(480ms, N*DRX cycle[ms]), when UE is in normal coverage, where N is the relaxation factor as defined in clause 4.7.2.1.1A

- min(800ms, N*DRX cycle[ms]), when UE is in enhanced coverage, where N is the relaxation factor as defined in clause 4.7.2.2.1A

UE shall be able to correct validate the TA provided the difference between the measured RSRP and the reference RSRP is larger than 4dB when UE is in normal coverage and 5dB when UE is in enhanced coverage.

<End of Change 1>