3GPP TSG-RAN WG4 Meeting #95-e	R4-2007893

# Electronic Meeting, 25 May – 05 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0854 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurement requirements for UEs under power saving mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2020-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support for relaxed measurement requirements for UEs under power saving mdoe is introduced in release 16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note: This CR needs to be revised based on the agreements reached during the meeting. Change #1: Intra-frequency requirements for UEs configured with measurement relaxation criterion Change #2: Inter-frequency requirements for UEs configured with measurement relaxation criterion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs under power saving mode won’t be able to fulfill the requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause introduced: 4.2.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

#### 4.2.2.8 	Measurements of intra-frequency NR cells for UEs configured with relaxed measurement criterion

##### 4.2.2.8.1 	Introduction

This section contains the requirements for measurements on intra-frequency NR cells when configured with any of following relaxed measurement criterion:

- Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

- Relaxed measurement criterion for UE not at cell edge defined in clause 5.2.4.X.2 in [1],

- Both low mobility criterion and not at cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

##### 4.2.2.8.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- UE is configured with low mobility criterion, or

- UE is configured with both low mobility criterion and not at cell edge criterion, and

- UE has fulfilled only the low mobility criterion and

- T331 timer is not running for EMR measurements on intra-frequency NR carrier.

The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.8.1-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.8.1-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.8.1-1.

Table 4.2.2.8.1-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 4, which is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

##### 4.2.2.8.2 	Measurements for UEs configured with not at cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- UE is configured with not at cell edge criterion, or

- UE is configured with both low mobility criterion and not at cell edge criterion, and

- UE has fulfilled only the not at cell edge criterion and

- T331 timer is not running for EMR measurements on intra-frequency NR carrier.

The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.8.2-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.8.2-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.8.2-1.

Table 4.2.2.8.2-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K2 (36 x N1 x M2 x K2) | 1.28 x N1 x M2 x K2 (4 x N1 x M2 x K2) | 5.12 x N1 x M2 x K2 (16 x N1 x M2 x K2) |
| 0.64 |  | 5 | 17.92 x N1 x K2 (28 x N1 x K2) | 1.28 x N1 x K2 (2 x N1 x K2) | 5.12 x N1 x K2 (8 x N1 x K2) |
| 1.28 |  | 4 | 32 x N1 x K2 (25 x N1 x K2) | 1.28 x N1 x K2 (1 x N1 x K2) | 6.4 x N1 x K2 (5 x N1 x K2) |
| 2.56 |  | 3 | 58.88 x N1 x K2 (23 x N1 x K2) | 2.56 x N1 x K2 (1 x N1 x K2) | 7.68 x N1 x K2 (3 x N1 x K2) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K2 = 2, which is the measurement relaxation factor applicable for UE fulfilling the not at cell edge criterion. |  |  |  |  |  |

##### 4.2.2.8.3 	Measurements for UEs configured with low mobility and not at cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- UE is configured with both low mobility criterion and not at cell edge criterion, and

- UE has fulfilled both low mobility and not at cell edge criterion and

- T331 timer is not running for EMR measurements on intra-frequency NR carrier.

The UE’s intra-frequency measurement is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1.

##### 4.2.2.8.4 	Minimum requirements at transitions

Upon transitioning from fulfilling the requirements for low mobility criterion defined in clause 4.2.2.8.1 to those for both low mobility and not at cell edge criterion defined in clause 4.2.2.8.3:

- the UE shall fulfil the requirements defined in clause 4.2.2.8.1 for at least N DRX cycles from the moment the UE has fulfilled the criterion for both low mobility and not at cell edge criterion.

Upon transitioning from fulfilling the requirements for not at cell edge criterion defined in clause 4.2.2.8.2 to those for both low mobility and not at cell edge criterion defined in clause 4.2.2.8.3:

- the UE shall fulfil the requirements defined in clause 4.2.2.8.2 for at least N DRX cycles from the moment the UE has fulfilled the criterion for both low mobility and not at cell edge criterion.

Upon transitioning from fulfilling the requirements for both low mobility criterion and not at cell edge criterion defined in clause 4.2.2.8.3 to those for low mobility defined in clause 4.2.2.8.1:

- the UE shall fulfil the requirements defined in clause 4.2.2.8.1 directly from the moment the UE has fulfilled the criterion for low mobility criterion.

Upon transitioning from fulfilling the requirements for both low mobility criterion and not at cell edge criterion defined in clause 4.2.2.8.3 to those for not at cell edge defined in clause 4.2.2.8.2:

- the UE shall fulfil the requirements defined in clause 4.2.2.8.2 directly from the moment the UE has fulfilled the criterion for low mobility criterion.

Upon transitioning from fulfilling the requirements defined in clause 4.2.2.3 to

- those defined for low mobility defined in clause 4.2.2.8.1, or

- to those defined for not at cell edge defined in clause 4.2.2.8.2, or

- to those defined for both low mobility and not at cell edge defined in clause 4.2.2.8.3,

the UE shall fulfil the requirements defined in clause 4.2.2.3 for at least N DRX cycles from the moment the UE has fulfilled the criterion for low mobility, not at cell edge criterion or both low mobility and not at cell edge.

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2 ------------------------------------------------------------

#### 4.2.2.9 	Measurements of inter-frequency NR cells for UEs configured with relaxed measurement criterion

##### 4.2.2.9.1 	Introduction

This section contains the requirements for measurements on inter-frequency NR cells when configured with any of following relaxed measurement criterion:

- Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

- Relaxed measurement criterion for UE not at cell edge defined in clause 5.2.4.X.2 in [1],

- Both low mobility criterion and not at cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

##### 4.2.2.9.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

- UE is configured with low mobility criterion, or

- UE is configured with both low mobility criterion and not at cell edge criterion, and

- UE has fulfilled only the low mobility criterion and

- T331 timer is not running for EMR measurements on intra-frequency NR carrier.

The requirements defined in clause 4.2.2.4 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.2-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.2-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.2-1.

Table 4.2.2.9.2-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 4, which is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

##### 4.2.2.9.2 	Measurements for UEs configured with not at cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- UE is configured with not at cell edge criterion, or

- UE is configured with both low mobility criterion and not at cell edge criterion, and

- UE has fulfilled only the not at cell edge criterion and

- T331 timer is not running for EMR measurements on intra-frequency NR carrier.

The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.8.2-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.8.2-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.8.2-1.

Table 4.2.2.9.2-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K2 (36 x N1 x 1.5 x K2) | 1.28 x N1 x 1.5 x K2 (4 x N1 x 1.5 x K2) | 5.12 x N1 x 1.5 x K2 (16 x N1 x 1.5 x K2) |
| 0.64 |  | 5 | 17.92x N1 x K2 (28 x N1 x K2) | 1.28 x N1 x K2 (2 x N1 x K2) | 5.12 x N1 x K2 (8 x N1 x K2) |
| 1.28 |  | 4 | 32 x N1 x K2 (25 x N1 x K2) | 1.28 x N1 x K2 (1 x N1 x K2) | 6.4 x N1 x K2 (5 x N1 x K2) |
| 2.56 |  | 3 | 58.88 x N1 x K2 (23 x N1 x K2) | 2.56 x N1 x K2 (1 x N1 x K2) | 7.68 x N1 x K2 (3 x N1 x K2) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K2 = 2, which is the measurement relaxation factor applicable for UE fulfilling the not at cell edge criterion. |  |  |  |  |  |

##### 4.2.2.9.3 	Measurements for UEs configured with low mobility and not at cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- UE is configured with both low mobility criterion and not at cell edge criterion, and

- UE has fulfilled both low mobility and not at cell edge criterion and

- T331 timer is not running for EMR measurements on intra-frequency NR carrier.

The UE’s inter-frequency measurement is not required to meet Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter as defined in Table 4.2.2.3-1.

##### 4.2.2.9.4 	Minimum requirements at transitions

The requirements defined in clause 4.2.2.8.4 apply also for this section.

------------------------------------------------------------- End of change 2 ------------------------------------------------------------