3GPP TSG-RAN4 Meeting #95-e 	R4-2007919

# Electronic Meeting, May 25 – June 05, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6902 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of requirements for preconfigured uplink resource transmission for cat-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support for transmission using preconfigured uplink resources (PUR) in IDLE state is introduced in release 16. This CR contains the PUR RRM requirements as agreed in earlier way forwards [R4-1907733, R4-1910107, R4-1912735, R4-1915889]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: PUR requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements when UE is transmitting using PUR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section: 4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

### 4.7.3	Requirements for transmission using preconfigured uplink resources for UE category M1

#### 4.7.3.1	Introduction

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation using pur-RSRP-ChangeThreshold for transmitting in uplink using preconfigured uplink resources (PUR) as specified in [TS 36.331].

#### 4.7.3.2	Requirements on UE synchronization for transmission using PUR

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage.

The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. Otherwise, the UE shall postpone the transmission until the next PUR transmission occasion which occurs immediately after the UE has obtained the synchronization towards serving cell within Tsearch, where:

- Tsearch is the time required by the UE for synchronizing towards the serving cell:

  - Tsearch = 5 ms when the UE is not configured with eDRX in normal and enhanced coverage

  - Tsearch = 960 ms when the UE is configured with eDRX in normal coverage

  - Tsearch = 320 s when the UE is configured with eDRX in enhanced coverage

The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331].

If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission.

#### 4.7.3.3	Requirements on TA validation for transmission using PUR

When only rsrp-ChangeThresh [TS 36.331] is configured, the UE is allowed to transmit using PUR using the timing derived using the latest available NTA value as specified in subclause 7.24.1 provided that

- the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

- timing alignment validation for transmission using PUR is valid according to the valdation criteria in section 5.3.3.x in [TS 36.331]

RSRP1 is considered valid provided that the following condition is met when in normal coverage:

(T1 – min(480 ms, N DRX cycle)) ≤  T1’ ≤  (T1 + min(480 ms, NDRX cycle))

RSRP1 is considered valid provided that the following condition is met when in enhanced coverage:

(T1 – min(800 ms, N DRX cycle)) ≤  T1’ ≤  (T1 + min(800 ms, NDRX cycle))

RSRP2 is considered valid provided that the following condition is met when in normal coverage:

T2 – min(480 ms, NDRX cycle) ≤ T2’ ≤ T2

RSRP2 is considered valid provided that the following condition is met when in enhanced coverage:

T2 – min(800 ms, NDRX cycle) ≤ T2’ ≤ T2

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions then the UE shall not validate the PUR using RSRP1 and RSRP2 and shall not transmit using PUR.

Where

-	T1 is the time when the latest NTAwas obtained by the UE for transmission on PUR,

-	T1’ is the time when the UE has completed RSRP1,

-	T2 is the time when the UE perfoms TA validation  defined in the present subclause for transmission using PUR,

-	T2’ is the time when the UE has completed RSRP2.

-	N is the relaxation factor and is given by Table 4.7.2.1.1A-1 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7.2.1.1A-2 if the UE is configured with eDRX_IDLE cycle in normal coverage.

-	N is the relaxation factor and is given by Table 4.7.2.2.1A-1 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7.2.2.1A-2 if the UE is configured with eDRX_IDLE cycle in enhanced coverage.

< END OF CHANGE #1 >