3GPP TSG RAN WG4 Meeting #95-e	R4-2007992

Electronic Meeting, May 25-June 05, 2020

Agenda Item:	6.1.3.2

Source: 	Ericsson

Title:	Handling 4 SMTC in RRC re-establishment and redirection for IAB-MT

Document for:	Discussion

- Introduction

In RAN4#94-ebis, text proposals on RRC re-direction and RRC release with redirection requirements for IAB-MT to TS 38.174 were approved. However, according to the agreements editor’s notes to address SMTC window requirements were added which need to be resolved.

In RRC re-direction requirements for IAB-MT (clause 12.1.1.1.2.1) the following editor’s notes exist:

[Editor’s note 1: The description regarding TSMTC  and TSMTC,i will be added after RAN4 finalizes the requirements regarding the number of SMTC windows (see editor’s note 2)]

[Editor’s Note 2: Requirement that the IAB-MT can be configured up to 4 SMTC windows can be introduced after the IAB-MT capability discussion for this feature is finalized by RAN1. Details of such requirements are FFS.]

In RRC release with redirection requirements for IAB-MT (clause 12.1.1.1.2.1) the following editor’s note exists [1]:

“[Editor’s note: The description regarding the relationship between Trs and SMTC configuration will be added after RAN4 finalizes the requirements regarding the number of SMTC windows]”.

RAN4 has now received RAN1 feature lists via LS to RAN4 [2], which also provides the latest RAN1 agreements on RAN1 feature list [3]. In this paper we address how to address SMTC window requirements in the RRC connection re-establishment and redirection requirements in view of the last RAN1 agreements [3].

# Analysis of handling SMTC windows

According to the RAN1 NR feature list, feature index 20-2 defines the IAB-MT support for up to 4 SMTCs as quoted below (only relevant parts are reproduced for brevity) [3]:

| Features | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 20. NR_IAB |  |  |  |  |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities | Optional with capability signalling. [Devices supporting IAB backhaul must report this FG as supported] |

According to the above agreements the support of up to 4 SMTCs is optional for IAB-MT. This means not every IAB-MT is required to support 4 SMTCs. Therefore, all IAB-MT should meet requirements for SMTC1 and SMTC2 defined in TS 38.133 (like for the UE). However, only IAB-MT which is capable of 4 SMTCs is required to meet requirements for 4 SMTCs.

Based on the above arguments the current editor’s notes are removed in RRC re-establishment requirements and the TSMTC and TSMTC,i are defined.

Also based on the above arguments the current editor’s note is removed in RRC release with redirection requirements and the relation between Trs and TSMTC is defined.

- Summary

In this paper we have analysed the open issues related to the editor’s notes to address SMTC window requirements for IAB-MT. Following are the proposals based on the analysis:

- Observation # 1: According to RAN1 NR feature list support of up to 4 SMTCs is optional for IAB-MT.

- Proposal # 1: All IAB-MT shall meet requirements for SMTC1 and SMTC2 defined in TS 38.331. However, only IAB-MT which is capable of 4 SMTCs is required to meet requirements for 4 SMTCs.

- Proposal # 2: Only IAB-MT, which is capable of up to 4 SMTC configurations, is required to meet corresponding requirements for 4 SMTCs.

TP defining TSMTC and TSMTC,i in RRC re-establishment requirements is provided in [4].

TP defining relation between Trs and TSMTC in RRC release with redirection requirements is provided in [5].

# References

- R4-2005318, WF on NR IAB RRM requirements, Qualcomm

- R1-2003070, LS on updated Rel-16 RAN1 UE features lists for LTE, RAN1 LS to RAN4

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T, NTT DOCOMO

- R4-2007993, TP to TS 38.174 v0.0.1: Updates to RRC re-establishment requirements for IAB MT, Ericsson

- R4-2007994, TP to TS 38.174 v0.0.1: Updates to RRC release with redirection requirements for IAB MT, Ericsson