3GPP TSG-RAN4 Meeting #95e	R4-2008052

# May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: Introduction of inter-band CA for FR2 into TS 38.101-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh |  |  |  |  |  | Date: |  |  | 2020-5-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introducion of feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of bands and configurations Addition of peak EIS and spherical coverage, max input level, ACS and in-band blocking |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.2A.2, 7.3A.3, 7.4A, 7.5A, 7.6A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes to TS38.101-2 >

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

∆EIRPBC	The beam correspondence tolerance, where ∆EIRPBC = EIRP2 – EIRP1

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔRB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB	Allowed reference sensitivity relaxation for inter-band CA operation

ΔRIB,SPH	Allowed EIS spherical coverage requirement relaxation for a band for in inter-band CA operation

ΔMBP,n	Allowed relaxation to each, minimum peak EIRP and reference sensitivity due to support for multi-band operation, per band in a combination of supported bands

ΔMBS,n	Allowed relaxation to each, EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, per band in a combination of supported bands

∑MBP	Total allowed relaxation to each, minimum peak EIRP and reference sensitivity due to support for multi-band operation, for all bands in a combination of supported bands

< end of changes to TS38.101-2 >

< start of next changes to TS38.101-2 >

### 5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2-1 where all operating bands are within FR2.

Table 5.2A.2-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n260-n261 | n260, n261 |

< end of changes to TS38.101-2 >

< start of next changes to TS38.101-2 >

#### 5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: NR CA configurations for inter-band CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum Aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_n260A-n261A | - | n260 | 60 | Yes | Yes | Yes | Yes | 800 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260A-n261G | CA_n261G | n260 | 60 | Yes | Yes | Yes | Yes | 600 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260A-n261H | CA_n261G CA_n261H | n260 | 60 | Yes | Yes | Yes | Yes | 700 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260A-n261I | CA_n261G CA_n261H CA_n261I | n260 | 60 | Yes | Yes | Yes | Yes | 800 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260A-n261J | CA_n261G CA_n261H CA_n261I CA_n261J | n260 | 60 | Yes | Yes | Yes | Yes | 900 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260A-n261K | CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | 60 | Yes | Yes | Yes | Yes | 1000 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260A-n261L | CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | 60 | Yes | Yes | Yes | Yes | 1100 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260A-n261M | CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | 60 | Yes | Yes | Yes | Yes | 1200 | 0 |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261A | CA_n260G | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 600 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260G-n261G | CA_n260G CA_n261G | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 400 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261H | CA_n260G CA_n261G CA_n261H | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 500 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261I | CA_n260G CA_n261G CA_n261H CA_n261I | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 600 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261J | CA_n260G CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 700 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261K | CA_n260G CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 800 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261L | CA_n260G CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260G-n261M | CA_n260G CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261A | CA_n260G CA_n260H | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 700 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260H-n261G | CA_n260G CA_n260H CA_n261G | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 500 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261H | CA_n260G CA_n260H CA_n261G CA_n261H | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 600 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261I | CA_n260G CA_n260H CA_n261G CA_n261H CA_n261I | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 700 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261J | CA_n260G CA_n260H CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 800 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261K | CA_n260G CA_n260H CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261L | CA_n260G CA_n260H CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260H-n261M | CA_n260G CA_n260H CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261A | CA_n260G CA_n260H CA_n260I | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 800 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260I-n261G | CA_n260G CA_n260H CA_n260I CA_n261G | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 600 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261H | CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 700 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261I | CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H CA_n261I | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 800 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261J | CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261K | CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261L | CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260I-n261M | CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1200 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261A | CA_n260G CA_n260H CA_n260I CA_n260J | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260J-n261G | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 700 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261H | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 800 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261I | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H CA_n261I | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261J | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261K | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261L | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1200 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260J-n261M | CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1300 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261A | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260K-n261G | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 800 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261H | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261I | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H CA_n261I | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261J | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261K | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1200 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261L | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1300 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260K-n261M | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1400 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261A | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260L-n261G | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 900 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261H | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261I | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H CA_n261I | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261J | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1200 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261K | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1300 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261L | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1400 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260L-n261M | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1500 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261A | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1200 | 0 |
|  |  | n261 | 60 | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 120 | Yes | Yes | Yes | Yes |  |  |
| CA_n260M-n261G | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1000 | 0 |
|  |  | n261 | See CA_n261G Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261H | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1100 | 0 |
|  |  | n261 | See CA_n261H Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261I | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H CA_n261I | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1200 | 0 |
|  |  | n261 | See CA_n261I Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261J | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H CA_n261I CA_n261J | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1300 | 0 |
|  |  | n261 | See CA_n261J Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261K | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1400 | 0 |
|  |  | n261 | See CA_n261K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261L | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1500 | 0 |
|  |  | n261 | See CA_n261L Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |
| CA_n260M-n261M | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | See CA_n260K Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  | 1600 | 0 |
|  |  | n261 | See CA_n261M Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |

< end of changes to TS38.101-2 >

< start of next changes to TS38.101-2 >

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

##### 7.3A.2.3.1	General

For inter-band carrier aggregation with uplink assigned to one band, the requirement applies per operating band, with one component carrier per operating band. The inter-band requirement applies with common AoA for all active component carriers. The throughput for each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity for each carrier specified in section 7.3.2 and is modified by ∆RIB. The requirement on each component carrier applies when the power in the component carrier in other band correspond its EIS spherical coverage requirements specified in sub clause 7.3.4. Reference sensitivity requirement is increased by value of ∆RIB is when UE is configured for inter-band CA.

The requirement is verified with the test metric of EIS (Link= RX Beam peak direction, Meas=Link Angle).

##### 7.3A.2.3.2	Reference sensitivity for power class 1

##### For power class 1, the value of ∆RIB for configuration with one operating band belonging to the band group 1 and the other band belonging to the band group 2 is defined in Table 7.3A.2.3.2-1.

Table 7.3A.2.3.3-1: ΔRIB reference sensitivity relaxation for inter-band CA for power class 1

| Band group 1 | Band group 2 | ΔRIB (dB) |
| --- | --- | --- |
| n261 | n260 | TBD |

##### 7.3A.2.3.3	Void

##### 7.3A.2.3.4	Reference sensitivity for power class 3

For power class 3, the value of ∆RIB with one operating band belonging to the band group 1 and the other band belonging to the band group 2 is defined in Table 7.3A.2.3.4-1.

Table 7.3A.2.3.4-1: ΔRIB reference sensitivity relaxation for inter-band CA for power class 3

| Band group 1 | Band group 2 | ΔRIB (dB) |
| --- | --- | --- |
| n261 | n260 | 1.5 |

< end of changes to TS38.101-2 >

< start of next changes to TS38.101-2 >

### 7.3A.3	EIS spherical coverage for CA

#### 7.3A.3.1	Void

#### 7.3A.3.2	Void

##### 7.3A.3.3  	EIS spherical coverage for inter-band CA

##### 7.3A.3.3.1	General

For inter-band carrier aggregation with uplink assigned to one band, the requirement applies per operating band, with one component carrier per operating band. The inter-band CA requirement applies with common AoA for all active component carriers. The requirement on each component carrier applies when the power in the component carriers in the other band correspond to its respective EIS spherical coverage requirement levels for single carrier with relaxation for inter-band CA for that AoA and test condition.

The inter-band CA spherical coverage requirement will be satisfied if the intersection set of spherical coverage areas exceeds the requirement. Intersection set of spherical coverage areas is defined as a fraction of area of full sphere measured around the UE where both bands meet their defined individual inter-band CA EIS spherical coverage requirement.

The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

The reference measurement channels and throughput criterion shall be as specified in clause 7.3A.2.3 for corresponding power class. The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in clause 7.3.2 for the corresponding power class.

Unless otherwise specified, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1) configured.

##### 7.3A.3.3.2	EIS spherical coverage for inter-band CA for power class 1

Required fraction of area as defined in clause 7.3A.3.3.1 for power class 1 is TBD %. The required spherical coverage EIS for each band is given in clause 7.3.4.1 and modified by ΔRIB,SPH. The value of ∆RIB,SPH for configurations with one operating band belonging to the band group 1 and the other band belonging to the band group 2 is defined in Table 7.3A.2.3.2-1.

Table 7.3A.2.3.2-1: ΔRIB,SPH EIS spherical coverage requirement relaxation for inter-band CA for power class 1

| Band group 1 | Band group 2 | ΔRIB (dB) |
| --- | --- | --- |
| n261 | n260 | TBD |

##### 7.3A.3.3.3	EIS spherical coverage for inter-band CA for power class 2

##### 7.3A.3.3.4	EIS spherical coverage for inter-band CA for power class 3

Required fraction of area as defined in clause 7.3A.3.3.1 for power class 3 is 50 %. The required spherical coverage EIS for each band is given in clause 7.3.4.3 and modified by ΔRIB,SPH. The value of ∆RIB,SPH for configurations with one operating band belonging to the band group 1 and the other band belonging to the band group 2 is defined in Table 7.3A.2.3.4-1

Table 7.3A.2.3.4-1: ΔRIB,SPH EIS spherical coverage requirement relaxation for inter-band CA for power class 3

| Band group 1 | Band group 2 | ΔRIB (dB) |
| --- | --- | --- |
| n261 | n260 | 1.5 |

< end changes to TS38.101-2 >

< start of next changes to TS38.101-2 >

## 7.4A	Maximum input level for CA

For intra-band carrier aggregation the input level is defined as the cumulative received power, summed over the transmission bandwidth configurations of each active DL CC. All DL CCs shall be active throughout the test. The input power shall be distributed among the active DL CCs so their PSDs are aligned with each other. At the maximum input level, the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel over each component carrier. The minimum requirement is specified in Table 7.4A-1.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.4A-1: Maximum input level for CA

| Rx Parameter | Units | All CA configurations included in BCS 0 |
| --- | --- | --- |
| Power summed over transmission bandwidth configurations of all active DL CCs | dBm | -25 (NOTE 2) |
| NOTE 1: The transmitter shall be set to 4 dB below the lower limit of the PUMAX,f,c inequality defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2 NOTE 2: Reference measurement channel in each CC is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |

Table 7.4A-2: Void

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in clause 7.4 for each component carrier while all downlink carriers are active.

< end of changes to TS38.101-2 >

## 7.5A	Adjacent channel selectivity for CA

For intra-band contiguous carrier aggregation, the SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.5.1A-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1). The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.5A-1: Adjacent channel selectivity for CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n257, n258, n261 | dB | 23 |
| n260 | dB | 22 |

Table 7.5A-2: Adjacent channel selectivity test parameters for CA, Case 1

| Rx Parameter | Units | All CA bandwidth Classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| PInterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| PInterferer for band n260 | dBm | Aggregated power + 20.5 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | + BWchannel CA / - BWchannel CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. |  |  |

Table 7.5A-3: Adjacent channel selectivity test parameters for CA, Case 2

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n257, n258, n261 | dBm | - 46.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n260 | dBm | - 45.5 |
| Pinterferer | dBm | - 25 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | + BWchannel CA / - BWchannel CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. |  |  |

For intra-band non-contiguous carrier aggregation with two component carriers, two different requirements apply for out-of-gap and in-gap. For out-of-gap, the UE shall meet the requirements for each component carrier as specified in clauses 7.5. For in-gap, the requirement applies if the following minimum gap condition is met:

∆fACS∆fACS ≥ BW1/2 + BW2/2 + max(BW1, BW2),

where ∆fACS ∆fACS is the frequency separation between the center frequencies of the component carriers and BWk/2 are the channel bandwidths of carrier k, k = 1,2.

If the minimum gap condition is met, the UE shall meet the requirements specified in clauses 7.5 for each component carrier considered. The respective channel bandwidth of the component carrier under test will be used in the parameter calculations of the requirement. In case of more than two component carriers, the minimum gap condition is computed for any pair of adjacent component carriers following the same approach as the two component carriers. The in-gap requirement for the corresponding pairs shall apply if the minimum gap condition is met.

For every component carrier to which the requirements apply, the UE shall meet the requirement with one active interferer signal (in-gap or out-of-gap) while all downlink carriers are active.

## For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in clause 7.5 for each component carrier while all downlink carriers are active.

< end changes to TS38.101-2 >

< start of next changes to TS38.101-2 >

### 7.6A.2	In-band blocking

For intra-band contiguous carrier aggregation, the SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6A.2-1 for in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel and an interferer power shall not exceed -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1). The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.6A.2-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260 | dBm | Aggregated power + 20.5 |
| BWInterferer | MHz | BWChannel_CA |
| FIoffset | MHz | +2*BWChannel_CA / -2*BWChannel_CA NOTE 5 |
| FInterferer | MHz | FDL_low + 0.5*BWChannel_CA To FDL_high - 0.5*BWChannel_CA |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Table 7.3.2-1. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 QPSK, R=1/3 with one sided dynamic OCNG pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: The FInterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |

For intra-band non-contiguous carrier aggregation with two component carriers, the requirement applies to out-of-gap and in-gap. For out-of-gap, the UE shall meet the requirements for each component carrier with parameters as specified in 7.6.2-1. The requirement associated to the maximum channel between across the component carriers is selected. For in-gap, the requirement shall apply if the following minimum gap condition is met:

∆fIBB ≥ 0.5(BW1 + BW2) + 2 max(BW1, BW2),

where ∆fIBB is the frequency separation between the center frequencies of the component carriers and BWk/2 are the channel bandwidths of carrier k, k = 1,2.

If the minimum gap condition is met, the UE shall meet the requirement specified in Table 7.6.2-1 for each component carrier. The respective channel bandwidth of the component carrier under test will be used in the parameter calculations of the requirement.  In case of more than two component carriers, the minimum gap condition is computed for any pair of adjacent component carriers following the same approach as the two component carriers. The in-gap requirement for the corresponding pairs shall apply if the minimum gap condition is met. For every component carrier to which the requirements apply, the UE shall meet the requirement with one active interferer signal (in-gap or out-of-gap) while all downlink carriers are active.

Table 7.6A.2-2: (Void)

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the in-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in clause 7.6.2 for each component carrier while all downlink carriers are active.