3GPP TSG-RAN WG4 Meeting # 95-e                                  R4-2008080

Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.886 to update MPR AMPR requirements for both PC3 and PC2 NR V2X in band n47

Agenda Item:	6.4.3.1

Document for:	Approval

Introduction

In this paper, it’s proposed to update the specific MPR\AMPR requirements for both PC3 and PC2 NR V2X in band n47 based on simulation results.

- References

[1] R4-2005739, TR38.886 v0.6.0: TR update for NR V2X service with SL operation, LG

- Text Proposal

<TP for TR 38.886 v0.6.0>

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[6]	R4-1910400, “TP on adjacent channel co-existence scenarios and parameters for NR V2X service on FR1”

[7]	R4-1911489, “ [V2X] NR V2X coexistence simulation results for ITS spectrum case1 and case3”

[8]	R4-1911490, “ [V2X] NR V2X coexistence simulation results for ITS spectrum case2 and case4”

[9]	R4-1910402, “Initial coexistence simulation results of NR V2X for ITS band”

[10]	R4-1912473, “NR V2X RF Coex Simulation results”

[11]	R4-1911491, “[V2X] NR V2X coexistence simulation results for licensed band”

[12]	R4-1912865, “Coexistence simulation results in licensed bands”

[xx]	3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".

[xx2]	ETSI TS 102 792: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range".

<Next of change>

8.1.2	UE maximum output power reduction

The following assumption can serve as a baseline for MPR simulation assumptions as shown in Table 8.1.2-1 and Table 8.1.2-2.

Table 8.1.2-1: MPR simulation assumption for NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 2.7GHz/5.9GHz |
| Bandwidth | 10/20/30/40MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

For NR V2X, simultaneous transmission of PSCCH and PSSCH in the same subframe is supported. The following constraints in table 8.1.2-2 can be assumed based on current RAN1’s agreement.

Table 8.1.2-2: MPR simulation assumption based on RAN1’s agreement

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | • Support {10, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,15,20,25,30,40,45,50,60,70,75,80,90,100,105,110,120,130,135,140,150,160,165,170,175,180,190,195,200,210 |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH, RAN4 assumed as follow

- The baseline MPR simulation assumptions for multiple PSFCH transmission

  - 1 RB per user

  - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed

    - MPR will be derived by non-contiguous PSFCH RB allocation

  - Total power of all users equals 23dBm

  - All users have the same power per RB

  - Simulation parameters as per Table 8.1.2-1 and Table 8.1.2-2

  - Feedback sequences are per Section 5.2 TS 38.211

  - The MPR will be specified as a formula or table based on number of allocated RBs.

  - PSFCH generation shall be based on existing RAN1 agreements

  - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision

Based on the simulation, RAN4 specify MPR requirements as follow

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 8.1-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for NR V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission.

8.1.2.1	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 8.1.2.1-1 for power class 3.

Table 8.1.2.1-1: Maximum Power Reduction (MPR) for power class 3 NR V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ [4.0] | ≤ [2.0] |
|  | 16QAM | ≤ [4.0] | ≤ [2.0] |
|  | 64 QAM | ≤ [4.5] |  |
|  | 256 QAM | ≤ [7.0] |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in TS38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC3 NR V2X UE will be specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

MA_PSFCH = 7.5   ;  0 ≤ NGap / NRB < 0.55

= 12  ;  0.55 ≤ NGap / NRB ≤ 1

Where

NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart)

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for V2X physical channel S-SSB shall be specified in table 8.1.2.1-2.

Table 8.1.2.1-2 S-SSB Maximum power reduction (MPR) for PC3 NR V2X UE

| Channel | MPR_S-SSB (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| S-SSB | ≤ 3.5 | ≤ 1.5 |

8.1.2.2	MPR for Power class 2 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 8.1.2.2-1 for power class 2.

Table 8.1.2.2-1: Maximum Power Reduction (MPR) for power class 2 NR V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 |
|  | 64 QAM | ≤ 4 |  |

Outer and Inner RB allocations is defined in clause 8.1.2.1.

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC2 NR V2X UE will be specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

MA_PSFCH = 8   ;  0 ≤ NGap / NRB < 0.4

= 9.5  ;  0.4 ≤ NGap / NRB < 0.55

= 14  ;  0.55 ≤ NGap / NRB ≤ 1

Where

NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart)

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for V2X physical channels S-SSB shall be specified in table 8.1.2.2-2.

Table 8.1.2.2-2 S-SSB Maximum power reduction (MPR) for PC3 NR V2X UE

| Channel | MPR_S-SSB (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| S-SSB | ≤ 4 | ≤ 2 |

8.1.3	UE maximum output power with additional requirements

To comply the EN 302 571 standard  emission mask, RAN4 need to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [] | 100 kHz |
| 0.5-5 | [] | 100 kHz |
| 5-10 | [] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC had regulatory requirements for 40MHz in ITS spectrum as shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply the regional regulatory requirements, RAN4 specifiy the A-MPR requirements according to different network signalling by pre-configured radio parameters or the cell and the indication from upper layers has indicated to NR V2X UE.

8.1.3.1	General for NR V2X UE AMPR

Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [xx].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 8.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 8.1.2. Outer and inner allocation notation used in clause 8.1.3 is defined in clause 8.1.2. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 8.1.3.1-1: Additional Maximum Power Reduction (A-MPR) for NR V2X UE

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Clause 8.1.3.2 |  |
| NS_yy | Table 8.1.3-3 (A-SEM) | n47 | 40 | Clause 8.1.3.3 |  |

8.1.3.2	AMPR for NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase which is specified for PSCCH and PSSCH transmission\S-SSB\PSFCH below is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in 38.101-1.

Table 8.1.3.2-1: A-MPR regions for PC3 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPRbase |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 | 5855~5865 | ≤ 1.8 | < 7.2 | A1 |
|  |  | ≤ 1.8 | ≥ 7.2 | A2 |
|  |  | > 1.8 && ≤ 2.88 | ≤ 2.7 | A3 |
|  |  | > 1.8 && ≤ 2.88 | > 2.7 | A4 |
|  |  | > 2.88 | ≤ 2.7 | A5 |
|  |  | > 2.88 | > 2.7 | A6 |
|  | 5865~5875, 5875~5885, 5885~5895, 5895~5905, 5905~5915, 5915~5925 | N/A | N/A | A7 |

Table 8.1.3.2-2: A-MPR for PC3 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Inner |
| CP-OFDM | QPSK | [16] | [13] | [8] | [10] | [8] | [8] | [3.5] | [2] |
|  | 16 QAM | [16] | [13] | [8] | [10] | [8] | [8] | [3.5] | [2] |
|  | 64 QAM | [16] | [13] | [8] | [10] | [8] | [8] | [3.5] | [2] |
|  | 256QAM | [16] | [13] | [8] | [10] | [8] | [8] | [3.5] | [2] |

Table 8.1.3.2-3: A-MPR for PC3 NR V2X UE (S-SSB)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | RBstart*12*SCS MHz | S-SSB A-MPRbase (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Outer | Inner |
| 10 | 5855~5865 | ≤ 1.8 | [15.5] |  |
|  |  | > 1.8 && ≤ 3.6 | [7.5] |  |
|  |  | > 3.6 && ≤ 5.22 | [5] |  |
|  |  | > 5.22 | [9.5] |  |
|  | 5865~5875, 5875~5885, 5885~5895, 5895~5905, 5905~5915, 5915~5925 | N/A | [3.5] | [2] |

Table 8.1.3.2-4: A-MPR for PC3 NR V2X UE (PSFCH)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | PSFCH A-MPRbase (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |
| 10 | 5855~5865 | [18.5] |  |  |
|  | 5865~5875, 5875~5885, 5885~5895, 5895~5905, 5905~5915, 5915~5925 | [14] | [7] | [18.5] |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |

Table 8.1.3.2-5: A-MPR regions for PC2 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPRbase |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 | 5855~5865 | < (LCRB*12*SCS)/2+0.72 | > 0 && ≤ NRB*12*SCS | A1 |
|  |  | ≥ (LCRB*12*SCS)/2+0.72 && < 2.88 | > 0 && ≤ 4.32 | A2 |
|  |  | ≥ 2.88 && ≤ 4.5 | ≤ 2.7 | A3 |
|  |  | > 4.5 | ≤ 2.7 | A4 |
|  |  | ≥ 2.88 | > 2.7 && ≤ 2*(RBstart*12*SCS – 0.72) | A5 |
| 10 | 5865~5875, 5875~5885, 5885~5895, 5895~5905, 5905~5915, 5915~5925 | N/A |  | A6 |

Table 8.1.3.2-6: A-MPR for PC2 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer | Inner |  |
| CP-OFDM | QPSK |  | [18] |  | [11] |  | [6] |  | [14] |  | [11.5] |  | 3.5 | 2 |  |
|  | 16 QAM |  | [18] |  | [11] |  | [6] |  | [14] |  | [11.5] |  | 3.5 | 2 |  |
|  | 64 QAM |  | [18] |  | [11] |  | [6] |  | [14] |  | [11.5] |  | 3.5 | 2 |  |
|  | 256QAM |  | [18] |  | [11] |  | [6] |  | [14] |  | [11.5] |  | 3.5 | 2 |  |

Table 8.1.3.2-7: A-MPR for PC2 NR V2X UE (S-SSB)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | RBstart*12*SCS MHz | S-SSB A-MPRbase (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Outer | Inner |
| 10 | 5855~5865 | ≤ 1.8 | 17.5 |  |
|  |  | > 1.8 && ≤ 3.6 | 9 |  |
|  |  | > 3.6 && ≤ 5.04 | 6.5 |  |
|  |  | > 5.04 | 13 |  |
| 10 | 5865~5875, 5875~5885, 5885~5895, 5895~5905, 5905~5915, 5915~5925 | N/A | [4] | [2.5] |

Table 8.1.3.2-8: A-MPR for PC2 NR V2X UE (PSFCH)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | PSFCH A-MPRbase (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 ≤ NGap / NRB < 0.2 | 0.2≤ NGap / NRB < 0.4 | 0.4≤ NGap / NRB ≤ 1 |
| 10 | 5855~5865 | [20.5] |  |  |
|  | 5865~5875, 5875~5885, 5885~5895, 5895~5905, 5905~5915, 5915~5925 | 16.5 | 12 | 20 |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |

8.1.3.3	AMPR for NS_yy

Table 8.1.3.3-1: A-MPR regions for PC3 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 40 | 5865~5905 | < - (LCRB*12*SCS)/4+12.6 | ≤14.4 | A1 |
|  |  | ≥ - (LCRB*12*SCS)/4+12.6 && ≤ - (LCRB*12*SCS)/2+25.2 | ≤14.4 | A2 |
|  |  | > - (LCRB*12*SCS)/2+25.2 | ≤14.4 | A3 |
|  |  | ≥ 0 | >14.4 | A4 |

Table 8.1.3.3-2: A-MPR regions for PC3 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| CP-OFDM | QPSK | [15] | [5] | [15] | [7.5] |
|  | 16 QAM | [15] | [5] | [15] | [7.5] |
|  | 64 QAM | [15] | [5] | [15] | [7.5] |
|  | 256QAM | [15] | [5] | [15] | [7.5] |

Table 8.1.3.3-3: A-MPR for PC3 NR V2X UE (S-SSB)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | RBstart*12*SCS MHz | S-SSB A-MPR (dB) |
| --- | --- | --- | --- |
|  |  |  | Outer/Inner |
| 40 | 5865~5905 | ≤ 5.94 | 15 |
|  |  | > 5.94 && ≤ 9 | 9.5 |
|  |  | > 9 && ≤ 12.6 | 5.5 |
|  |  | >12.6 && < 24.48 | 2 |
|  |  | ≥ 24.48&& < 27.36 | 5.5 |
|  |  | ≥ 27.36&& < 30.6 | 9.5 |
|  |  | ≥ 30.6 | 15 |

Table 8.1.3.3-4: A-MPR for PC3 NR V2X UE (PSFCH)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | PSFCH A-MPR (dB) |
| --- | --- | --- |
| 40 | 5865~5905 | [23.5] |

Table 8.1.3.3-5: A-MPR regions for PC2 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 40 | 5865~5905 | < - (LCRB*12*SCS)/4+12.6 | ≤14.4 | A1 |
|  |  | ≥ - (LCRB*12*SCS)/4+12.6 && ≤ - (LCRB*12*SCS)/2+25.2 | ≤14.4 | A2 |
|  |  | > - (LCRB*12*SCS)/2+25.2 | ≤14.4 | A3 |
|  |  | ≥ 0 | >14.4 | A4 |

Table 8.1.3.3-6: A-MPR for PC2 NR V2X UE (Contiguous PSCCH and PSSCH transmission)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| CP-OFDM | QPSK | [17] | [5.5] | [17] | [9] |
|  | 16 QAM | [17] | [5.5] | [17] | [9] |
|  | 64 QAM | [17] | [5.5] | [17] | [9] |
|  | 256QAM | [17] | [5.5] | [17] | [9] |

Table 8.1.3.3-7: A-MPR for PC2 NR V2X UE (S-SSB)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | RBstart*12*SCS MHz | S-SSB A-MPR (dB) |
| --- | --- | --- | --- |
|  |  |  | Outer/Inner |
| 40 | 5865~5905 | ≤ 5.94 | 17.5 |
|  |  | > 5.94 && ≤ 9 | 13 |
|  |  | > 9 && ≤ 12.6 | 10.5 |
|  |  | >12.6 && < 24.48 | 4 |
|  |  | ≥ 24.48&& < 27.36 | 10.5 |
|  |  | ≥ 27.36&& < 30.6 | 13 |
|  |  | ≥ 30.6 | 17.5 |

Table 8.1.3.3-8: A-MPR for PC2 NR V2X UE (PSFCH)

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | PSFCH A-MPR [dB] |
| --- | --- | --- |
| 40 | 5865~5905 | 25.5 |

8.1.4	Configured transmitted power for NR V2X UE

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c – TC,c,  (PPowerClass – ΔPPowerClass) –– MAX(MAX(MPRc , A-MPRc)+ ΔTIB,c + TC,c + ∆TRxSRS, P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c,  (PPowerClass– ΔPPowerClass),  PRegulatory,c }

where

-	PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

- For the total transmitted power PCMAX,PSSCH\PSCCH, PCMAX,S-SSB and PCMAX,PSFCH , PEMAX,c is the value given by IE maxTxPower, defined by [TS 38.331], when the UE is not associated with a serving cell on the NR V2X carrier .

- PPowerClass is the maximum UE power specified in Table 6.2.1-1 in TS38.101-1 without taking into account the tolerance specified in the Table 6.2.1-1 in TS38.101-1;

-	 MPRc and A-MPRc for serving cell c are specified in subclause 6.2.2 and subclause 6.2.3 in TS38.101-1 for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	TIB,c, TC,c, ∆TRxSRS, PPoweclass and P-MPRc are specified in subclause 6.2.4 in TS38.101-1.

- PRegulatory,c= 10 - Gpost connector dBm the V2X UE is within the protected zone [xx2] of CEN DSRC tolling system and operating in Band n47; PRegulatory,c= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in subclause 6.2.4 in TS38.101-1 shall be applied.

<End of TP >