3GPP TSG-RAN4 WG4 Meeting # 95-e	R4-2008157

# Electronic meeting, 25 May- 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0195 | rev | - | Current version: | 15.9.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for reference maximum transmission power for maxUplinkdutyCycle-FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WF R4-2002733 was approved in RAN4#94e meeting which agrees on clarifying the reference maximum transmission power for maxUplinkdutyCycle-FR2 indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the reference maximum transmission power for maxUplinkdutyCycle-FR2 indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference output power for UE capability maxUplinkdutyCycle-FR2 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR’s revision history: |  |
| --- | --- |

<Start of change>

6.2.4	Configured transmitted power

The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PPowerclass – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

with PPowerclass the UE power class as specified in sub-clause 6.2.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c as specified in sub-clause 6.2.2 , A-MPRf,c as specified in sub-clause 6.2.3, ΔMBP,n the peak EIRP relaxation as specified in clause 6.2.1 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1.

maxUplinkDutyCycle-FR2, as defined in TS 38.306 [14], is a UE capability to facilitate electromagnetic power density exposure requirements. This UE capability is applicable to all FR2 power classes, and is only valid for the reference maximum configured transmitted power PCMAX,ref, where PCMAX,ref is the PCMAX,f,c without using MPR, AMPR and PMPR.

If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c.

If the field of UE capability maxUplinkDutyCycle-FR2 is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means.

P-MPRf,c is the allowed maximum output power reduction. The UE shall apply P-MPRf,c for carrier f of serving cell c only for the cases described below. For UE conformance testing P-MPRf,c shall be 0 dB.

a)	ensuring compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic power density exposure requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

NOTE 1:	P-MPRf,c  was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRf,c and maxUplinkDutyCycle-FR2 may impact the maximum uplink performance for the selected UL transmission path.

The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4-1.

Table 6.2.4-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3.1 |  |  |

<End of change>