3GPP TSG-RAN WG4 Meeting # 95-e	R4-2008185

# Electronic Meeting, 25 May – 06 June, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	On Release 15 and 16 two Intra-band Transmit Chain Cases

Agenda Item:	6.11.1.2	Uplink Tx Full Power transmission [NR_eMIMO-Core]

Document for:	Discussion

# Introduction

In RAN4#94-Bis e-meeting, two Way Forwards [1, 2] addresses a long standing issue in RAN4 requirements and associated tests and signaling for the multiple implementations options using 2 transmit chain. To ensure the same type of issue doesn’t arise again this contribution looks into all the different intra-band 2Tx cases in Release 16 for examination.

# Discussion

## Disclaimer: This paper is trying to list all the Release 15 and 16 two intra-band transmit chain cases and their implementation options. It may not always capture all the case and the signaling and requirement status accurately, but the idea is to understand if there are missing aspects in the specification, test or signaling.

## Release 15 two Intra-band Transmit Cases

In release 15, the use of two intra-band transmit chains in has been confined to UEs supporting UL MIMO or intra-band TDD ENDC, Table 1 tries to collect all the cases that have been discussed in release 15.

Table 1: list of two intra-band transmit cases for Release 15

| Driving Feature | Implementation | Mode Potentially affected | Requirement status | Signaling |
| --- | --- | --- | --- | --- |
| PC2 UL MIMO | 2x23dBm PA | one layer operation using TX Diversity | MPR in UL MIMO and one layer mode TBD | 2TX signaling for single port mode? |
| PC2 UL MIMO | 2x26dBm PA | none | One layer PC2 MPR can apply to all modes | SA Power class + UL MIMO |
| PC2 UL MIMO | 26dBm PA + 23dBm PA | none | MPR in UL MIMO mode TBD | SA Power class + UL MIMO |
| TDD PC2 intra band ENDC | 2x23dBm PA | UL MIMO support in ENDC mode | No UL MIMO in ENDC mode MPR and AMPR cover all cases | 2TX + LTE/NR/ENDC power class |
| TDD PC2 intra band ENDC | 2x26dBm PA |  |  |  |
| TDD intra band ENDC | 1xPA | applied MPR in 1TX case, SUO may apply in some cases | no MPR in spec for single PA | LTE/NR/ENDC power class |
| FDD PC3 intra band ENDC | 2x23dBm PA or 1x23dBm PA |  |  | 2TX or SUO(1TX) + LTE/NR/ENDC power class |
| intra band ENDC | 1 PA case | ENDC mode not evaluated, SUO may apply in some cases | no MPR in spec for single PA | SUO(1TX) in some cases |

Observation:

- Most of issues that are tackled in [1] relate to Tx Diversity support and MPR for different PC2 UL MIMO implementations

- Intra-band ENDC support in release 15 is focused to very specific cases and 1TX case is not well supported.

## Release 16 two Intra-band Transmit Cases

In release 16, the use of two intra-band transmit chains is spreading to multiple cases and affect almost all new Release 16 UL features and Table 2 tries to collect those together with potential need for new requirements and new signaling.

Table 2: list of two intra-band transmit cases for Release 16

| Driving Feature | Implementation | Mode Potentially affected | Requirement status | Signaling |
| --- | --- | --- | --- | --- |
| PC2 UL MIMO | 2x23dBm PA | one layer operation using TX Diversity | MPR in UL MIMO and one layer mode TBD | 2TX signaling for single port mode? |
| PC2 UL MIMO | 2x26dBm PA | none | One layer PC2 MPR can apply to all modes | SA Power class + UL MIMO |
| PC2 UL MIMO | 26dBm PA + 23dBm PA | none | MPR in UL MIMO mode TBD | SA Power class + UL MIMO |
| TDD PC3 intra band ENDC | 2x23dBm PA | UL MIMO support in ENDC mode | No UL MIMO in ENDC mode MPR and AMPR cover all cases | 2TX + LTE/NR/ENDC power class + modified MPR bit |
| TDD PC2 intra band ENDC | 2x23dBm PA |  |  |  |
| TDD PC2 intra band ENDC | 2x26dBm PA |  |  |  |
| TDD 29dBm intra band ENDC | 2x26dBm PA |  |  |  |
| intra band ENDC | 1 PA case | ENDC mode not evaluated, SUO may apply in some cases | reuse of NR UL CA at least for contiguous case? | SUO(1TX) in some cases |
| PC3 intra band contiguous NR UL CA | 1x23dBm 2x23dBm | 2TX cases uses 1TX MPR (large margin) UL MIMO cannot be supported simultaneously | MPR for 1TX case | 2TX when applicable implies no MIMO 4TX? For UL CA + UL MIMO? |
| PC3 intra band contiguous NR UL CA | 1x23dBm 2x23dBm | UL MIMO cannot be supported simultaneously if 2TX 1TX fallback option for some BW | MPR for 2TX case FFS for 1TX | 2TX when applicable implies no MIMO 4TX? For UL CA + UL MIMO? 1TX/2TX vs BW class? |
| PC3 NRU | 2x20dBm PA | Tx Diversity requirement | PC3 ACLR not agreed | 2TX w/wo UL MIMO |
| NR V2X | 2xPA | Tx Diversity mode | ? | ? |

Observations:

- NR-V2X data is missing due to lack of time to recover it

- There are cases with 3 different power classes (P2, PC3, 29dBm) and 3 PA capabilities (20, 23 and 26dBm)

- There is potentially one case with 4TX: non-contiguous intra-band UL CA with simultaneous UL MIMO

- Many cases rely on well-defined Tx Diversity requirement and test.

- Even in release 16 some default 1TX cases may not have quantified requirements for EN DC or UL CA

  - Some of these cases may have to rely on SUO.

We encourage companies to look at Table 2, complete and amend it as needed to ensure that we identify required capability signaling and requirement gaps for release 16.

# Conclusions

In this contribution, we tried to collect as many intra-band 2TX cases as possible for Release 15 and 16 and we make the following suggestion:

We encourage companies to look at Table 2, complete and amend it as needed to ensure that we identify possible capability signaling and requirement gaps for release 16.

# References

[1] R4-2005216, WF on Power Class related UL MIMO and other requirements, Vivo, RAN4#94Bis-e

[2] R4-2005652, WF on Uplink Full Power Transmission, Samsung, RAN4#94Bis-e