3GPP TSG-RAN WG4 Meeting #95-e                 	R4-2008188

Online, May 20th – June 5th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on CGI reading requirements with autonomous gap

Agenda item:	6.15.1.3

Document for:	Discussion

# 1	Introduction

In RAN4#94e-bis meeting, the way forward for CGI reading requirements were agreed in [1].

In this contribution we will continue to discuss and provide our views on the remaining open issues in CGI reading requirements with autonomous gap for NR capable UE.

2	Discussion

## 2.1	known / unknown cell

In RAN4#93 and RAN4#94-e meeting, we discussed CGI reading requirements with autonomous gap in known and unknown cell. It was agreed that we will not define the requirements for unknown cell, however, how to consider a cell is known need further discussion.

- known cell conditions - FFS if known cell conditions is needed based on agreements that SSB selection is up to UE implementation. - FFS how to specify known cell conditions if necessary.

In RAN4#94e-bis meeting, it is agreed that SSB selection procedure is up to UE implementation. When UE receives reportCGI, the target cell should have successfully measured and reported, it means that the SSB(s) which triggered the valid measurement report for the target cell should remain detectable when UE starts to handle CGI report. How to guarantee the target cell has been detected and identified, specification should have the relevant definition. From our point of view, the definition of cell known condition should be needed to guarantee the CGI reading procedure. We can use the similar definition way as the cell known condition in NR handover.

known cell condition for CGI reading with autonomous gap could be:

In FR1 and FR2, the target cell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the report CGI command,

- the UE has sent a valid L3-RSRP measurement report for the target cell and

- One of the SSBs measured from the target cell remains detectable according to the cell identification conditions specified in clause 9.3.

- One of the SSBs measured from the target cell remains detectable during CGI reading delay according to the cell identification conditions specified in clause 9.3.

2.2	SIB1 reading delay

In RAN4#94e-bis meeting, how the SIB1 decoding is derived and the delay was discussed, further input should be taken into account on the GTW discussion.

As we discussed in [2], for the SIB acquisition time, the simulation results show that the agreed SIB1 acquisition success rate is obtained assuming soft combining of 4 repetitions around at -6dB SNR. For the case of AWGN, the simulation shows similar result. Hence, the SIB1 acquisition time should be 4 * SMTC period. As assuming beam sweeping is not needed for SIB1 decoding in CGI reading, SIB1 decoding delay in FR2 should be same as in FR1.

SIB1 decoding delay should be 4 * SMTC period in FR1 and FR2

2.3	CGI reading requirements

In last meeting, we had discussed the delay requirements for CGI reading of NR cell. The open issues summarized in the way forward [1].

- MIB decoding delay for FR2 - Option 1 : [5] * TSMTC, where TSMTC is SMTC periodicity of target cell. - Option 2 : [5] * N * TSMTC, where N = 8 and TSMTC is SMTC periodicity of target cell. - Note: Depending on outcome of if RX beam sweeping is needed for MIB decoding.

As the agreement in RAN4#93 meeting, CGI reading of NR cell will be defined for known NR cell. the reported SSB should remain detectable during CGI reading since the target NR cell is known, UE should decode MIB directly with this SSB index. From this point of view, beam sweeping is not needed for MIB decoding.

Beam sweeping is not needed for MIB decoding.

We assume beam sweeping is not needed for MIB decoding according to the analysis, UE should decode MIB directly with the SSB index which remains detectable during CGI reading, no scaling factor is needed for MIB decoding delay in FR2.

MIB decoding delay in FR2 is [5] * SMTC period.

Based on the analysis, CGI reading delay of NR cell should be include MIB decoding delay and SIB1 decoding delay.

MIB decoding delay could be [5] * SMTC period for FR1 and FR2. As assuming beam sweeping is not needed for SIB1 decoding, SIB1 decoding delay in FR1 and FR2 should be same. According to our simulation result, SIB1 decoding delay could be 4 * SMTC period.

CGI reading delay of NR cell should be [9] * SMTC period

2.4	T321 timer

RAN2 has sent LS [3] to RAN4 for the T321 timer value of autonomous gap. T321 timer is defined in 38.331 as below,

Timer; Start; Stop; At expiry. T321; Upon receiving measConfig including a reportConfig with the purpose set to reportCGI; Upon acquiring the information needed to set all fields of cgi-info, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportCGI and upon detecting that a cell is not broadcasting SIB1.; Initiate the measurement reporting procedure, stop performing the related measurements..

According to the definition,  T321 timer value should be the CGI reading delay with autonomous gap.

T321 timer value of autonomous gap should be [9] * SMTC period

Draft response LS to RAN2 for the T321 timer value will be provided in [4].

3	Conclusion

In this contribution we discussed requirements for CGI reading of NR cell with autonomous gaps for NR capable UE. We have made the following observations and proposals:

- known cell condition for CGI reading with autonomous gap could be:

In FR1 and FR2, the target cell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the report CGI command,

- the UE has sent a valid L3-RSRP measurement report for the target cell and

- One of the SSBs measured from the target cell remains detectable according to the cell identification conditions specified in clause 9.3.

- One of the SSBs measured from the target cell remains detectable during CGI reading delay according to the cell identification conditions specified in clause 9.3.

SIB1 decoding delay should be 4 * SMTC period

Beam sweeping is not needed for MIB decoding.

MIB decoding delay in FR2 is [5] * SMTC period.

CGI reading delay of NR cell should be [9] * SMTC period

T321 timer value of autonomous gap should be [9] * SMTC period

References

- R4-2005843,	WF on R16 NR RRM enhancements - CGI reading with autonomous gap, ZTE

- R4-2004516, discussion on CGI reading with autonomous gap, Nokia, Nokia Shanghai Bell

- R2-2002219, Reply LS on CGI reading with autonomous gaps, RAN2

- R4-2008189, Draft Response LS on CGI reading with autonomous gaps, Nokia, Nokia Shanghai Bell

# A