3GPP TSG-RAN WG4 Meeting #95-e	R4-2008280

Electronic Meeting, 25 May – 5 June, 2020

Source:	ZTE

Title:	TCI state switching under NR-U

Agenda item:	6.1.5.7

Document for:	Discussion

# 1	Introduction

In RAN4 94-e-bis, the WF [1] captures the agreements shown below:

The LS [2] asks if RAN2 can support to declare beam failure in this case:

Known state: - RRC-based: - LRRC,known,max =[2] for TSSB≤40 ms, LRRC,known,max =[1] for TSSB>40 ms - Upon exceeding LRRC,known,max the UE may stop the active TCI state switching procedure and FFS: declare beam failure - MAC-CE based: - LMAC,known,max =[2] for TSSB≤40 ms, LMAC,known,max =[1] for TSSB>40 ms - Upon exceeding LMAC,known,max the UE may stop the active TCI state switching procedure and FFS: stay in the old state Unknown state: - RRC-based: - L1RRC,unknown,max =[2] for TCSI-RS/SSB ≤40 ms, L1MAC,unknown,max = [1] for TCSI-RS/SSB>40 ms - L2RRC,unknown,max =[2] for TSSB ≤40 ms, L2MAC,unknown,max = [1] for TSSB>40 ms - Upon exceeding L1RRC,unknown,max or L2RRC,unknown,max the UE may abandon the active TCI state switching procedure and FFS: declare beam failure - MAC-CE based switching: - L1MAC,unknown,max = [2] for TCSI-RS/SSB≤40 ms, L1MAC,unknown,max = [1] for TCSI-RS/SSB>40 ms - L2MAC,unknown,max =[2] for TSSB≤40 ms, L2MAC,unknown,max = [1] for TSSB>40 ms - Upon exceeding L1MAC,unknown,max or L2MAC,unknown,max the UE may stop the active TCI state switching procedure and FFS: stay in the old state In the above, L*,max is the maximum number of SSB occasions not available at the UE due to CCA failure for the corresponding state and switching type.

This paper intends to provide our view on this topic.

2	Discussion

The intention of declaring beam failure is that such behavior can

- Make sure the UE and the network are on the same page, all knowing the current condition at the UE side

- The UE can provide the network with a better candidate beam when sending RACH to the network

- However, after re-thinking about this option, we find out that declaring beam failure cannot fulfil its goal. For 1), since the network is always aware of DL LBT failures, so once a RS is not transmitted due to DL LBT failure, the UE and the network all know that even without signalling.

- Observation 1: Once a RS is not transmitted due to DL LBT failure, the UE and the network all know that even without signalling.

- For 2), we think this purpose can only be fulfilled if the UE can perform directional LBT. In R16, the UE only performs omni-directional LBT, which means the LBT result will only depend on the channel occupancy. Changing the Rx or Tx beam of UE won’t change LBT result.

- Observation 2: In R16, the UE only performs omni-directional LBT, which means the LBT result will only depend on the channel occupancy. Changing the Rx or Tx beam of UE won’t change LBT result.

- Observation 2 means that if LBT fails with an old beam, with a new beam it will most likely to fail because LBT is done with an omni-directional manner.

- Based on the above observations, we propose that:

The UE shall not declare beam failure when RRC based TCI state switch fails.

The UE shall not declare beam failure because such behavior brings not much benefit and won’t solve the problem.

For RRC and MAC-CE based TCI switching, there are no ideal options right now. For MAC-CE it’s because letting the UE stay in the old state won’t provide good performance, since TCI state switching was triggered because the old TCI state is not satisfactory. For RRC based TCI state switching, the old state is discarded and makes the situation worse. Thus, we propose to further study how to handle these cases while waiting for RAN2 LS.

Further study how to handle these TCI state switching failures while waiting for RAN2 LS.

3	Conclusion

- Observation 1: Once a RS is not transmitted due to DL LBT failure, the UE and the network all know that even without signalling.

- Observation 2: In R16, the UE only performs omni-directional LBT, which means the LBT result will only depend on the channel occupancy. Changing the Rx or Tx beam of UE won’t change LBT result.

- Proposal 1: The UE shall not declare beam failure when RRC based TCI state switch fails.

Proposal 2: Further study how to handle these TCI state switching failures while waiting for RAN2 LS.

References

- R4-2005366

- R4-2005365