3GPP TSG-RAN4 Meeting #95-e	R4-2008905

# Online, , 25th May 2020 - 5th Jun 2020 								Revision of R4-2006646

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0019 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.307: Introduction of Power Class 1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_B41_Bn41_PC29dBm-Core |  |  |  |  |  | Date: |  |  | 2020-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of the new 29 dBm power class for intra-band EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds release independent requirements for the 29 dBm power class |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for 29 dBm HPUE are not release independent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 8.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.101-3 CR 0249 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 deleted changes to section 5.1 because there is not agreement for PC1.5 for UL MIMO and Tx diversity yet. |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

### 8.1.1	Intraband EN-DC

Requirements for a Rel-16 UE for additional EN-DC intraband configurations within FR1 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.1-0: EN-DC intraband UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Intraband contiguous EN-DC power class 1.5 | TDD | Rel-15 | Table B.4.6-1 |
| Intraband contiguous EN-DC power class 2 | TDD | Rel-15 |  |
| Intraband contiguous EN-DC power class 3 | FDD, TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 1.5 | TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 2 | TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 3 | FDD, TDD | Rel-15 |  |

Table 8.1.1-1: EN-DC contiguous intraband configurations within FR1

| Feature | DL/UL | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| intraband contiguous EN-DC | DL | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | FDD, TDD | Rel-15 |  |

Table 8.1.1-2: EN-DC non-contiguous intraband configurations within FR1

| Feature | DL/UL | maximum number of sub-blocks | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| intraband non-contiguous EN-DC | DL | 2 | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 2 | 1 | 1 | FDD, TDD | Rel-15 |  |

* * * End of Changes * * *