3GPP TSG-RAN WG4 Meeting #96-e                       	 R4-2010040

Electronic Meeting, 17 Aug. – 28 Aug., 2020

Agenda Item:	7.13.1.1

Source:	MediaTek Inc.

Title:	Remaining issues on interruption at SRS Carrier Switching

Document for:		Discussion

- Introduction

In last meeting, a WF of SRS carrier switching has been agreed [1]. The remaining issues are approved in last RAN plenary #88 as follow.

- FFS interruption requirements for case 1, case 2 and case 3. - Case 1: CA is co-location deployed - Case 2: Single TAG CA, or carriers in the same TAG for multiple TAG CA - Case 3: uplink time difference does not exceed a threshold X - X = [5] us - Note: SRS carrier switching requirement for inter-band FR2 CA is out of scope

In this paper, we will continue to discuss the impact on SRS carrier switching based on this WF.

- Interruption at SRS Carrier Switching in NR

In last meeting, some companies suggested to differentiate CA case with different requirement. However, whether CCs are in CA or not, the uplink TA can be larger than 1 slot in NR. This will result in additional interruption length of UE’s DL reception both in sync and async. scenarios.

Originally, compared to synchronous DC/CA scenarios, we should add 1 more interrupted slots in asynchronous DC scenarios. Nevertheless, considering UL TA would definitely introduce 1 more interrupted slot into requirement, the SRS switching doesn’t need to differentiate synchronous or asynchronous scenarios because of this uncertainty UL TA.

Proposal 1: Define Interruption requirements of sync case for CA the same as async case.

- Summary

In this paper, we propose the SRS Carrier Switching Interruption requirement for NR.

Proposal 1: Define Interruption requirements of sync case for CA the same as async. case.

- References

- R4-2009100, “Way forward on R16 NR RRM enhancements – SRS carrier switching”, ZTE