3GPP TSG-RAN4 Meeting #96-e	R4-2010057

# Electronic meeting, August 17-28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0970 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MRTD for FR2 inter-band CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Revise CSSF when CSI-RS resources for L3 measurement are considered on top of SSB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise CSSF when CSI-RS resources for L3 measurement are considered on top of SSB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS resources for L3 measurement are not considered for CSSF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes >>

### 9.1.5	Carrier-specific scaling factor

This clause specifies the derivation of carrier-specific scaling factor (CSSF) values, which scales the measurement delay requirements given in clause 9.2, 9.3 and 9.4 when UE is configured to monitor multiple measurement objects. The CSSF values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i, for the measurements conducted outside measurement gaps and within measurement gaps, respectively.

#### 9.1.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement in clause xxx, when none of CSI-RS resources for L3 measurement of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement in clause xxx, when part of CSI-RS resources for L3 measurement of this intra-frequency measurement object are overlapped by the measurement gap.

-	Inter-frequency measurement with no measurement gap in clause 9.3.5, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap.

-	Inter-frequency measurement with no measurement gap in clause 9.3.5, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap.

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in clause 9.3.8 when no measurement gaps are provided.

The requirements in this clause apply provided that

- 	There are only SCCs in FR2, or

- 	The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met

-	If smtc2 is configured on any FR2 CC,

-	All CCs have the same configuration for smtc1, and

-	All CCs configured with smtc2 have the same configuration for smtc2

-	If smtc2 is not configured on any FR2 CC,

-	The total number of different SMTC periodicities on all serving CCs does not exceed 4

- SMTC and CSI-RS resources in the same MO, which can be fully or partially overlapped, are aligned and confined within 5ms window.

Note:	Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on FR2 serving carriers. The longer delay applies for the FR2 intra-frequency measurement objects with the longest SMTC periodicity/periodicities.

##### 9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS L3 measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency, inter-frequency SSB-based measurements and intra-frequency CSI-RS L3 measurement performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gp |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1: if only SSB or CSI-RS L3 configured for PCC 2 if both SSB and CSI-RS L3 configured for PCC | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS configured+ Number of configured FR1 SCell(s) with either SSB or CSI-RS configured+Y | N/A | N/A | N/A | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS configured+ Number of configured FR1 SCell(s) with either SSB or CSI-RS configured+Y |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1: if only SSB or CSI-RS L3 configured for PCC 2 if both SSB and CSI-RS L3 configured for PCC | N/A | 2*Number of configured FR2 SCell(s) with both SSB and CSI-RS configured+ Number of configured FR2 SCell(s) with either SSB or CSI-RS configured+Y | 2*Number of configured FR1 SCell(s) with both SSB and CSI-RS configured+ Number of configured FR1 SCell(s) with either SSB or CSI-RS configured+Y |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1: if only SSB or CSI-RS L3 configured for PCC 2 if both SSB and CSI-RS L3 configured for PCC | 2×(Number of configured SCell(s)+Y-1) | N/A | 2: if only SSB or CSI-RS L3 configured for the SCC 4: if both SSB and CSI-RS L3 configured for the SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y -1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y | 1: if only SSB or CSI-RS L3 configured for PSCC 2 if both SSB and CSI-RS L3 configured for PSCC | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note3: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG |  |  |  |  |  |  |

##### 9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y | N/A | N/A | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y |
| FR2 only intra band CA | N/A | N/A | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) | 2: if only SSB or CSI-RS L3 configured for PSCC 4: if both SSB and CSI-RS L3 configured for PSCC | 2 | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note3: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG |  |  |  |  |  |  |

##### 9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1 | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y) | 2: if only SSB or CSI-RS L3 configured for PSCC 4: if both SSB and CSI-RS L3 configured for PSCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y) |
| Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. Note 2: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG |  |  |  |  |  |

##### 9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NE-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.4-1.

Table 9.1.5.1.4-1: CSSFoutside_gap,i scaling factor for NE-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y | N/A | N/A | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | N/A | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y | 2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1: if only SSB or CSI-RS L3 configured for PCC 2: if both SSB and CSI-RS L3 configured for PCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) | N/A | 2: if only SSB or CSI-RS L3 configured for SCC 4: if both SSB and CSI-RS L3 configured for SCC | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) | 2×(2*Number of configured SCell(s) with both SSB and CSI-RS configured+ Number of configured SCell(s) with either SSB or CSI-RS configured +Y-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note3: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG |  |  |  |  |  |  |

<< end of changes >>