3GPP TSG-RAN WG4 Meeting #96-e				  											R4-2010428 Online, 17th Aug. 2020 – 28th Aug. 2020

Source: 	Nokia, T-mobile

Title: 		TP for 37.717-11-21 to introduce DC_3A-28A_n1A-n40A

Agenda Item: 			10.7.2 [DC_R17_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-11-21 to introduce DC_3A-28A_n1A-n40A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 7.x	DC_3-28_n1-n40

### 7.x.1	Operating bands for DC

Table 7.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 7.x.2	Configuration for DC

Table 7.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A | CA_3A-28A | CA_n1A-n40A |

### 7.x.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n1-n40 | 3 | 0.5 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |
|  | n40 | 0.5 |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n1-n40 | 3 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0 |

### 7.x.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

*********************** End of the TP ***************************************************

# 3	References