3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010496

# Electronic Meeting, 17-28 Aug., 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on NR-U UE ACS

Agenda Item:	7.1.2.2

Document for:	Approval

Introduction

In this contribution, we discuss the UE ACS requirement for NR-U.

Discussion

The UE ACS requirement has been discussed in previous meetings. There are still some open issue need further discussion [1].

- Single Carrier: - ACS value is TBD at 20 MHz CBW - Interferer BW is 20 MHz for all CBW - Scale the ACS value by 10log(bandwidth/20MHz) for BW larger than 20 MHz. - Case 1: Wanted signal power is REFSENS + 14 dB - Case 2: [Not specified] - Intra-band contiguous CA: - Interferer BW is 20 MHz (BW Classes B, C, D, E, I, M, N, O) - [ACS level of intra-band CA set according to the actual configured aggregated bandwidth]

Firstly we look at the ACS requirement for 20MHz channel BW. WIFi does not include ACS definition but define Adjacent Channel Rejection (ACR) as the RX selectivity requirements. ACR is defined as a function of the adopted MCS. As discussed in Rel-13 LAA WI, the equivalent ACS is in the range from 22 dB to 29 dB [2]. Worse ACS requirement than WIFI might not be a good option. And the ACS requirements for LAA are 27 dB for 20 MHz channel BW. It is proposed to reuse LAA 27 dB ACS for NR-U UE.

Table 8.4.2-2 of TR 36.889: ACIR values when Wi-Fi is the victim system.

| Study Case | Wi-Fi ACS (dB) | Aggressor ACLR (dBc) | ACIR (dB) |
| --- | --- | --- | --- |
| LAA node to Wi-Fi AP/STAs | 22 | 45 | 21.98 |
|  | 25 |  | 24.96 |
|  | 29 |  | 28.89 |
| LAA UE to Wi-Fi AP/STAs | 22 | 30 | 21.36 |
|  | 25 |  | 23.81 |
|  | 29 |  | 26.46 |
| Wifi AP/STAs to Wi-Fi AP/STAs | 22 | 26.35 | 20.64 |
|  | 25 |  | 22.61 |
|  | 29 |  | 24.47 |

Proposal 1: ACS for NR-U UE is 27 dB for 20 MHz channel BW.

Regarding the need of case 2, the dynamic range may be smaller than that of NR UE, and the interference level could be less than -25 dBm which is defined for NR case 3 ACS. One consideration could be LBT power level + ACLR= -72+27= -45 dBm. It is 5 dB higher than that already close to the level for case 1 which is -89.7+39.5 = -50.2 dBm. Hence it will be ok not to specify case 2 ACS

Proposal 2: Case 2 ACS is not specified.

Conclusions

In this contribution, we discussed the remaining issues on UE ACS requirement in NR-U, and the following proposal was made:

Proposal 1: ACS for NR-U UE is 27 dB for 20 MHz channel BW.

Proposal 2: Case 2 ACS is not specified.

References

R4-2008435, WF on NR-U ACS and blocking requirements, Apple

TR 36.889 Study on Licensed-Assisted Access to Unlicensed Spectrum