3GPP TSG-RAN WG4 Meeting#96-e                	R4-2010564

E-meeting, August 17th – 28th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurement Performance Requirements test for euCA

Agenda item:	5.3

Document for:	Discussion

Introduction

During last RAN5#87-e meeting, when RAN5 was developing the euCA RF testcases for LTE Rel-15, it was observed that RAN4 specification are missing the necessary accuracy testcases Annex 9.1. Hence, although RAN4 in 36.133 specifications chapter 4.9 defines accuracy requirements for the euCA RSRP and RSRQ measurements, the necessary performance requirement test cases are missing.

This paper will discuss what additions are needed in RAN4, such that RAN5 can develop the necessary test cases for euCA (idle mode measurements).

Discussion

According to 36.331, for Rel-15 LTE early measurement reporting scheme was standardized such that UE will perform the idle mode measurements in idle mode, according to the EMR configuration which is provided either in system information or RRC release message. UE shall only report the EMR measurement results for the cells which at least meet the configured threshold (qualityThreshold). This means that the UE will perform measurements, but shall only report them to network, if the measurements fulfil quality criteria (as per network configuration) -:

TS 36.331- fa0 (LTE Rel-15):

#### 5.6.20.2 Performing measurements … store the derived measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within the VarMeasIdleReport in decreasing order of the sorting quantity, i.e. the best cell is included first, as follows: 5> if qualityThreshold is configured: 6> include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold; 5> else: 6> include the measurement results from all cells applicable for idle/inactive measurement reporting; … #### MeasIdleConfig The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE. MeasIdleConfig information element … MeasIdleCarrierEUTRA-r15::= SEQUENCE { carrierFreq-r15 ARFCN-ValueEUTRA-r9, allowedMeasBandwidth-r15 AllowedMeasBandwidth, validityArea-r15 CellList-r15 OPTIONAL, -- Need OR measCellList-r15 CellList-r15 OPTIONAL, -- Need OR reportQuantities ENUMERATED {rsrp, rsrq, both}, qualityThreshold-r15 SEQUENCE { idleRSRP-Threshold-r15 RSRP-Range OPTIONAL, -- Need OR idleRSRQ-Threshold-r15 RSRQ-Range-r13 OPTIONAL -- Need OR } OPTIONAL, -- Need OP ... } …

As RAN2 specified the quality threshold as criteria for UE to report the measurements performed in idle, this aspect needs to be reflected also in RAN4 specifications. Therefore, RAN4 need to define the measurement performance test case to ensure that RSRP/Q measurement accuracy is within the specified limits.

RAN4 has defined the requirements for absolute accuracy of +/- 6 dB [as per Table 9.1.2B/3B – 36.133] for both idle mode serving cell (Table 9.1.2B) and inter-frequency carrier measurements (Table 9.1.3B), but there are no test cases to ensure that the accuracy of the measurements are within these limits. The purpose of such test is to verify that the UE RSRP/Q measurement accuracies are within the specified limits.

Define measurement performance test cases for RSRP and RSRQ measurement accuracies for euCA.

Define intra-frequency measurement performance test cases for RSRP and RSRQ.

Define inter-frequency measurement performance test cases for RSRP and RSRQ.

In [1] we provided the CR.

Conclusion

In this paper we discuss missing tests for accuracy requirements for the euCA RSRP and RSRQ measurements.

- Define measurement performance test cases for RSRP and RSRQ measurement accuracies for euCA.

Define intra-frequency measurement performance test cases for RSRP and RSRQ.

Define inter-frequency measurement performance test cases for RSRP and RSRQ.

References

- R4-2010565, CR on performance requirements tests for euCA, Nokia, Nokia Shanghai Bell