3GPP TSG-RAN WG4 Meeting # 96-e                	R4-2010906

Electronic Meeting, 17-28 Aug., 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 	2-step RACH BS demodulation simulation results

Agenda item: 	7.18.3

Document for: 	Discussion

Introduction

This document extends the discussion on the 2-step RACH BS demodulation requirements introduced in [1] after the discussion on the RAN4 #95-e summarized in [2] and with a proposed way forward in [3].

In this contribution we present our simulation results for 2-step RACH demodulation performance.

Simulation assumptions

The simulation assumptions used to derive the simulation results are shown Table 1, which is based on [3]. From the options available in [3], both PUSCH mapping types were simulated, and the 1+1+1 DM-RS configurations was chosen. The simulation results are shown in Table 2.

Table 1 Simulation assumptions for 2-step RACH based on [3]

| Parameter | Values for FR1 |  | Values for FR2 |  |
| --- | --- | --- | --- | --- |
| Preamble format | Free choice |  |  |  |
| Waveform | CP-OFDM |  |  |  |
| Power offset between preamble and MsgA | Free choice |  |  |  |
| Subcarrier spacing for PUSCH | 15kHz, 30kHz |  | 60kHz, 120 kHz |  |
| PUSCH Mapping Type | Option 1: Type A and Type B Option 2: Type A |  | Option 1: Type A and Type B Option 2: Type A |  |
| MCS level | 1 |  | 3 |  |
| Number of symbols | 14 |  | 10 |  |
| Number of PRBs | 2 |  | 2 |  |
| TBS | 56 - 72 bits |  |  |  |
| DMRS | Option 1: 1+1+1 Option 2: 1+1 |  | Option 1: 1+1+1 Option 2: 1+1 |  |
| Antenna configuration | 1T2R |  | 1T2R |  |
| Propagation channel | TDLC300-100 |  | TDLA30-300 |  |
| TO values | Medium level | SCS 15k: 0.8:0.2:2 | Medium level | SCS 60k: 0.2:0.05:0.5 |
|  |  | SCS 30k: 0.4:0.1:1 |  | SCS 120k: 0.1:0.025:0.25 |
|  | High level | SCS 15k: 0:0.1:3.8 | High level | SCS 60k: 0:0.1:0.6 |
|  |  | SCS 30k: 0:0.1:3.8 |  | SCS 120k: 0:0.1:0.6 |
| Test metric (BLER of MsgA when preambles are correctly detected) | - Option 1: 0.01 - Option 2: 0.1 |  | - Option 1: 0.01 - Option 2: 0.1 |  |
| Re-transmission | No retransmission considered |  |  |  |

Table 2 Simulation results

| Channel model | SCS | PUSCH mapping type | DM-RS | TO level | 10% | 1% |
| --- | --- | --- | --- | --- | --- | --- |
| TDLC300-100 | 15 kHz | TypeA | 1+1+1 | High | 3,16 | 8,63 |
| TDLC300-100 | 15 kHz | TypeA | 1+1+1 | Medium | 0,97 | 5,94 |
| TDLC300-100 | 15 kHz | TypeB | 1+1+1 | High | 3,17 | 8,58 |
| TDLC300-100 | 15 kHz | TypeB | 1+1+1 | Medium | 1,03 | 5,89 |
| TDLA30-300 | 60 kHz | TypeA | 1+1+1 | High | 2,69 | 8,10 |
| TDLA30-300 | 60 kHz | TypeA | 1+1+1 | Medium | 2,64 | 7,96 |
| TDLA30-300 | 60 kHz | TypeB | 1+1+1 | High | 2,72 | 8,17 |
| TDLA30-300 | 60 kHz | TypeB | 1+1+1 | Medium | 2,69 | 8,01 |
| TDLA30-300 | 120 kHz | TypeA | 1+1+1 | High | 8,37 | 15,0 |
| TDLA30-300 | 120 kHz | TypeA | 1+1+1 | Medium | 2,65 | 7,87 |
| TDLA30-300 | 120 kHz | TypeB | 1+1+1 | High | 8,17 | 14,63 |
| TDLA30-300 | 120 kHz | TypeB | 1+1+1 | Medium | 2,66 | 7,89 |

Conclusion

In this contribution we provided the results of our simulation campaign on 2-step RACH baseline simulations for NR_HST Rel-16 BS demodulation performance requirements.

These results are discussed in detail in our companion discussion paper [4].

References

- RP-200085, “Revised WID on 2-step RACH for NR”, 3GPP TSG RAN Meeting #87e, ZTE Corporation

- R4-2009046, “Email discussion summary for [95e][#325] 2-step RACH BS demodulation requirements”, 3GPP TSG RAN WG4 #95-e, Moderator (ZTE Corporation)

- R4-2008864, “WF on BS demodulation requirements for 2-step RACH”, 3GPP TSG RAN WG4 #95-e, Moderator (ZTE Corporation)

- R4-2010907, “On 2-step RACH BS demodulation requirements”, 3GPP TSG RAN WG4 #96-e, Nokia, Nokia Shanghai Bell