3GPP TSG-RAN WG4 Meeting #96-e	R4-2010977

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion on URLLC UE performance requirements with ultra-low BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.1.1

Document for:	Discussion

Background

The simulation results of URLLC UE high reliability with ultra-low BLER are presented in this paper based on the parameters agreed in WF [1].

Simulation

Simulation assumption

The agreed UE parameters are concluded in Table 2-1:

Table 2-1: Simulation assumption for UE PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 1x2, ULA low 1x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Processes |  | 1 |
| PT-RS |  | Disabled |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS14 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 1-10-5 |

Simulation results

The simulation results are shown in below:

Table 2-2: SNR values for PDSCH performance requirements

| SCS Antenna configuration SNR | 1x2 | 1x4 |
| --- | --- | --- |
| 15 kHz/10 MHz | 0.2 | -2.3 |
| 30 kHz/40 MHz | 0.5 | -2.3 |

Reference

[1] R4-2008805, Way forward on ultra-low BLER requirements, #95, Ericsson.