3GPP TSG-RAN WG4 Meeting #96-e	R4-2011043

Electronic Meeting, 17-28 Aug., 2020

Agenda item:		7.16.1.1

Source:				NTT DOCOMO, INC.

Title:		Views on CA PDSCH requirements

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed CA PDSCH requirements and RAN4 agreed a WF [1]. This contribution presents our views on CA PDSCH requirements.

2. Discussion

For test applicability rule, following options were also captured in [1].

- Categorizing of CA capabilities - Option 1: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. - Option 2: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA. - Companies to bring proposals on the demod spec structure for CA, with the motivation to minimize future maintenance. - Test of different CA capabilities - Option 1: Test intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with the largest number of bands. - Option 2: Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands.

Categorizing of CA capabilities

In LTE, CA capability is defined for different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands, i.e. Option 1.We do not see any motivation to modify and/or simplify such approach so we consider that Option 1 should be baseline for further discussion although we still need to discuss the details about how to reduce future maintenance work as some companies proposed.

Test of different CA capabilities

For example, UE implementation (e.g. FFT and/or baseband signal processing) can be different between intra-band contiguous CA and intra-band non-contiguous CA. Therefore, in LTE, it is assumed that all the CA capabilities which is supported by UE shall be tested. Even for NR, we consider that all the CA capabilities should be tested so our preference is Option 2.

Proposal 1: Use the following approach on CA test applicability

Categorizing of CA capabilities

- Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands.

Test of different CA capabilities

- Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands.

3. Conclusion

In this contribution, we present our views on CA PDSCH. Our proposals and observations are summarized below.

Proposal 1: Use the following approach on CA test applicability

Categorizing of CA capabilities

- Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands.

Test of different CA capabilities

- Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands.

References

- R4-2008838, Intel, Way forward on PDSCH CA normal demodulation requirements, RAN4 #95e