3GPP TSG-RAN WG4 Meeting #96-e	R4-2011208

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6954 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eMTC DL channel quality report mapping table and RSS measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For BL UE or UE in CE, channel quality is reported as Quality Report in MAC CE Downlink Channel Quality Report. The Quality Report is represented as 4-bit Codepoint/index. Mapping table should be also given as codepoint/index. RSS change: Measurement delay requirements for RSS compared to CRS based RSRP measurement is clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Reported value in the mapping table for DCQR MAC-CE is changd to codepoint/index. Removal of [] for accuracy requirements. Change #2: Scaling factor for measurement delay in normal coverage is removed. Change #3: Scaling factor for measurement delay for enhanced coverage is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch between TS36.321 and TS36.133. The relation between RSS based measurement delay and CRS based measurement delay is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.21.22, 9.1.21.23, 9.1.21.24, 4.7.2.1.1, 4.7.2.1.2, 4.7.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

#### 9.1.21.22	Downlink Channel Report Mapping for UE Category M1

Table 9.1.21.22-1: Downlink channel quality measurement report mapping when the Downlink Channel Quality Report (DCQR) MAC Control Element is supported [7]

| Codepoint/Index | MPDCCH repetition level | MPDCCH aggregation level | Applicable CE Mode |
| --- | --- | --- | --- |
| 0000 | No measurement reporting | No measurement reporting | A, B |
| 0001 | 1 | 1 | A |
| 0010 | 1 | 2 | A |
| 0011 | 1 | 4 | A |
| 0100 | 1 | 8 | A |
| 0101 | 1 | 16 | A |
| 0110 | 1 | 24 | A, B |
| 0111 | 2 | 24 | A, B |
| 1000 | 4 | 24 | A, B |
| 1001 | 8 | 24 | A, B |
| 1010 | 16 | 24 | A, B |
| 1011 | 32 | 24 | A, B |
| 1100 | 64 | 24 | A, B |
| 1101 | 128 | 24 | A, B |
| 1110 | 256 | 24 | A, B |

The MPDCCH repetition level for Short Downlink Channel Quality Report (DCQR) MAC Control Element is chosen with regard to the signalled parameter Rmax, the maximum number of repetitions for MPDCCH common search space for random access response (mpdcch-NumRepetition-RA) in SystemInformationBlockType2. The report mapping is defined in Table 9.1.21.22-2.

Table 9.1.21.22-2: Downlink channel quality measurement report mapping when Short Downlink Channel Quality Report (DCQR) MAC Control Element is supported [7]

| Reported value | MPDCCH repetition level |
| --- | --- |
| No short DCQR | No measurement reporting |
| Short DCQR 1 | Rmax/8 (Note 1) |
| Short DCQR 2 | Rmax (Note 3) |
| Short DCQR 3 | 4xRmax (Note 2) |
| Note 1: When Rmax is less than 8, set Short DCQR 1 to 1. Note 2: When Rmax is more than 32, set Short DCQR 3 to 256. Note 3: When Rmax is 1, set Short DCQR 2 to 2. Note 4: Aggregation level (ECCE) is assumed to be L’max = 24. |  |

#### 9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A

The requirements for accuracy of downlink channel quality reporting in this clause apply only to the serving cell on the anchor carrier for UE Category M1.

The accuracy requirements in Table 9.1.21.23-1, Table 9.1.21.23-2, Table 9.1.21.23-3, and Table 9.1.21.23-4 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.23-1: Downlink channel quality reporting accuracy for UE Category M1 with CE Mode A for FDD and TDD

| MPDCCH Repetition | Pm-Dsg (%) | Ês/Iot | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB ≤ Ês/Iot ≤ -3 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB ≤ Ês/Iot ≤ -3 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| NOTE 1: R is the reported MPDCCH repetition level that UE has reported in DCQR MAC CE or Short DCQR MAC CE. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.23-2: Downlink channel quality reporting accuracy for UE Category M1 with CE Mode A for HD-FDD

| MPDCCH Repetition | Pm-Dsg (%) | Ês/Iot | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB ≤ Ês/Iot ≤ -3 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB ≤ Ês/Iot ≤ -3 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| NOTE 1: R is the reported MPDCCH repetition level that UE has reported in DCQR MAC CE or Short DCQR MAC CE.. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.23-3: Downlink channel quality reporting accuracy for UE Category M1 with CE Mode A for FDD and TDD

| MPDCCH Aggregation level | Pm-Dsg (%) | Ês/Iot | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| Lk NOTE 1 | ≤1 | ≥ -3 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| Lk-2 NOTE 1 | >1 | ≥ -3 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| NOTE 1: Lk∈{1,2,4,8,16,24} is the reported MPDCCH aggregation level that UE has reported in DCQR MAC CE where k is the index to the aggregation level set and 1 ≤ k ≤ 6. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.23-4: Downlink channel quality reporting accuracy for UE Category M1 with CE Mode A for HD-FDD

| MPDCCH Aggregation level | Pm-Dsg (%) | Ês/Iot | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| Lk NOTE 1 | ≤1 | ≥ -3 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| Lk-2 NOTE 1 | >1 | ≥ -3 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| NOTE 1: Lk∈{1,2,4,8,16,24} is the reported MPDCCH aggregation level that UE has reported in DCQR MAC CE where k is the index to the aggregation level set and 1 ≤ k ≤ 6. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B

The requirements for accuracy of downlink channel quality reporting in this clause apply only to the serving cell on the anchor carrier for UE Category M1.

The accuracy requirements in Table 9.1.21.24-1 and Table 9.1.21.24-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.24-1: Downlink channel quality reporting accuracy for UE Category M1 with CE Mode B for FDD and TDD

| MPDCCH Repetition | Pm-Dsg (%) | Ês/Iot | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| NOTE 1: R is the reported MPDCCH repetition level that UE has reported in DCQR MAC CE or Short DCQR MAC CE. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.24-2: Downlink channel quality reporting accuracy for UE Category M1 with CE Mode B for HD-FDD

| MPDCCH Repetition | Pm-Dsg (%) | Ês/Iot | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| NOTE 1: R is the reported MPDCCH repetition level that UE has reported in DCQR MAC CE or Short DCQR MAC CE. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

------------------------------------------------------------- Start of change 2 ------------------------------------------------------------

##### 4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage

The requirements in this subclause apply if UE is in the normal coverage area of the serving cell. The UE is considered to be in normal coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The requirements defined in section 4.2.2.1 apply for this section.

The UE is allowed to perform RSRP measurements based on RSS signals provided UE is configured with rss-ConfigCarrierInfo [2] and following conditions are met:

-	At least two subframes containing RSS are available within the MPDCCH bandwidth for Nserv successive DRX cycles and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH,

-	RSS power offset (PRSS)with respect to CRS as defined in RSS-Config [2], where PRSS ≥ 0 dB

-	RSS-based measurement period (Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC) is not longer than CRS-based measurement period.

-	RSS periodicity (TRSS) as defined in RSS-Config [2]

##### If UE performs RSRP measurement based on RSS for serving cell, it is not expected to perform RSRP measurement based on CRS on that serving cell.

##### 4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage

The UE which supports wakeUpSignal-r15 or wakeUpSignal-TDD-r15 shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.7.2.1.1, provided the following conditions are met:

-	WUS has been configured in the serving cell using WUS-Config-r15, and

-	The serving cell measurement relaxation is signalled by the network using num-DRX-CyclesRelaxed, and

-	Serving cell S criteria is met with at least 2 dB margin.

-	The relaxed monitoring criteria for neighbour cells in TS 36.304 [1] clause 5.2.4.12.1 is fulfilled,

Otherwise the requirements defined for the configured DRX cycle length in Section 4.7.2.1.1 shall apply.

The UE shall further meet the requirements in section 4.7.2.1.1 during time period T0 after following occasions:

-	after the end of reception of latest paging message, or

-	from the moment UE has switched from RRC_CONNECTED state to RRC_IDLE state.

T0 = N*DRX cycle if the UE is not configured with eDRX_IDLE cycle and T0 = one eDRX IDLE cycle if the UE is configured with eDRX_IDLE cycle.

The relaxation factor N is given by Table 4.7.2.1.1A-1 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7.2.1.1A-2 if the UE is configured with eDRX_IDLE cycle.

Table 4.7.2.1.1A-1: The relaxation factor N for a UE not configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 32) |
| 0.64 | Min(n , 16) |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| NOTE: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |

Table 4.7.2.1.1A-2: The relaxation factor N for a UE configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |  |  |
| --- | --- | --- | --- |
|  | 1.28 ≤ PTW length [s] < 2.56 | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] |
| 0.32 | Min(n , 2) | Min(n , 4) | Min(n , 8) |
| 0.64 | 1 | Min(n , 2) | Min(n , 4) |
| 1.28 | N/A | 1 | Min(n , 2) |
| 2.56 | N/A | N/A | 1 |
| NOTE: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |  |  |

##### 4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage

The requirements in this subclause apply if UE is in the normal coverage area of the serving cell. The UE is considered to be in normal coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall be able to identify new intra-frequency cells and perform RSRP and RSRQ measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE is allowed to perform RSRP measurements based on RSS signals provided UE is configured with rss-ConfigCarrierInfo [2] and following conditions are met:

-	At least two subframes containing RSS of the measured cell are available within the MPDCCH bandwidth for 3 successive DRX cycles in the window of [n-5, n-1] and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH for UEs which are capable of measuring neighbour cell RSS on the same MPDCCH bandwidth or

-	At least two subframes containing RSS of the measured cell are available within the same RSS RB location of the serving cell for 3 successive DRX cycles in the window of [n-5, n-1] and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH for UEs which are not capable of measuring neighbour cell RSS on the same MPDCCH bandwidth or

-	RSS-based measurement period (Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC) is not longer than CRS-based measurement period.

-	RSS power offset (PRSS)with respect to CRS as defined in RSS-Config [2], where PRSS ≥ 0 dB

-	RSS periodicity (TRSS) as defined in RSS-Config [2]

If UE performs RSRP measurement based on RSS on detected intra-frequency cell, it is not expected to perform RSRP measurement based on CRS on that measured cell.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Tdetect,EUTRAN_Intra_NC when that Treselection= 0 . An intra frequency cell is considered to be detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.3 for a corresponding Band.

The UE shall measure RSRP and RSRQ at least every Tmeasure,EUTRAN_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter RSRP and RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN_Intra_NC/2.

The UE shall not consider a E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,E-UTRAN_Intra_NC when Treselection = 0, provided that the cell is at least 4dB better ranked for Cat-M1 UE.

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC are specified in Table 4.7.2.1.2-1. For UE configured with eDRX_IDLE cycle, Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC are specified in Table 4.7.2.1.2-2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC when multiple PTWs are used.

Table 4.7.2.1.2-1 : Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC

| DRX cycle length [s] | Tdetect,EUTRAN_Intra_NC [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra_NC [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Table 4.7.2.1.2-2: Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate,E-UTRAN_Intra_NC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_Intra_NC [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra_NC [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 5.12 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: 3 x TRSS if RSRP is measured on RSS signals defined in RSS-Config [2].; otherwise N1=1. |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 of TS 36.304 [1] are fulfilled then the UE's intra-frequency measurement is not required to meet Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate,E-UTRAN_intra_NC as defined in Table 4.7.2.1.2-1 and Table 4.7.2.1.2-2.

------------------------------------------------------------- End of change 2 ------------------------------------------------------------

------------------------------------------------------------- Start of change 3 ------------------------------------------------------------

##### 4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage

The requirements in this subclause apply if UE is in the enhanced coverage area of the serving cell. The UE is considered to be in enhanced coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall be able to identify new intra-frequency cells and perform RSRP and RSRQ measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities. The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

The UE is allowed to perform RSRP measurements based on RSS signals provided UE is configured with rss-ConfigCarrierInfo [2] and following conditions are met:

-	At least two subframes containing RSS of the measured cell are available within the MPDCCH bandwidth for 3 successive DRX cycles in the window of [n-5, n-1] and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH for UEs which are capable of measuring neighbour cell RSS on the same MPDCCH bandwidth or

-	At least two subframes containing RSS of the measured cell are available within the same RSS RB location of the serving cell for 3 successive DRX cycles in the window of [n-5, n-1] and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH for UEs which are not capable of measuring neighbour cell RSS on the same MPDCCH bandwidth or

-	RSS-based measurement period (Tmeasure,EUTRAN_Intra_NC and Tevaluate, E-UTRAN_Intra_NC) is not longer than CRS-based measurement period.

-	RSS power offset (PRSS)with respect to CRS as defined in RSS-Config [2], where PRSS ≥ 0 dB

-	RSS periodicity (TRSS) as defined in RSS-Config [2]

If UE performs RSRP measurement based on RSS on detected intra-frequency cell, it is not expected to perform RSRP measurement based on CRS on that measured cell.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Tdetect,EUTRAN_Intra_EC when that Treselection= 0 . An intra-frequency cell is considered to be detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.3 for a corresponding Band.

The UE shall measure RSRP and RSRQ at least every Tmeasure,EUTRAN_Intra_EC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter RSRP and RSRQ measurements of each measured intra-frequency cell using at least 4 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN_Intra_EC/2.

The UE shall not consider an E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,E-UTRAN_intra_EC when Treselection = 0, provided that the cell is at least 5dB better ranked.

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_Intra_EC, Tmeasure,EUTRAN_Intra_EC and Tevaluate, E-UTRAN_intra_EC are specified in Table 4.7.2.2.2-1. For UE configured with eDRX_IDLE cycle, Tdetect,EUTRAN_Intra_EC, Tmeasure,EUTRAN_Intra_EC and Tevaluate, E-UTRAN_intra_EC are specified in Table 4.7.2.2.2-2. Additionally, the requirements in Table 4.7.2.2.2-2 apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,EUTRAN_Intra_EC, Tmeasure,EUTRAN_Intra_EC and Tevaluate, E-UTRAN_intra_EC when multiple PTWs are used.

Table 4.7.2.2.2-1 : Tdetect,EUTRAN_Intra_EC, Tmeasure,EUTRAN_Intra_EC and Tevaluate, E-UTRAN_intra_EC

| SCH Ês/Iot of neighboring cell: Q2 [dB] | DRX cycle length [s] | Tdetect,EUTRAN_Intra_EC [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra_EC [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra_EC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0.32 | 330.24 (1032) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 330.24 (516) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 524.8 (410) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 1039.36 (406) | 2.56 (1) | 15.36 (6) |
| Q2-6 | 0.32 | 16.64 (52) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 23.04 (36) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 38.4 (30) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 66.56 (26) | 2.56 (1) | 15.36 (6) |

Table 4.7.2.2.2-2: Tdetect,EUTRAN_Intra_EC, Tmeasure,EUTRAN_Intra_EC and Tevaluate, E-UTRAN_intra_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_Intra_EC [s] (number N of DRX cycles) for neighboring cell with SCH Es/IoT: -15≤ Q2 < -6 [dB] | Tdetect,EUTRAN_Intra_EC [s] (number N of DRX cycles) for neighboring cell with SCH Es/IoT: Q2-6 [dB] | Tmeasure,EUTRAN_Intra_EC [s] (number N of DRX cycles) | Tevaluate,E-UTRAN_intra_EC [s] (number N of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 5.12 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | Note 3 (406) | Note 3 (26) | 0.32 x N1 (1 x N1) | Note 3 (6 x N1) |
|  | 0.64 | ≥1.28 (1) |  |  | 0.64 x N1 (1 x N1) | Note 3 (6 x N1) |
|  | 1.28 | ≥2.28 (1) |  |  | 1.28 x N1 (1 x N1) | Note 3 (6 x N1) |
|  | 2.56 | ≥2.56 (2) |  |  | 2.56 x N1 (1 x N1) | Note 3 (6 x N1) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: The detection period and the evaluation period depend on the number N of DRX cycles and are calculated according to the formula below: . NOTE 4: N1 = 5 x TRSS if RSRP is measured on RSS signals defined in RSS-Config [2].; otherwise N1=1. |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 of TS 36.304 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,EUTRAN_Intra_EC, Tmeasure,EUTRAN_Intra_EC and Tevaluate,E-UTRAN_intra_EC as defined in Table 4.7.2.2.2-1 and Table 4.7.2.2.2-2.

------------------------------------------------------------- End of change 3 ------------------------------------------------------------