3GPP TSG-RAN4 Meeting #96-e		R4-2011251

# Electronic Meeting, August 17-28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1092 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interruption requirements under dual DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To define interruption requirements when secondary DRX is configured in FR1 and FR2 CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN2 CRs in RP-201198 introducing secondary DRX for FR1-FR2 CA was approved in Rel-16 as TEI16. Corresponding RRM requirements, which comprise interruption requirements due to transtions between active/non-active and non-DRX/DRX are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior when configured with secondary DRX for FR1-FR2 CA will be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1, 8.2.2.2.13, 8.2.2.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

### 8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

#### 8.2.2.1	Introduction

This clause contains the requirements related to the interruptions on PCell and activated SCell if configured, when

up to 7 SCells are configured, de-configured, activated or deactivated, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell or SCell, or

UE-specific CBW is changed on PCell or SCell, or

NR SRS carrier based switching, or

UE dynamic Tx switches between two uplink carriers or

transitions between active and non-active during DRX if configured with DRX-ConfigSecondaryGroup [2], or

transitions from non-DRX to DRX if configured with DRX-ConfigSecondaryGroup [2].

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure according to TS38.331 [2] for SCell addition/release or MAC control signalling according to TS37.340 [17] for SCell activation/deactivation command.

This clause additionally contains requirements related to interruptions at inter-frequency SFTD between PCell in FR1 and neighbour cell in FR2.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

In addition to standalone NR carrier aggregation when no CCA is configured, the requirements in clause 8.2.2. and all subclauses of 8.2.2 apply when the UE is configured with

-A PCell not using CCA in downlink and one or more SCells using CCA in downlink or

-A PCell and one or more SCells using CCA in downlink

----------------------NEXT CHANGE----------------------------

##### 8.2.2.2.13	Interruptions at transitions between active and non-active during DRX when configured with secondary DRX

The requirement in this section shall apply for a UE:

- capable of secondaryDRX-Group [14],

- configured with at least one serving cell in FR1 and at least one SCell in FR2 and

- configured with DRX-ConfigSecondaryGroup [2].

For a UE which does not support per-FR measurement gaps:

- When FR1 serving cells are in non-DRX and FR2 serving cells are in DRX, interruptions on FR1 PCell and the FR1 activated SCell(s) if configured due to transitions from active to non-active and from non-active to active during DRX of FR2 serving cells are allowed with up to 1% probability of missed ACK/NACK when the configured DRX cycle of FR2 serving cells is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured DRX cycle FR2 serving cells is 640 ms or longer. Each interruption shall not exceed X slot as defined in table 8.2.2.2.13-1.

- When FR2 serving cells are in non-DRX and FR1 serving cells are in DRX, interruptions on the activated FR2 SCell(s) due to transitions from active to non-active and from non-active to active during DRX of FR1 serving cells are allowed with up to 1 % probability of missed ACK/NACK when the configured DRX cycle of FR1 serving cells is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured DRX cycle of FR1 serving cells is 640 ms or longer. Each interruption shall not exceed X slot as defined in table 8.2.2.2.13-1.

For a UE which supports per-FR measurement gaps no interruptions are allowed in the above scenarios.

Table 8.2.2.2.13-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |

When serving cells of both FR1 and FR2 are in DRX then no interruption is allowed.

##### 8.2.2.2.14	Interruptions at transitions from non-DRX to DRX when configured with secondary DRX

The requirement in this section shall apply for a UE:

- capable of secondaryDRX-Group [14],

- configured with at least one serving cell in FR1 and at least one SCell in FR2 and

- configured with DRX-ConfigSecondaryGroup [2].

For a UE which does not support per-FR measurement gaps:

- Interruption on FR1 PCell and the FR1 activated SCell(s) if configured due to transitions on FR2 serving cells from non-DRX to DRX when FR1 serving cells are in non-DRX shall not exceed X slots as defined in table 8.2.2.2.13-1.

- Interruption on the activated FR2 SCell(s) due to transitions on FR1 serving cells from non-DRX to DRX when FR2 serving cells are in non-DRX shall not exceed X slots as defined in table 8.2.2.2.13-1.

For a UE which supports per-FR measurement gaps no interruptions are allowed in the above scenarios.

----------------------END OF CHANGES----------------------------