3GPP TSG-RAN WG4 Meeting #96-e	R4-200xxxx

Electronic Meeting, August 17th – 28th 2020

Agenda item:			4.9, 5.4, 6.5.4, 7.19.6

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [96e][313] Demod_Maintenance

Document for:	Information

# Introduction

The scope of this email thread is:

- Rel-15 NR maintenance – UE demodulation and CSI requirements (AI 4.9.1 and 4.9.2)

- LTE maintenance up to Rel-15 - UE demodulation and CSI requirements (AI 5.4.1)

- Note: There are no tdocs submitted in this meeting for:

  - Rel-15 NR maintenance – BS demodulation requirements (AI 4.9.3)

  - LTE maintenance up to Rel-15 - BS demodulation requirements (AI 5.4.2)

  - Rel-16 LTE maintenance (AI 6.5.4)

  - Rel-16 NR maintenance (AI 7.19.6)

Email discussion targets for the 1st round and 2nd round

- 1st round:

  - Collect comments for NR and LTE CRs.

- 2nd round:

  - Collect comments for revised NR and LTE CRs from the 1st round.

# Topic #1: Rel-15 NR maintenance - UE demodulation and CSI requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009540 | ANRITSU LTD | Rel-15 CR with the following changes for TS 38.101-4: - Changed Reference Channel from R.PDSCH.1-1.4 FDD to R.PDSCH.1-1.3 FDD in Table 5.2.2.1.3-3 and Table 5.2.3.1.3-3 |
| R4-2009541 | ANRITSU LTD | Rel-16 Cat A CR of R4-2009540 |
| R4-2010798 | Rohde & Schwarz | Rel-15 CR with the following changes for TS 38.101-4: - Readded R.PDSCH.1-1.4 FDD to the specification |
| R4-2010799 | Rohde & Schwarz | Rel-16 Cat A CR of R4-2010798 |
| R4-2011401 | Qualcomm Incorporated | Rel-15 CR with the following changes for TS 38.101-4: - PDCCH allocation for LTE-NR coexistence PDSCH tests is specified. PDCCH is allocated on symbol 2. - Number of HARQ processes for Test 2-1 in Section 5.2.3.1.1 is fixed. - PDSCH precoding configuration for FR2 SDR tests is fixed. - Editor’s notes are removed in Annex C.3.1 and C.5.1 |
| R4-2009538 | ANRITSU LTD | Rel-15 CR with the following changes for TS 38.101-4: - Resource type of ZP CSI-RS is changed from “Aperiodic” to “Periodic” so that no DCI 1-1 (DL grant, i.e. with PDSCH) transmission is required on slot#1. - Periodicity of Periodic ZP CSI-RS is set to 8 slot with offset 1 so that it is sent on the same timing as Aperiodic NZP-CSI-RS - ZP CSI-RS trigger at ZP CSI-RS configuration is removed. |
| R4-2009539 | ANRITSU LTD | Rel-16 Cat A CR of R4-2009538 |

## Open issues summary

N/A

## Companies views’ collection for 1st round

### Open issues

N/A

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2009540 |  |
| R4-2010798 |  |
| R4-2011401 |  |
| R4-2009538 |  |

## Summary for 1st round

### Open issues

Status summary

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: LTE requirements maintenance (up to Rel-15) - UE demodulation and CSI requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010460 | Ericsson | Rel-14 CR with the following changes for TS 36.101: - Set OP.1 FS3 as OCNG patter for LAA CC for SDR requirements |
| R4-2010461 | Ericsson | Rel-15 Cat A CR of R4-2010460 |
| R4-2010462 | Ericsson | Rel-16 Cat A CR of R4-2010460 |
| R4-2010463 | Ericsson | Rel-15 CR with the following changes for TS 36.101: - Add note PDSCH demodulation requirements with 64QAM for MTC UE is applicable for MTC UE capable of ce-PDSCH-64QAM. |
| R4-2010464 | Ericsson | Rel-16 Cat A CR of R4-2010463 |

## Open issues summary

N/A

## Companies views’ collection for 1st round

### Open issues

N/A

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2010460 |  |
| R4-2010463 |  |

## Summary for 1st round

### Open issues

N/A

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |