3GPP TSG-RAN WG4 Meeting #97-e                                                          R4-2014034

Electronic Meeting, 2 - 13 November 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717.21-11: Addition of DC_5-66_n7

Agenda item:	10.4.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_5A-66A_n7A and DC_5A-66A-66A_n7A as defined in WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 5.x	DC_5-66_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |

### 5.x.2	Co-existence studies

Co-existence studies from lower order DCs show that the IMD3 of the dual UL of DC_66_n7 may fall into the DL of band 5 while there is no own Rx impact of the 3rd band from dual UL of DC_7_n5.

### 5.x.3	∆TIB and ∆RIB values

For DC_5-66_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_5-7-66, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66-n7 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66-n7 | 5 | 0 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.x.4	REFSENS requirements

The MSD values for own Rx impact of the 3rd band stated in 5.x.2 are shown in the following table.

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | 835 | 5 | 25 | 880 | 18.0 | FDD | IMD3 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |

##### ---End of changes---

# Reference

[1]	RP-201728, “WID revision Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, RAN#89-e