3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014242

# Electronic Meeting, 2 – 13 Nov, 2020

Agenda item:	7.8.1.2.1

Source:	Apple Inc.

Title:	UE demodulation requirements with higher BLER

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#96e the UE demodulation requirements for URLLC with higher BLER was discussed and way forward [1] was approved. In this contribution we present our views on the open issues related to UE demod requirements for higher BLER and present simulation results.

# 2. Discussion

## PDSCH Slot Aggregation

The agreements in [1] related to PDSCH slot aggregation in FR1:

- HARQ process number: 2 for FDD and 4 for TDD - Methodology for MCS selection - Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM) - BLER calculation method - BLER = NpacketFail/NpacketTx, where NpacketFail is the number of packets with CRC fail after all transmissions (initial and retransmissions), NpacketTx is the total number of packets transmitted during the test.

The MCS in FR1 is still undecided for requirements with PDSCH slot aggregation. Based on the simulation assumptions, for tests with higher reliability with PDSCH slot aggregation, we provide the simulation results for different MCS.

Table 1: Simulation results with PDSCH slot aggregation in FR1

| SNR @1% BLER |  | MCS: 13 | MCS:16 | MCS:19 |
| --- | --- | --- | --- | --- |
| FDD | 2x2 | -5 | -3.4 | -0.9 |
|  | 2x4 | -8.4 | -7 | -4.8 |
| TDD | 2x2 | -4.5 | -3 | -0.6 |
|  | 2x4 | -7.9 | -6.3 | -4.5 |

Based on the results in Table 1, the target SNR would be reasonable and not very low if requirements are defined with MCS 19.

Proposal #1: Define requirements with PDSCH slot aggregation in FR1 with MCS 19

The agreements in [1] related to PDSCH slot aggregation in FR2:

- Introduce test cases with PDSCH repetition in FR2 with 1% BLER as test metric - Test applicability rule for FR2 - The performance requirements are only applicable for UE supporting FR2 operating bands - Test applicability rule for FR1 and FR2 if UE support both - UE should be tested for both FR1 and FR2 if UE support FR1 and FR2 - Companies are encouraged to provide view on detailed test parameters in the next RAN4 meeting: - Aggregation factor, MCS, Channel bandwidth, SCS, Channel model, TDD pattern, PDSCH Mapping type etc.

In RAN4#96-e it was agreed to introduce requirements with PDSCH slot aggregation in FR2, but the simulation assumptions are still FFS. We propose to re-use the parameters from FR1 test as much as possible. We propose the following simulation parameters for requirements with PDSCH slot aggregation in FR2.

Table 2: Simulation Parameters for PDSCH slot aggregation testcase in FR2

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Antenna configuration |  | 2x2, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PUSCH aggregation factor | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Processes |  | 4 |
| Number of HARQ Transmissions |  | 8 |
| Propagation condition |  | TDLA30-75 |
| MCS Table |  | Table 3 |
| SCS and BW |  | 120KHz, 100MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | DDDSU with S = 10:2G:2U |
| Testing metric |  | Target BLER: 10-2 |

Proposal #2: Define requirements in FR2 with PDSCH slot aggregation re-using most of the parameters from FR1 test, and the following:

Propagation condition: TDLA30-75

TDD Pattern: DDDSU with S=10D:2G:2U

CBW and SCS: 100MHz/ 120KHz

Based on the simulation parameters in Table 2, the simulation results with PDSCH slot aggregation for different MCS are provided in Table 3.

Table 3: Simulation results with PDSCH slot aggregation in FR2

| SNR @1% BLER | 2x2 |
| --- | --- |
| MCS: 13 | -6.5 |
| MCS:16 | -4.3 |
| MCS:19 | -2.5 |

Based on the results in Table 3, MCS 16 is suitable for defining requirement with PDSCH slot aggregation in FR2.

Proposal #3: Define requirements with PDSCH slot aggregation in FR2 with MCS 16

## PDSCH Mapping Type-B and Processing Capability 2

The agreements in [1] related to requirements for PDSCH Mapping type B and UE processing capability 2 in FR1:

- Number of HARQ process for TDD: 2 - MCS: Only MCS 4

For the simulation assumptions agreed in [1] the alignment and impairment results are provided in tables below.

Table 4: Alignment results in FR1 for PDSCH Mapping Type B and Processing Capability 2

| SNR @70% Max TP |  | MCS: 4 |
| --- | --- | --- |
| FDD | 2x2 | -1.38 |
|  | 2x4 | -4.63 |
| TDD | 2x2 | -1.80 |
|  | 2x4 | -4.86 |

Table 5: Impairment results in FR1 for PDSCH Mapping Type B and Processing Capability 2

| SNR @70% Max TP |  | MCS: 4 |
| --- | --- | --- |
| FDD | 2x2 | 0.62 |
|  | 2x4 | -2.63 |
| TDD | 2x2 | 0.20 |
|  | 2x4 | -2.86 |

For requirements in FR2 with PDSCH mapping Type B it was agreed to re-use most of the parameters from FR1 testcase where applicable.

- Test applicability rule for FR2: No need to define the applicability rule. - Test applicability rule for FR1 and FR2: No test applicability rule is needed. - SCS/CBW: 120 kHz/100 MHz - TDD pattern: DDDSU with S = 10D:2G:2U - PDSCH Configuration - Scheduling: No PDSCH in slot 0 within 20 ms - MCS: [MCS4] from table 1. Confirming the MCS depends on the simulation results to ensure a proper SNR value. - Start symbol: 1 - Symbol length: 2 - Max number of HARQ transmissions: 4​ - Number of HARQ process: 8 - Antenna configuration: 2x2, ULA low - Channel model: - Option 1: TDLA30-75 - Other options are not precluded - Test metrics: 70% throughput

Based on the simulation assumptions in [1], the results with MCS 4 in FR2 are provided in table 6 below.

Table 6: Simulation results for PDSCH Mapping Type B in FR2

| SNR @70% Max TP | Alignment | Impairment |
| --- | --- | --- |
| 2x2 (MCS:4) | -2.5 | -0.5 |

The operating SNR with MCS of 4 gives reasonable results and we propose to define requirements in FR2 with PDSCH mapping type B with MCS of 4.

Proposal #4: Define requirements with PDSCH mapping Type-B in FR2 with MCS 4

## Pre-emption Indication

The agreements in [1] related to pre-emption indication:

- The assumption of UE behaviours for buffer flushing and decoding - If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. - URLLC interference modelling - SNR: Same as for eMBB transmission - Structure: Some random data - Pre-emption probability - Option 1: 10% within 1 radio frame - Option 2: 20% within 1 radio frame - eMBB MCS - Option 1: MCS13 in Table 1 - Option 2: MCS4 in Table 1 - Test metric - Option 1: 70% of max T-put - Other options are not precluded

Companies were encouraged to bring simulation results comparing performance with and without HARQ buffer flushing in order to decide the test parameters. The table below shows the results with different options for parameters as discussed in [1].

Table 7: Simulation results for Pre-emption Indication

| SNR @ 70% Max TP |  | Pre-empted Slots: 10 % |  | Pre-empted Slots: 20 % |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without Buffer Flush | With Buffer Flush | Without Buffer Flush | With Buffer Flush |
| MCS:4 | 2x2 | -2.56 | -2.59 | -1.62 | -1.70 |
|  | 2x4 | -5.91 | -5.92 | -5.28 | -5.31 |
| MCS:13 | 2x2 | 6.53 | 5.47 | Inf | 6.66 |
|  | 2x4 | 2.38 | 1.71 | 6.94 | 2.47 |

Based on the results in Table 7, we observe that for MCS 4 there is very small delta in performance with or without buffer flush. It would be easy for UE without correct buffer flush implementation to pass the requirement if introduced with low MCS. On the other hand, with MCS of 13 and 20% pre-empted slots, the UE doesn’t achieve 70% max TP without correct buffer flush. Hence, we propose to introduce requirements for pre-emption indication with 20% pre-empted slots with MCS of 13. Also, test metric of 70% of max throughput seems appropriate for this test.

Proposal #5: Introduce requirements for pre-emption indication with 20% pre-empted slots and MCS of 13.

Proposal #6: Define requirements for pre-emption indication at 70% of maximum throughput.

# 3. Conclusion

In this paper, we provide our views on the remaining open issues related to PDSCH demodulation requirements for URLLC with higher BLER. We also provide simulation results for consideration in defining requirements. Our proposals are captured below:

Proposal #1: Define requirements with PDSCH slot aggregation in FR1 with MCS 19

Proposal #2: Define requirements in FR2 with PDSCH slot aggregation re-using most of the parameters from FR1 test, and the following:

Propagation condition: TDLA30-75

TDD Pattern: DDDSU with S=10D:2G:2U

CBW and SCS: 100MHz/ 120KHz

Proposal #3: Define requirements with PDSCH slot aggregation in FR2 with MCS 16

Proposal #4: Define requirements with PDSCH mapping Type-B in FR2 with MCS 4

Proposal #5: Introduce requirements for pre-emption indication with 20% pre-empted slots and MCS of 13.

Proposal #6: Define requirements for pre-emption indication at 70% of maximum throughput.

# Reference

- R4-2012648, “Way forward for NR UE URLLC performance requirements”, Intel.