3GPP TSG-RAN WG4 Meeting #97-e	R4-2014356

Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation

Title:	Principles for 2-step RACH test cases

Agenda item:	7.18.2.1

Document for:	Discussion

# 1	Introduction

- The work split for 2-step RACH TCs is copied from the WF [1] and shown below.

- When preparing the test cases in draft CRs, we find out that there are many details worth noticing for all source companies. Thus, we prepare this paper in hope of providing a sheet for source companies to check if all details are taken care of.

2	Principles

The 2-step RACH test cases can take 4-step RACH test cases as a base line. The main differences are:

- There is no CSI-RS based non-contention based 2-step RA type random access.

- In the tables for configurations, the PRACH configuration shall be changed to MsgA configurations.

- RSRP-ThresholdSSB shall be replace with msgA-RSRP-ThresholdSSB.

- If there is no RSRP-ThresholdSSB in the original configuration table, msgA-RSRP-ThresholdSSB shall be configured so that the TC can configure the UE correctly.

- In the tests, PRACH transmissions shall be replaced with MsgA transmissions.

Agree on the above principles and check all corresponding details in the CR.

3	Conclusion

Proposal 1: Agree on the above principles and check all corresponding details in the CR.

References

- R4-2012183