3GPP TSG-RAN WG4 Meeting #97-e                 	R4-2014359

# Electronic Meeting, 2nd – 13th Nov., 2020

Agenda item:		7.5.2

Source:	MediaTek Inc.

Title:	Discussion on interruption time for CA with non-aligned frame boundaries

Document for:		Discussion

# 1	Introduction

In RAN #88-e meeting [1], it was agreed to introduce support for unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA:

Agreements: - Introduce support for unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA [RAN1, RAN2] - Misalignment should be limited to ±76800Ts - Signaling support for slot offset if necessary Note: Unaligned frame boundary is only allowed for certain band combination Note: Necessity of signaling support of slot offset should be discussed in RAN1 Note: Blind detection of slot offset is not in scope Note: No optimization for MAC Note: Feature is optional and capability signaling is introduced by RAN2

In RAN #89-e meeting [2], companies further reached consensus on the following way forward:

Concluded way forward: - Rel-16 inter-band CA with non-aligned frame boundaries support the following cases - Case A: Within the same cell group, the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for PCell/PSCell is smaller than or equal to the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for each of the Scells - Case B: Within the same cell group, the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for PCell/PSCell is larger than the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for at least one of the Scells - How to handle the ambiguities, including SMTC window & DRX timing, for Case B is up to WGs - RAN2 to work on the remaining details on the UE capability signaling to support the above cases separately without impacting ASN.1 backward compatibility once the ambiguities for Case B are resolved - For UE capability signaling design, UE capability for Case A is a pre-requisite of UE capability for Case B

In this paper, we discuss the RAN4 spec impact caused by introducing R16 NR inter-band CA with non-aligned frame boundaries.

# 2	Discussion

According to 38.133 [3] 9.1.2 Measurement gap, the configurable measurement gap length (MGL) is summarized below in Table I. It can be seen that MGL is a multiple of 1ms for FR1 and a multiple of 0.5ms for FR2.

Table I: Gap Pattern Configurations in 38.133 [3]

Referring to the Figure 9.1.2-1-(b), and Table 9.1.2-4 in 38.133 [3], it can be seen that a slot partially overlapped with the measurement gap is also included in the interruption time.

Figure 9.1.2-1-(b) in 38.133 [3]

Therefore, following the same logic as Figure 9.1.2-1-(b) in 38.133 [3], SCC slot 0/slot 6 shown in the figure below should also be included in the interruption time in CA with non-aligned frame boundaries.

The following proposal is hence drawn:

Proposal 1:RAN4 to clarify in the spec that for the CA with non-aligned frame boundaries scenario, total interruption time shall also consider and count the time duration of the slot which is partially overlapped with the measurement gap.

# 5	Summary

In this contribution, we have the following proposal:

Proposal 1:RAN4 to clarify in the spec that for the CA with non-aligned frame boundaries scenario, total interruption time shall also consider and count the time duration of the slot which is partially overlapped with the measurement gap.

# Reference

- RP-200791, Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements, Jun., 2020.

- RP-202030, Moderator’s Summary for Email Discussion, MediaTek, Sep., 2020.

- TS 38.133 v16.5.0, 3GPP, Sep., 2020.