3GPP TSG-RAN WG4 Meeting #97-e                                                               R4-2014399

# Electronic Meeting, 2 - 13 Nov., 2020

Source: 	CATT

Title: 	Simulation results for NR HST PRACH demodulation requirement

Agenda item:	7.15.3.2.2

Document for:	Discussion

- Introduction

In the RAN4#96-e meeting, introducing TDLC300-100 and  FO= 0Hz for PRACH with restricted set type A and B in WF [1] was approved as follows:

- TDLC300-100 propagation conditions for long preamble formats - Introduce TDLC300-100 FO= 0Hz for PRACH with restricted set type A and B.

This paper will provide our evaluation results for NR HST PRACH for format 0 with restricted set type A and B for TDLC300-100 FO= 0Hz for simulation alignment.

- Evaluation results

The simulation assumptions in WF [1] are shown in Table A-1 in Annex.

The evaluation results for PRACH format 0 with restricted set type A and B for TDLC300-100 and FO= 0Hz are shown in Table 2-1 and Table 2-2 respectively.

Table 2-1: Evaluation results for PRACH format 0 with restricted set type A

| Format | SCS | Restricted set type | Tx/Rx | Carrier frequency(GHz) | Propagation condition | Frequency offset (Hz) | SNR @ 99% detection probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | Type A | 1T2R | 2.1 | TDLC300-100 | 0 | -8.8 | -6.8 |
| Format 0 | 1.25kHz | Type A | 1T4R | 2.1 | TDLC300-100 | 0 | -14.14 | -12.14 |
| Format 0 | 1.25kHz | Type A | 1T8R | 2.1 | TDLC300-100 | 0 | -16.04 | -14.04 |

Table 2-2: Evaluation results for PRACH format 0 with restricted set type B

| Format | SCS | Restricted set type | Tx/Rx | Carrier frequency(GHz) | Propagation condition | Frequency offset (Hz) | SNR @ 99% detection probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | Type B | 1T2R | 2.1 | TDLC300-100 | 0 | -8.16 | -6.16 |
| Format 0 | 1.25kHz | Type B | 1T4R | 2.1 | TDLC300-100 | 0 | -13.46 | -11.46 |
| Format 0 | 1.25kHz | Type B | 1T8R | 2.1 | TDLC300-100 | 0 | -15.86 | -13.86 |

- Conclusion

This paper provides evaluation results for NR HST PRACH with format 0 with restricted set type A and B for TDLC300-100  and FO= 0Hz for simulation alignment.

- References

- R4-2012676, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- Annex

Table A-1: Simulation assumptions for NR HST PRACH with PRACH format 0

| Parameter | Value |
| --- | --- |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Preamble format | Format 0, |
| SCS | 1.25kHz |
| Ncs | 15 |
| v | Restricted set type A: 0 Restricted set type B: 30 |
| Logical sequence index | Restricted set type A: 384 Restricted set type B: 30 |
| Restricted set type | Type A, Type B |
| Frequency offset | 0Hz |
| Propagation condition | TDLC300-100 |
| Timing error tolerance | 2.55us |
| Carrier frequency (GHz) | 2.1 |
| Test metric | SNR @ 99% detection probability |