3GPP TSG-RAN4#97-e	R4-2014519

2nd November – 13th November 2020

Source: 	Nokia

Title: 					Simulation results for  CA_7B A-MPR.

Agenda Item: 			7.11.1.2

Document for:	Discussion

# 1	Introduction

This contribution presents A-MPR concept for CA_n7B.

# 2	Simulation scenario and assumptions

Additional spurious emission limits for UE co-existence are considered according to [1], as shown in Table 1.

Table 1: Additional spurious emissions for UE co-existence (CA_n7B)

| Frequency range (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2570 MHz – 2575 MHz | +1.6 | 5 MHz |
| 2575 MHz – 2595 MHz | -15.5 | 5 MHz |
| 2595 MHz – 2620 MHz | -40 | 1 MHz |

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

No IBE or EVM was simulated

Comprehensive simulation results were presented in [2].

# 3	A-MPR for Contiguous Allocations

For contiguous allocations four separate regions are defined. Right hand region and bottom left corner are high A-MPR regions (A-MPR = 9 dB). Bottom middle region has A-MPR = 0 dB. The remaining top middle region has intermediate A-MPR = 4 dB. The regions are illustrated in Figure 1.

In addition, three different scenarios are separated based on the combined bandwidth of the CA combination and the position of the channel inside the band.

Figure 1. A-MPR regions for contiguous allocations, with 20+30 MHz CA, QPSK and OFDMA and channel position at the top edge of n7 shown as example.

For all modulations and scs when BWChannel_CA > 25 MHz and Fedge,high = 2570 MHz

IF			RBEnd > NRB_agg 5/6 OR

LCRB > NRB_agg - LCRB + NRB_agg /3

OR

RBEnd < NRB_agg /6 AND LCRB < 5

THEN A-MPR = 9 dB,

ELSE IF	LCRB 2/3 < RBend < NRB_agg 5/6 AND LCRB < NRB_agg /4

THEN A-MPR = 0 dB,

OTHERWISE A-MPR = 4 dB.

When BWChannel_CA <= 25 MHz and Fedge,high = 2570 MHz

IF 			LCRB > NRB_agg - LCRB + NRB_agg /2

THEN AMPR = 6 dB.

OTHERWISE A-MPR = 0 dB.

When Fedge_high <= 2570 MHz - BWChannel_CA, A-MPR = 0 dB.

# 4	A-MPR for Non-contiguous Allocations

Four non-contiguous allocations four different scenarios are separated based on the combined bandwidth of the CA combination and the position of the channel inside the band. A-MPR for these scenarios is shown in Figures 2 – 5.

When BWChannel_CA > 25 MHz and Fedge_high = 2570 MHz,

A-MPR =

18 - 6e-06 B;      			0 <= B <= 5e+05

15.9 - 1.75e-06 B; 	5e+05 < B <= 4.5e+06

Figure 2. A-MPR regions for non-contiguous allocations when BWChannel_CA > 25 MHz and Fedge_high = 2570 MHz

When BWChannel_CA <= 25 MHz and Fedge_high = 2570 MHz,

A-MPR =

11;               				0 <= B <= 1e+06

11.4 - 3.85e-07 B; 	1e+06 < B <= 7.5e+06

9.14 - 8.57e-08 B;  7.5e+06 < B <= 2.5e+07

Figure 3. A-MPR for non-contiguous allocations when BWChannel_CA <= 25 MHz and Fedge_high = 2570 MHz

When Fedge_high <= 2570 MHz - BWChannel_CA and 25 MHz < BWChannel_CA <= 35 MHz,

A-MPR =

11;               				0 <= A <= 2e+06

12.2 - 5.77e-07 A; 	2e+06 < A <= 1.5e+07

3.5;               			1.5e+07 < A <= 3.5e+07

Figure 4. A-MPR for non-contiguous allocations when Fedge_high <= 2570 MHz - BWChannel_CA and 25 MHz < BWChannel_CA <= 35 MHz

When Fedge_high <= 2570 MHz - BWChannel_CA and BWChannel_CA <= 25 MHz,

7.5;              				0 <= A <= 1e+06

7.89 - 3.89e-07 A; 	1e+06 < A <= 1e+07

4.67 - 6.67e-08 A; 	1e+07 < A <= 2.5e+07

Figure 5. A-MPR for non-contiguous allocations when Fedge_high <= 2570 MHz - BWChannel_CA and BWChannel_CA <= 25 MHz

# 5	Conclusion

We presented A-MPR concept for CA_n7B contiguous and non-contiguous allocations.

# 6	Reference

[1]		TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, 3GPP

[2]		R4-2010229 CA_n7B A-MPR Simulation Results, Nokia.