3GPP TSG-RAN4 Meeting #97-e 		R4-2014659

Electronic meeting, 2nd Nov – 13th Nov, 2020

Title:	Discussion on performance requirements for CSI-RS L3 measurements

Source:	Xiaomi

Agenda Item:	7.14.2.1

Document for:	Discussion

- Introduction

In RAN#89-e plenary meeting, the WI of RRM requirements for CSI-RS based L3 measurement has been completed. And the related performance requirements for CSI-RS based L3 measurements should be defined, e.g. CSI-RSRP/CSI-RSRQ/CSI-SINR accuracy and report mapping requirements.

In this contribution, we would like to discuss the performance requirements for CSI-RS based L3 measurements.

- Discussion

In core part phase, the CSI-RS based measurement delay requirements are defined based on the configuration of {PRB = 48 and D = 3}. And 5 samples of measurement is assumed to derive the delay requirements. In addition, according to the simulation results from companies [1-5], it can be generally seen that the CSI-RSRP accuracy can be achieved the same accuracy as defined for SS-RSRP with 5 measurement samples. However, according to the agreements made in last meeting [6], CSI-RS L3 measurement is based on serving cell timing.

- Agreements - UE supports using the serving cell timing for CSI-RS based L3 measurement for intra-frequency measurements in Rel-16 - Note: the measurement degradation can be expected for the case when timing difference is larger than CP and it can be discussed in the performance part

Since the CSI-RS based measurement is based on the serving cell timing, UE may drop partial SSB symbols in one SMTC duration due to timing difference between serving cell and neighbour cell. And the measurement performance is expected to be degraded. Thus, we think it is necessary to add some additional margin when introducing the accuracy requirements for CSI-RS L3 measurement, e.g. additional 1 dB margin.

Figure 1: Timing difference between serving cell and neighbour cell

Proposal 1: The accuracy requirement of CSI-RS L3 measurement can be defined as adding 1dB margin on the basis of SSB based accuracy requirement.

Regarding the reporting mapping for CSI-RS L3 measurement, according to RAN4’s agreement made in RAN4#85 meeting [7], RAN4 conclusion is:

- Reporting range of SS-RSRP and CSI RSRP can be represented with up to 128 reported values (7 bits) and the granularity shall be 1dB. - Reporting range of SS-RSRQ, CSI-RSRQ, SS-SINR and CSI-SINR can be represented with up to 128 reported values (7 bits) and the granularity shall be 0.5dB - The range in the signaling may be larger than the guaranteed accuracy range. - RAN4 will specify the mapping between reported value and measured quantity value in the NR performance work item.

Since the CSI-RS L3 measurement is not supported in Rel-15, therefore, the related agreements for CSI-RS L3 reporting mapping were not implemented in spec 38.133. Thus, the reporting mapping requirement for CSI-RS L3 measurement should be introduced in Rel-16 according to the previous agreements.

Proposal 2: The reporting rang of CSI-RSRP, CSI-RSRQ and CSI-SINR shall be defined according to the agreement in [7].

The CSI-RSRP, CSI-RSRQ and CSI-SINR accuracy requirements are defined with the side conditions on input signal level and SINR (Es/Iot), and the side condition of SS-RSRP/SS-RSRQ/SS-SINR is defined as -6dB and -4dB for intra-frequency and inter-frequency measurement, respectively. And we think the same value can be defined for the side condition of CSI-RSRP/CSI-RSRQ/CSI-SINR.

Proposal 3: The side condition of CSI-RSRP/CSI-RSRQ/CSI-SINR for intra-frequency measurement and inter-frequency measurement in both FR1 and FR2 can be defined as -6dB and -4dB, respectively.

We also prepared the corresponding CRs [8-11] to introduce the CSI-RSRP/CSI-RSRQ/CSI-SINR performance requirements.

- Conclusion

In this contribution, we discuss the performance requirements for CSI-RS based L3 measurements and provide our proposals as follows:

Proposal 1: The accuracy requirement of CSI-RS L3 measurement can be defined as adding 1dB margin on the basis of SSB based accuracy requirement.

Proposal 2: The reporting rang of CSI-RSRP, CSI-RSRQ and CSI-SINR shall be defined according to the agreement in [7].

Proposal 3: The side condition of CSI-RSRP/CSI-RSRQ/CSI-SINR for intra-frequency measurement and inter-frequency measurement in both FR1 and FR2 can be defined as -6dB and -4dB, respectively.

- Reference

[1]	R4-1913451	Simulation results for CSI-RS L3 measurement for FR1, Intel

[2]	R4-1913588	Simulation results for CSI-RS measurement BW, MediaTek

[3]	R4-1914837	Simulation results of CSI-RSRP measurement for NR mobility, Huawei, HiSilicon

[4]	R4-1914462	Simulation results for CSI-RS based measurements, Nokia, Nokia Shanghai Bell

[5]	R4-1910950	Simulation results for CSI-RSRP accuracy, CATT

[6]	RAN4#96e Chairman Report

[7]	R4-1713941	LS on RSRP and signal quality measurement report mapping, RAN4/Ericsson

[8]	R4-2014661	CR on CSI-RSRP performance requirements for CSI-RS based measurements, Xiaomi

[9]	R4-2014662	CR on CSI-RSRQ performance requirements for CSI-RS based measurements, Xiaomi

[10]	R4-2014663	CR on CSI-SINR performance requirements for CSI-RS based measurements, Xiaomi

[11]	R4-2014664	CR on side condition for CSI-RS based measurement, Xiaomi