3GPP TSG-RAN WG4 Meeting #97-e	R4- 2015069

# Electronic Meeting, 2nd to 13th Nov, 2020

Agenda Item:	10.1.2

Source: 	MediaTek Inc.,

Title: 	MSD for CA_n71(2A)

Document for:	Approval

- 1. Introduction

The combination in [1] partial TP had been approved. However MSD(ΔRIBNC) need to be further discussed due to wider uplink bandwidth that makes IMD3, CIM3 and CIM5 fall into secondary receiver. The contribution is to characterize MSD for the CA.

- 2. Discussion

For downlink NCCA, the main aggressor to RX is n71 TX IMD3/CIM3/CIM5 are shown in Figure 1. In this contribution, these cases are required to be analysed: (5, 10), (10, 10), (10, 15) in [MHz]

s

Figure 1. IMD of wanted TX signal and its’ image

Due to the uplink/downlink gap of n71 is 11MHz, for 5+10 MHz, there will be only IM5 of UL CBW=10MHz fall in the receiving range. For 10+10 MHz would be also same condition. For 10+15 MHz, there will be CIM3 of UL CBW=15MHz fall in the receiving range as well as CIM5+PAIMD3 (IMD3 of wanted and its image generated by PA nonlinearity). Here we analyse them separately.

Calculation assumptions are listed in Table 1:

| Parameter | Value | Unit |
| --- | --- | --- |
| n71 filter T/R isolation | 55 | dB |
| CIM3 | -60 | dBc |
| CIM5 | -70 | dBc |
| Measured IMD3 spur level due to wanted and its image at PA output | -17.5 | dBm |
| Front-end loss | 4 | dB |
| Thermal noise at n71 RX ANT port | -164.2 | dBm/Hz |
| Transceiver effective phase noise | -140 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

Hence we got MSD values as below table:

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15 | 50RB+25RB | Wgap = 20.0 | [5] | [4.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.1] |  |
| CA_n71(2A) | 15 | 50RB+50RB | Wgap = 15.0 | [5] | [3.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [2.8] |  |
| CA_n71(2A) | 15 | 50RB+75RB | Wgap = 10.0 | [5] | [2.9] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [2.4] |  |
| CA_n71(2A) | 15 | 75RB+50RB | Wgap = 10.0 | [5] | [7.7] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.6] |  |
| CA_n71(2A) | 15 | 75RB+50RB | Wgap = 10.0 | [5] | [21.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.6] |  |

Table 2: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

3. Reference:

- R4-2011619, TP for TR 37.817-01-01: CA_n71(2A)

4. Text Proposal for TR 38.717-01-01:

- ---Start of changes---

6.x	CA_2DL_n71(2A)_1UL_n71A

6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_n71(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | - | 5,10,15,20 | 5,10,15, 20 |  |  | 30 | 0 |

6.x.2	UE co-existence studies

There are no co-existence issues for this combination.

6.x.3	REFSENS

REFSENS for CA_n71(2A) need to be added in below table of TS 38.101-1. MSD values proposed are tentative values for the RAN4 #96 meeting, and these will be crosschecked and to be concluded at the RAN4 #96 bis meeting.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15 | 50RB+25RB | Wgap = 20.0 | [5] | [4.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.1] |  |
| CA_n71(2A) | 15 | 50RB+50RB | Wgap = 15.0 | [5] | [3.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [2.8] |  |
| CA_n71(2A) | 15 | 50RB+75RB | Wgap = 10.0 | [5] | [3.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.6] |  |
| CA_n71(2A) | 15 | 75RB+50RB | Wgap = 10.0 | [5] | [7.7] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.6] |  |
| CA_n71(2A) | 15 | 75RB+50RB | Wgap = 10.0 | [5] | [21.6] | FDD |
|  |  |  | Wgap = 5.0 | [20] | [3.6] |  |

---End of changes---