3GPP TSG-RAN WG4 Meeting #97-e             	R4-20151119

Electronic meeting, 2nd Nov– 13th Nov, 2020

Agenda item:	7.15.3.2.3

Source:	Samsung

Title:	Discussion and simulation results for NR HST UL timing adjustment

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issues for NR HST UL timing adjustment requirement were discussed and test parameters for related requirement were captured in the WF [1].

In this contribution, we present our view on the remaining issue for UL timing adjustment requirement. Meanwhile, the updated simulation results are provided for alignment purpose.

# 2	Discussion

- SCS/CBW combinations

In the last meeting, it was agreed to introduce UL TA additional scenario X requirement. Regarding the SCS/BW configuration, the following options are considered as

- Option 1: Have requirement for - 15KHz: 10MH/5MHz; 30KHz: 40MHz/10MHz - Option 2: Have requirement for - 15KHz: 5MHz; 30KHz: 10MHz - Option 3: Have requirements for all Rel-15 non-HST PUSCH bandwidth - Other options are not precluded

For UL timing adjustment, SCS/BW combination requirement for 15 KHz SCS/10MHz, 5MHz CBW and 30 KHz SCS/10MHz, 40MHz CBW were introduced for agreed scenarios Y and Z. The similar approach can be applied for scenario X. According the requirement of scenarios Y and Z, the performance different is very minor. Since the requirement of scenario X was agreed to be added into non-HST section, we can apply the same test applicability rule from non-HST section. To reduce the test effort, we prefer to only introduce the requirement of UL timing adjustment for scenario X with SCS/BW combination as 15 KHz/5MHz and 30 KHz/10MHz.

Proposal 1: Only introduce the requirement of UL timing adjustment for scenario X with SCS/BW combination as 15 KHz/5MHz and 30 KHz/10MHz.

- Applicability rules for SCS/BW

RAN4 has defined the test applicability for different SCS and BW combination in Rel-15. For UL timing adjustment, the requirements for 15 KHz SCS/10MHz, 5MHz CBW and 30 KHz SCS/10MHz, 40MHz CBW are defined, it is agreed to use applicability rule to only test the supported SCS/CBW. In that sense, we prefer to re-use non-HST PUSCH applicability rule.

Proposal 2:  Re-use non-HST PUSCH applicability rule for SCS/BW with requirement of UL timing adjustment scenario X

# 3	Simulation results

In this section, the results for UL timing adjustment are provided.

Table 1: Ideal and impairment result for UL timing requirement with scenario X under 120km/h (to be updated)

| SCS,BW | MCS | Antenna Configuration | DMRS configuration | Mapping Type | Symbol length | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz 5MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 8.52 | 10.52 |
| 15KHz 5MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 8.63 | 10.63 |
| 30KHz 10MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 8.17 | 10.17 |
| 30KHz 10MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 8.15 | 10.15 |
| 15KHz 10MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 9.38 | 11.38 |
| 15KHz 10MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 9.61 | 11.61 |
| 30KHz 40MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 8.92 | 10.92 |
| 30KHz 40MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 8.21 | 10.21 |

Table 2: Ideal and impairment result for UL timing requirement with scenario Y under 350km/h

| SCS,BW | MCS | Antenna Configuration | DMRS configuration | Mapping Type | Symbol length | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz 5MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 6.34 | 8.34 |
| 15KHz 5MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 6.32 | 8.32 |
| 30KHz 10MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 6.34 | 8.32 |
| 30KHz 10MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 6.32 | 8.32 |

Table 3: Ideal and impairment result for UL timing requirement with scenario Z under 500km/h

| SCS,BW | MCS | Antenna Configuration | DMRS configuration | Mapping Type | Symbol length | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz: 10MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 6.54 | 8.54 |
| 15kHz: 10MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 6.53 | 8.53 |
| 30kHz: 40MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 6.53 | 8.53 |
| 30kHz: 40MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 6.53 | 8.53 |
| 15KHz 5MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 6.34 | 8.34 |
| 15KHz 5MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 6.33 | 8.33 |
| 30KHz 10MHz | MCS16 | 1x2 | 1+1+1 (l0=2) (2,7,11) | Type A | 14 | 6.34 | 8.34 |
| 30KHz 10MHz | MCS16 | 1x2 | 1+1+1 (0,5,10) | Type B | 14 | 6.33 | 8.33 |

# 4	Conclusion

In this contribution, the view on UL timing requirement for HR HST is provided.

Proposal 1: Only introduce the requirement of UL timing adjustment for scenario X with SCS/BW combination as 15 KHz/5MHz and 30 KHz/10MHz.

Proposal 2: Re-use non-HST PUSCH applicability rule for SCS/BW with requirement of UL timing adjustment scenario X .

# References

R4-2012676, WF on Rel-16 NR HST BS demodulation requirement, Nokia