3GPP TSG-RAN WG4 Meeting #97-e				  											R4-2015249 Online, 2nd Nov. 2020 – 13th Nov. 2020

Source: 	Nokia, Verizon

Title: 		TP for 37.717-31-11 to introduce DC_2-13-66_n77A

Agenda Item: 			10.5.2 [DC_R17_3BLTE_1BNR_4DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-31-11 to introduce DC_2A-13A-66A_n77A, DC_2A-2A-13A-66A_n77A and DC_2A-13A-66A-66A_n77A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	DC_2-13-66_n77A

### 5.1.x.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n77A DC_2A-2A-13A-66A_n77A DC_2A-13A-66A-66A_n77A | DC_2A_n77A DC_13A_n77A DC_66A_n77A |

### 5.1.x.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-13-66-66_n77 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-13-66-66_n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.x.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

*********************** End of the TP ***************************************************

# 3	References