3GPP TSG-RAN WG4 Meeting # 97-e                                  R4-2015556

Electronic Meeting, 2nd – 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on how to support EN-DC band combinations for Roaming UEs

Agenda Item:	7.5.1

Document for:	Approval

Introduction

Currently, some operators choose option 3 (EN-DC) NSA architecture to evolve from 4G to 5G networks. However, due to the lack of global unified ENDC band combinations, UE vendors have to implement lots of EN-DC band combinations to meet global 5G roaming function. Meanwhile, dual UL was assumed as baseline for EN-DC band combinations. It will bring huge costs for UE vendors. This is disadvantage to the popularization of 5G terminals and roaming experience. In this paper, we’d like to discuss this issue.

Discussion

Roaming issue for NSA UE

Global unified frequency band expanded the IMT industry scale, met the terminal’s roaming function and reduced product implementation costs. In the past, UE only need to support a few global frequency band to implement global roaming function, such as band 1, 2, 3, 4, 8. Currently, some operators choose option 3 (EN-DC) NSA architecture to evolve from 4G to 5G networks. However, the evolution direction of frequency band combinations is completely different for every operator because of the different situation about evolution strategy and owned frequency band. The main ENDC band combinations in three regions are listed as below in table 1, table 2 and table 3.

Table 1 The main ENDC band combinations in region 1

| Region 1 | LTE/NR bands | band n1 | band n3 | band n7 | band n8 | band n20 | band n28 | band n38 | band n78 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | band 1 | NA | DC_1_n3 | DC_1_n7 | DC_1_n8 | DC_1_n20 | DC_1_n28 | DC_1_n38 | DC_1_n78 |
|  | band 3 | DC_3_n1 | DC_3_n3 | DC_3_n7 | DC_3_n8 | DC_3_n20 | DC_3_n28 | DC_3_n38 | DC_3_n78 |
|  | band 7 | DC_7_n1 | DC_7_n3 | NA | DC_7_n8 | DC_7_n20 | DC_7_n28 | NA | DC_7_n78 |
|  | band 8 | DC_8_n1 | DC_8_n3 | DC_8_n7 | NA | DC_8_n20 | DC_8_n28 | NA | DC_8_n78 |
|  | band 20 | DC_20_n1 | DC_20_n3 | DC_20_n7 | DC_20_n8 | NA | DC_20_n28 | DC_20_n38 | DC_20_n78 |
|  | band 28 | DC_28_n1 | DC_28_n3 | DC_28_n7 | DC_28_n8 | NA | NA | NA | DC_28_n78 |
|  | band 38 | NA | NA | NA | NA | NA | NA | DC_(n)38 | DC_38_n78 |

Table 2 The main ENDC band combinations in region 2

| Region 2 | LTE/NR bands | band n2 | band n12 | band n25 | band n26 | band n28 | band n41 | band n48 | band n66 | band n71 | band n77 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | band 2 | DC_2_n2 | DC_2_n12 | NA | NA | NA | DC_2_n41 | DC_2_n48 | DC_2_n66 | DC_2_n71 | DC_2_n77 |
|  | band 4 | DC_4_n2 | NA | NA | NA | DC_4_n28 | DC_4_n41 | NA | NA | NA | NA |
|  | band 12 | DC_12_n2 | DC_(n)12 | DC_12_n25 | NA | NA | DC_12_n41 | NA | DC_12_n66 | NA | NA |
|  | band 26 | NA | NA | DC_26_n25 | NA | NA | DC_26_n41 | NA | NA | NA | DC_26_n77 |
|  | band 41 | NA | NA | NA | NA | DC_41_n28 | DC_41_n41 | NA | NA | NA | DC_41_n77 |
|  | band 48 | NA | DC_48_n12 | DC_48_n25 | NA | NA | NA | DC_48_n48 | DC_48_n66 | DC_48_n71 | NA |
|  | band 66 | NA | DC_66_n12 | DC_66_n25 | NA | NA | DC_66_n41 | DC_66_n48 | DC_66_n66 | DC_66_n71 | DC_66_n77 |
|  | band 71 | NA | NA | NA | NA | NA | NA | DC_71_n48 | DC_71_n66 | DC_(n)71 | NA |

Table 3 The main ENDC band combinations in region 3

| Region 3 | LTE/NR bands | band n41 | band n77 | band n78 | band n79 |
| --- | --- | --- | --- | --- | --- |
|  | band 1 | DC_1_n41 | DC_1_n77 | DC_1_n78 | DC_1_n79 |
|  | band 3 | DC_3_n41 | DC_3_n77 | DC_3_n78 | DC_3_n79 |
|  | band 5 | NA | NA | DC_5_n78 | DC_5_n79 |
|  | band 8 | DC_8_n41 | DC_8_n77 | DC_8_n78 | DC_8_n79 |
|  | band 18 | NA | DC_18_n77 | DC_18_n78 | DC_18_n79 |
|  | band 19 | NA | DC_19_n77 | DC_19_n78 | DC_19_n79 |
|  | band 28 | DC_28_n41 | DC_28_n77 | DC_28_n78 | DC_28_n79 |
|  | band 39 | DC_39_n41 | NA | DC_39_n78 | DC_39_n79 |
|  | band 40 | DC_40_n41 | DC_40_n77 | DC_40_n78 | DC_40_n79 |
|  | band 41 | DC_41_n41 | DC_41_n77 | DC_41_n78 | DC_41_n79 |

It can be easily found that almost all possibilities for the ENDC band combinations are introduced into the specifications. No one can find the EN-DC band combinations that will be used as a global unified EN-DC band combination or even in most of the areas in the world. In order to meet the global 5G roaming function, UE vendors have to implement lots of EN-DC band combinations due to the lack of global unified ENDC band combinations.

Observation 1: In order to meet the global 5G roaming function, UE vendors have to implement lots of EN-DC band combinations due to the lack of global unified ENDC band combinations.

Huge costs vs roaming experience

Figure 1 The example UE RF architecture for the EN-DC band combinations

Generally, dual UL was assumed as baseline for EN-DC band combinations. It means tighter requirements on terminal technology implementation and corresponding more expensive costs for UE. If UE need to support a dual Tx EN-DC band combination, additional a pair of RF chain is needed. In order to remove the impact of IMD and harmonic interference, good performance PA, wideband high efficiency antenna, post PA filter and diplexer/duplexer need to be implemented into the terminal. The example UE RF architecture for the EN-DC band combinations is shown in figure 1.

All of the above, the only one EN-DC band combination is considered as an example. However, for the specific ENDC band combination, maybe only a few countries or operators will use them. UE may never roam to that country or network. UE vendor or IMT industry have to pay a disproportionate price to the roaming revenue. Considering so many ENDC band combination globally, it’s unrealistic to ask the roaming UE to implement all of them with dual UL Tx.

Observation 2: UE vendor or IMT industry have to pay an expensive price to the roaming experience, if UE implements all the EN-DC band combinations with dual UL Tx that is only used in the roaming scenario.

Advantage to relax EN-DC UE Tx capability for roaming scenario

Regarding to the implementation of EN-DC band combinations for the roaming scenario, we provide a proposal as below to balance the terminal’s costs and roaming experience.

Proposal 1: UE is allowed to indicate "only single switched UL" for any EN-DC band combinations when roaming.

At least, there are five advantages listed as below.

1) Enhance the UL coverage

As it is known to all, the uplink coverage of a UE is always a bottleneck instead of the uplink capacity. Simultaneous dual Tx doesn’t help improve uplink coverage. On the contrary, UE needs to maintain two uplinks using the limited transmission power. Using single UL transmission can help improve uplink coverage.

2) Decrease the costs when UE roaming

Default dual Tx assumption and the lack of global unified ENDC band combinations seriously increase the costs when UE implement the 5G roaming function. Currently, it’s very hard to specify a global unified ENDC band combination. But we can allow UE to declare and indicate the "only single switched UL" for EN-DC band combinations when roaming. It will drastically reduce the implementation costs.

3) Extend the roaming experience of 5G terminals under NSA networks

Currently, UE doesn’t choose to support ENDC band combinations which consumers have few opportunities to use due to the expensive costs. However, under the roaming scenario, allowing UE to declare and indicate the "only single switched UL" for EN-DC band combinations can help UE vendor reduce the costs and support more ENDC band combinations. Then, the roaming experience is extended under the NSA networks.

4) Increase 5G device market penetration

Allowing UE to declare and indicate the "only single switched UL" for EN-DC band combinations can easily meet the global 5G roaming function. Extensive 5G roaming functions is very important from consumers’ perspective. Global roaming function will increase the market’s confidence on 5G device. Meanwhile, it also helps local operator reduce the 5G terminal’s costs. In the end, it will increase 5G device market penetration rate.

5) No impact on the capability of UE under the local network

This proposal doesn’t affect the UE capability on the local network or target market. This proposal just focus on the roaming scenario.

Based on the current situation, the UE capabilities are listed as below for single UL / dual UL ENDC band combinations.

Table 4 UE transmission capability between local network and roaming scenario

| Local network | Supporting simultaneous dual Tx | Only single switched UL for specific ENDC band combination | Not supporting this band combination |
| --- | --- | --- | --- |
| Roaming scenario | Supporting simultaneous dual Tx | Only single switched UL for any ENDC band combinations | Not supporting this band combination |

From network perspective, most of UE are camping on the local network generally. Thus, this relax on EN-DC UE Tx capability for roaming scenario won’t have any effects from network perspective. Besides, network could choose to use filter indication to avoid compatibility issue.

From UE perspective, it will drastically reduce the implementation costs. Meanwhile, consumers can achieve the 5G global roaming experience.

In total, the global roaming issue has become a pain point for NSA UE. It’s very necessary to solve this roaming issue for NSA UE. If UE is allowed to indicate "only single switched UL" for any EN-DC band combinations when roaming, it can decrease the huge costs for terminals and help accelerate evolution from 4G to 5G.

Summary

Based on the discussion, all the proposals are listed below:

Observation 1: In order to meet the global 5G roaming function, UE vendors have to implement lots of EN-DC band combinations due to the lack of global unified ENDC band combinations.

Observation 2: UE vendor or IMT industry have to pay an expensive price to the roaming experience, if UE implements all the EN-DC band combinations with dual UL Tx that is only used in the roaming scenario.

Proposal 1: UE is allowed to indicate "only single switched UL" for any EN-DC band combinations when roaming.

- References