3GPP TSG-RAN WG4 meeting #97-e	R4-2015832

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.9.4.2.2

Source:	Ericsson

Title:	PDSCH requirements for mDCI/sDCI-based SDM transmission

Document for:	Discussion

# 1	Introduction

RAN1 has introduced one multi-DCI based transmission scheme and 5 single-DCI based transmission schemes as shown in Table 1 and RAN4 has agreed to define at least PDSCH with multi-DCI based scheme and single-DCI based SDM scheme.

Table 1	Multi-TRP transmission schemes in traduced in NR eMIMO WI.

| Transmission scheme | Description |
| --- | --- |
| Multi-DCI based scheme | - Each DCI can schedule PDSCH independently. |
| Single-DCI based SDM scheme | - Transmit one or two transport blocks. - Two TCIs from different TRP can schedule different set of layers, e.g., layer 0 from TRP#0 and layer 1 from TRP#1. - Two PDSCH symbols are fully-overlapped in frequency and time domain. |
| Single-DCI based FDM Scheme A | - Single PDSCH transmission occasion for one transport block scheduled with a single redundancy version. - Some PDSCH PRBs from TRP#0, other PDSCH PRBs from TRP#1. They are non-overlapped in frequency domain, but overlapped in time domain. |
| Single-DCI based FDM Scheme B | - Two PDSCH transmission occasions for one transport block each scheduled with two redundancy versions. - PDSCH from different TRPs are non-overlapped in frequency domain but overlapped in time domain. - Subject to UE capability, UE can combine PDSCH symbols from two PDSCH transmission occasions. |
| Single-DCI based TDM Scheme A | - Subslot-based TDM, i.e., PDSCH from two TRPs are non-overlapped in time domain but overlapped in frequency domain. - Two PDSCH transmission occasions for one transport block each scheduled with two redundancy versions. - UE can combine PDSCH symbols from two PDSCH transmission occasions. |
| Single-DCI based inter-slot TDM | - Slot-based TDM, i.e., PDSCH from two TRPs are non-overlapped in time domain but overlapped in frequency domain. - The number of repetitions is configured by RepNumR16. - Multiple PDSCH transmission occasions for one transport block scheduled with up to four redundancy versions. - UE can combine PDSCH symbols from multiple PDSCH transmission occasions. |

RAN4#96-e agree with the WF on PDSCH demodulation requirements for multi-DCI based and single-DCI based SDM transmission [1]. This contribution discusses the remaining open issues on mDCI/sDCI-based SDM transmission schemes.

# 2	Discussion

## 2.1	Number of test cases

Number of test cases for single-DCI/multi-DCI - Test 1b Single-DCI with positive time offset and overlapping scheduling. - Test 2a Multi-DCI with frequency offset and negative time offset and non-overlapping scheduling - FFS on Test 1a Single-DCI with frequency offset and negative time offset and overlapping scheduling. - Applicability rule: - Option 1 (If Test 1a will be defined): - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b - Interested companies are encouraged to provide simulation results next meeting for tests 1a, 1b and 2a.

We think the options proposed in WF [1] is a good compromise to keep the total number of test cases and verify several FO/TO settings.

Proposal 1: RAN4 defines three test cases with the following applicability rule:

If UE supports single-DCI based SDM transmission scheme (i.e., singleDCI-SDM-scheme) but not support multi-DCI based SDM transmission scheme (i.e., multiDCI-MultiTRP), it should be tested with test case 1a and test case 1b.

If UE supports both single-DCI based SDM transmission scheme (i.e., singleDCI-SDM-scheme) and multi-DCI based SDM transmission scheme (i.e., multiDCI-MultiTRP), it should be tested test 1b and test 2a.

Test 1a/1b/2a is given as follows:

- Test 1a: Single-DCI SDM with frequency offset, with negative time offset, and overlapping scheduling.

- Test 1b: Single-DCI SDM without frequency offset, with positive time offset, and overlapping scheduling.

- Test 2a: Multi-DCI SDM with frequency offset, with negative time offset, and non-overlapping scheduling

## 2.2	PDSCH FRC for mDCI/sDCI-based SDM transmission schemes

According to TS38.101-4 A.1.1, throughput is defined as follows:

The throughput values defined in the measurement channels specified in Annex A, are calculated and are valid per codeword. For multi-codeword transmissions, the throughput referenced in the minimum requirements is the sum of throughputs of all codewords

For multi-DCI based transmission, each DCI schedules the different codeword, and RAN4 agreed to allocate half of PRBs from one DCI and the rest of PRBs from another DCI. Therefore RAN4 need to define new FRC for multi-DCI transmission, where FDD case allocates 26PRBs and TDD case allocates 53PRB for each DCI.

Another difference from Rel-15 PDSCH requirement is the number of TRS configurations. In Rel-15, RAN4 assumes one TRS configuration but Rel-16 multi-TRP transmission requirements require two TRS configurations. It means we also need to configure 2 sets of TRS configurations, and it requires new FRC for single-DCI based SDM schemes also.

Proposal 2: Introduce new FRC for multi-DCI based SDM transmission requirements with 26PRB per codeword for FDD and with 53PRB per codeword for TDD. It also configures two TRS configurations every 20ms.

Proposal 3: Introduce new FRC for single-DCI based SDM transmission requirements with two TRS configurations every 20ms.

We also propose to schedule PDSCH in the TDD special slots as same as the existing PDSCH rank 2 test cases.

Proposal 4: Schedule PDSCH in the TDD special slots.

Table 2 and Table 3 give our proposals for FRC, where we assume

- R.PDSCH.1-3.2 FDD: used for single-DCI based SDM schemes in FDD

- R.PDSCH.1-3.3 FDD: used for multi-DCI based SDM schemes in FDD

- R.PDSCH.2-3.2 TDD: used for single-DCI based SDM schemes in TDD

- R.PDSCH.2-3.3 FDD: used for multi-DCI based SDM schemes in TDD

Table 2	Proposed FDD FRCs for single DCI and multi-DCI based SDM transmissions

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 FDD | [R.PDSCH.1-3.2 FDD] | [R.PDSCH.1-3.3 FDD] |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 26 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 19 | 19 | 19 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 |  | 39936 | 37896 | 18960 |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 42016 | 42016 | 21000 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | CBs | 5 | 5 | 3 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 78624 | 74800 | 37440 |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 82368 | 82368 | 41184 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 39.707 | 39.503 | 18.216 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table 3	Proposed TDD FRCs for single DCI and multi-DCI based SDM transmissions

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-3.1 TDD | [R.PDSCH.2-3.2 TDD] | [R.PDSCH.2-3.3 TDD] |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Allocated resource blocks | PRBs | 106 | 106 | 53 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames |  | 31 | 31 | 31 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 19 | 19 | 19 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 83976 | 77896 | 38936 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 | 27144 | 13576 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 83976 | 83976 | 42014 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 | 4 | 2 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 10 | 10 | 5 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 160272 | 152640 | 76320 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 | 53424 | 26712 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 167904 | 167904 | 83952 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 | 118.188 | 59.126 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

## 2.3	Random precoder configuration

In the Rel-15 PDSCH requirements, PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. However it is not clear whether TE sets the common random PMI for both TRPs or sets random PMI independently for each TRP in the case of multi-DCI based SDM transmission.

Figure 1 shows our simulation result comparing the ‘common random precoder’ and ‘independent random precoder’ with multi-DCI based SDM transmission (FDD SCS=15kHz, 2Rx, TDLA30-10) according to [2]. From the simulation result, we don’t see significant performance difference due to the random PMI setting. Considering the simple TE implementation, we propose to set the common random precoder for two TRP.

Proposal 5: For multi-DCI SDM transmission, the same precoding is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity.

Figure 1	Comparison of random precoder setting for multi-DCI based SDM transmission.

# 3	Summary

Proposal 1: RAN4 defines three test cases with the following applicability rule:

If UE supports single-DCI based SDM transmission scheme (i.e., singleDCI-SDM-scheme) but not support multi-DCI based SDM transmission scheme (i.e., multiDCI-MultiTRP), it should be tested with test case 1a and test case 1b.

If UE supports both single-DCI based SDM transmission scheme (i.e., singleDCI-SDM-scheme) and multi-DCI based SDM transmission scheme (i.e., multiDCI-MultiTRP), it should be tested test 1b and test 2a.

Test 1a/1b/2a is given as follows:

- Test 1a: Single-DCI SDM with frequency offset, with negative time offset, and overlapping scheduling.

- Test 1b: Single-DCI SDM without frequency offset, with positive time offset, and overlapping scheduling.

- Test 2a: Multi-DCI SDM with frequency offset, with negative time offset, and non-overlapping scheduling

Proposal 2: Introduce new FRC for multi-DCI based SDM transmission requirements with 26PRB per codeword for FDD and with 53PRB per codeword for TDD. It also configures two TRS configurations every 20ms.

Proposal 3: Introduce new FRC for single-DCI based SDM transmission requirements with two TRS configurations every 20ms.

Proposal 4: Schedule PDSCH in the TDD special slots.

Proposal 5: For multi-DCI SDM transmission, the same precoding is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity.

# References

- R4-2012662, “Way Forward for general and PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes (eMBB)​​”, Huawei, HiSilicon.

- R4-2012664, “Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes”, Ericsson.