3GPP TSG-RAN WG4 Meeting #97-e	R4-2015990

Electronic Meeting, 17 Nov. – 28 Nov. 2020

Source: 	Intel Corporation

Title: 					Discussion paper on NR-U PRACH requirements

Agenda Item: 			7.1.8.4.3

Document for:	Discussion

# 1	Introduction

During RAN#96-e it was agreed to define requirements for NR-U wideband PRACH [1]. In this contribution we would like to present our view on the PRACH test configuration.

# 2	Discussion

For Rel-16 NR-U enhanced PRACH was introduced – ZC sequence of the following lengths is now supported: LRA = 1151 for SCS = 15kHz and LRA = 571 for SCS = 30kHz. We propose to define the performance requirements for both types of new preambles.

Proposal 1: RAN4 to define the performance requirements for both LRA = 1151 and LRA = 571 preamble length.

We think that the test coverage and test parameters should follow the ones that we have for Rel-15 short preamble. New PRACH test preambles can be defined in the same way as Rel-15 short preamble. Together with same logical sequence index (0), v (0) and set of burst formats (A1, A2, A3, B4, C0, C2), same zeroCorrelationZoneConfig = 11 can be considered. This gives us Ncs = 127 for LRA = 1151 and Ncs = 63 for LRA = 571. The proposed test preambles for wideband are listed in Table 1.

Table 1: Test preambles for wideband PRACH

| Burst format | LRA | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- | --- |
| A1, A2, A3, | 1151 | 15 | 127 | 0 | 0 |
| B4, C0, C2 | 571 | 30 | 63 | 0 | 0 |

Proposal 2: RAN4 to define new test preambles as listed in Table 1

For the configuration parameters, we also propose to reuse the ones used for Rel-15 LRA = 139 preamble. The parameters are listed in Table 2.

Table 2: Wideband PRACH performance test configuration

| Number of TX Antennas | 1 |
| --- | --- |
| Number of RX Antennas | 2, 4, 8 |
| Channel model | AWGN TDL-C fading channel, frequency offset 400 Hz |
| PRACH formats | A1, A2, A3, B4, C0, C2 |

Proposal 3: For NR-U PRACH performance requirements RAN4 to reuse the test configuration parameters used for Rel-15 LRA = 139 preamble as listed in Table 2.

We also do not see reasons to use any other test metrics than already defined for Rel-15 PRACH. So, we propose to keep the following test metrics:

- False alarm probability: 0.1%

- Requirements on missed detection: 99%

- Time error tolerance as specified in Table 8.4.2.1-1 of TS38.104

Proposal 4: For NR-U PRACH performance requirements RAN4 to keep using existing test metrics: the false alarm probability shall be less than or equal to 0.1%, the probability of detection shall be equal to or exceed 99% and time error tolerance requirements given in Table 8.4.2.1-1 of TS38.104

# 3	Conclusion

In this contribution, we discussed the NR-U PRACH requirements. After discussion, the following conclusions are provided:

Proposal 1: RAN4 to define the performance requirements for both LRA = 1151 and LRA = 571 preamble length.

Proposal 2: RAN4 to define new test preambles as listed in Table below

| Burst format | LRA | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- | --- |
| A1, A2, A3, | 1151 | 15 | 127 | 0 | 0 |
| B4, C0, C2 | 571 | 30 | 63 | 0 | 0 |

Proposal 3: For NR-U PRACH performance requirements RAN4 to reuse the test configuration parameters used for Rel-15 LRA = 139 preamble as listed in Table below.

| Number of TX Antennas | 1 |
| --- | --- |
| Number of RX Antennas | 2, 4, 8 |
| Channel model | AWGN TDL-C fading channel, frequency offset 400 Hz |
| PRACH formats | A1, A2, A3, B4, C0, C2 |

Proposal 4: For NR-U PRACH performance requirements RAN4 to keep using existing test metrics: the false alarm probability shall be less than or equal to 0.1% and the probability of detection shall be equal to or exceed 99%

# 4	References

- R4-2012611, Way Forward on NR-U BS demodulation requirement, Ericsson