3GPP TSG-RAN WG4 Meeting #97-e		R4-2016002

E-meeting, 2 – 13 November, 2020

Agenda item:	17.2.2

Source:	Intel Corporation

Title:	CRS-IC for LTE-NR coexistence scenarios

Document for:	Discussion

# Introduction

In RAN #89-e meeting the scope of the Rel-17 UE and BS demodulation performance work scope was discussed. Based on the approved WID [1] the following objectives related to UE demodulation performance were included in the Rel-17 scope:

- MMSE-IRC receiver for suppressing inter-cell interference

- MMSE-IRC receiver for suppressing intra-cell inter-user interference

Same time, there multiple additional objectives were discussed [2], but were deprioritized and not included in the final WID [1]. One of such items is “NR PDSCH demodulation requirements for handling neighbouring cell CRS in LTE-NR coexistence scenarios”. In this paper we provide our views on this scenario and present link-level simulation results.

# Discussion

LTE/NR Dynamic Spectrum Sharing (DSS) (also referred as LTE-NR co-existence scenario in RAN4 specifications) is one of the important 5G features introduced to enable fast and cost-efficient rollout of 5G networks.  In DSS deployments both LTE and NR technologies share the same spectrum (from the network perspective) and the base stations dynamically share the resources based on the specific traffic load conditions.

The DSS deployments are expected to be transparent from the LTE UE perspective and to ensure backward compatibility all LTE signals remain unchanged. In order to ensure that the NR performance is not affected by the always-on LTE CRS signals, specific CRS rate-matching feature was introduced to make sure that NR PDSCH performance is not affected by the LTE CRS signals transmitted from the serving cell.

The respective RAN4 demodulation requirements for noise limited conditions for LTE-NR coexistence scenarios were introduced for FDD case in Rel-15 timeframe and for TDD case in Rel-16 timeframe. Also, requirements were defined for different NR PDSCH durations (9 and 11 symbols). In Figure 1 we provide the illustration of slot structure for LTE-NR coexistence requirements.

| Figure 1. Slot structure for LTE-NR coexistence requirements. |  |
| --- | --- |

The CRS rate-matching solution mainly ensures the NR PDSCH protection from the LTE CRS signals transmitted from the LTE cell co-located with the NR serving cell. Same time, neighboring cell may have a different CRS patterns and the respective CRS transmissions may still collide with the NR PDSCH transmissions from the serving cell. In the dense deployments interference limited conditions are more typical and substantial impact on the NR demodulation performance can be expected (see Figure 2).

Figure 2. CRS interference in LTE-NR coexistence scenarios.

Based on LTE experience, CRS interference can be efficiently mitigated using limited network assistance signaling (i.e. cell ID, number of ports, MBSFN configuration) and CRS-IM receivers provide significant improvement of the PDSCH performance, especially in scenarios with low-medium system loading [3].

One of potential CRS interference mitigation technics is CRS interference cancellation (CRS-IC). The main idea of such processing is the reconstruction of CRS interference signal and cancellation from received signal, followed by the demodulation and decoding of the serving PDSSH signal. Below we provide an initial analysis of the CRS-IC receivers performance for the NR PDSCH. The simulation assumptions are presented in Table 1.

Table 1. Test parameters for CRS-IC analysis

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 |
| Duplex mode |  |  | FDD |  |  |
| CBW |  |  | 10 MHz |  |  |
| SCS |  |  | 15 kHz |  |  |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 |  |  |  |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | Starting symbol (S) |  | 3 |  |  |
|  | Length (L) |  | 9 and 11 |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Loading |  | 100% (i.e. slots 1…19 within 20 ms) | 20% (i.e. slots 3 and 4 within 10 ms) |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Position of the first DM-RS for downlink |  | 3 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |  |  |
|  | LTE carrier BW | MHz | 10 |  |  |
|  | Number of antenna ports |  | 4 |  |  |
|  | v-shift |  | 0 |  | 1 |
| Es/Noc |  |  | Range |  | 10.45 dB |
| FRC |  |  | Rank 1, MCS 4 or MCS 13 or MCS 19 |  |  |
| Channel model |  |  | TDL-A, 30 ns, 10 Hz |  |  |
| Antenna configuration |  |  | 4x2, ULA low |  |  |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |  |  |

| NR PDSCH duration 9 symbols | NR PDSCH duration 11 symbols |
| --- | --- |
| MCS 4 | MCS 4 |
| MCS 13 | MCS 13 |
| MCS 19 | MCS 19 |
| Figure 3. CRS-IC performance analysis |  |

From this analysis we can observe that CRS-IC allows achieving around 4 dB performance improvements for all considered scenarios and should be considered for implementation to ensure robust NR performance under practical DSS deployments.  Therefore, we recommend to specify corresponding NR UE performance requirements in the Rel-17 timeframe. The respective objectives can be added to the newly approved Rel-17 Further enhancement on NR demodulation performance WI, which is scheduled to start in 2021.

Proposal 1:	Include the following objective in the Rel-17 Further enhancement on NR demodulation performance WID: “NR PDSCH demodulation requirements for handling neighbouring cell CRS in LTE-NR coexistence scenario”.

# Conclusion

In this paper we provided view on CRS-IC requirements for LTE-NR coexistence scenarios and made the following proposal:

Proposal 1:	Include the following objective in the Rel-17 Further enhancement on NR demodulation performance WID: “NR PDSCH demodulation requirements for handling neighbouring cell CRS in LTE-NR coexistence scenario”.

# References

- RP-202000 “New WID: Further enhancement on NR demodulation performance”, China Telecom, RAN #89-e, September 2020.

- RP-202035 “Email discussion summary for Rel-17 Demodulation working area [89E][22][RAN4_R17_Demod] (intermediate summary)”, Moderator (Intel Corporation), RAN #89-e, September 2020.

- TR 36.363 “Study on Cell-specific Reference Signals (CRS) interference mitigation for homogenous deployments of LTE”