3GPP TSG-RAN WG4 #97-e		                              	  R4-2016010

Online, November 2nd – 13th, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	n71 35MHz A-MPR and MSD Measurements

Agenda Item:		10.22.3 - UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

# Introduction

A new WI to study the introduction of 35 and 45 MHz channel bandwidth (CBW) has been agreed [1 and study assumptions agreed in WF [2]. This contribution brings measurement data and makes A-MPR and MSD TP proposals.

# Discussion

## n71 35MHz Spectrum Context

Figure 1 summarizes the n71 spectrum for 35MHz downlink (DL) channel bandwidth (CBW) operation and 3 uplink (UL) CBW scenarii:

- Case A (top): case of 20MHz UL CBW using the highest n71 channel. For this case, the upper adjacent channel leakage (ACL) shoulders slightly overlap the n29 SDL band. This case is similar to the current 20MHz DL/20MHz UL situation, no A-MPR is needed, but MSD may require evaluation,

- Case B (middle): case of 20MHz UL CBW using the lowest n71 channel. For this case, the lower ACL shoulders overlap the DL n71 channel by approximately 9MHz. Impact on Reference Sensitivity (REFSENS) is expected,

- Case C (bottom): case of 35MHz UL CBW. For this case, the lower ACL shoulders overlap the n71 DL channel by approximately 24MHz. Strong impact on REFSENS is expected. The upper ACL shoulders completely overlap the n29 SDL band, and partially overlaps n12 DL band. A-MPR is expected as agreed in [2].

Figure 1: n71 35MHz DL CBW spectrum perspective for A) 20MHz uplink CBW at highest channel (top), B) 20MHz uplink CBW at lowest channel (middle), C) 35MHz uplink CBW(bottom)

This paper presents measurement results to evaluate the impact of all cases A, B and C on maximum sensitivity degradation (MSD) and A-MPR. Broader perspective on specification impact can be found in [3].

## Power Amplifier Calibration and Assumptions

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5:-70dBc

- Power amplifier is tested against 2 different supply voltages to account for large back-offs.

- Transmit (Tx) to Antenna n71 filter rejection at 717MHz: 9dB based on [1].

- Duplexer Tx to receiver (Rx) Tx noise rejection in Rx band: 55dB.

- Duplexer Tx to antenna attenuation at band n29: 9dB [4]

## Measurement Results

### Dataset for A-MPR Proposal

Raw power amplifier measured back-off levels obtained with DFT-s-OFDM 35MHz UL waveforms are shown in Figure 2. The data assumes 9dB Tx to antenna filter rejection at band n29. Results are analysed only at Vcc1 due to lack of time. Considering that measured raw back-off exceeds 10dB, additional analysis is required using Vcc2 to obtain a complete picture of the required back-off. For these measurements, RB allocations have been selected to trigger a direct hit into band n29. The plain blue line is a first attempt to define the required A-MPR for band 29 protection for which extra margin is proposed to account for missing Vcc2 data.

Figure 2: NS-27 A-MPR proposal for 50MHz CP-OFDM Class B operation

Observation 1: Measured PA back-off levels using only a single high power mode Vcc1 settings range from 6 to 12 dB for band 29 protection. More analysis is needed using Vcc2.

Proposal 1:

For an IMD3 falling in band 29, the proposed MPR curve coefficients versus RB bandwidth “B” are:

-50dBm/MHz A-MPRCA_IM3 =		14; 		0 ≤ B <1.08

13; 		1.08 ≤ B <2.16

12; 		2.16 ≤ B <3.24

11; 		3.24 ≤ B < 5.04

9.5; 		5.04 ≤ B < 10.08

7.5; 		10.08 ≤ B < 16.56

7; 		16.56 ≤ B

### Dataset for MSD Proposal

Case A: 35MHz DL CBW MSD due to 20MHz UL CBW at highest channel

Tx noise level in 35MHz DL channel measured with 20MHz UL LCRB =25 is -36.8dBm.

Case B: 35MHz DL CBW MSD due to 20MHz UL CBW at lowest channel

Figure 3 shows measured 20MHz UL CBW Tx noise level in 35MHz DL CBW vs LCRB for the case when the UL channel is closest to the DL band.

Figure 3: 20MHz UL CBW Transmitter noise level in 35MHz DL CBW vs LCRB – Case B.

Tx noise level reaches a minimum for LCRB ranging from approximately 25 to 45 RB. We therefore propose to adopt LCRB =25 for MSD evaluation, value at which the transmitter noise level -16.4 dBm, i.e. approximately 20dB higher than in Case A. This is expected as ACL shoulders overlap significantly the DL CBW.

Case C: 35MHz DL CBW MSD due to 35MHz UL CBW.

The measured 35MHz UL CBW Tx noise level in 35MHz DL CBW is plotted in Figure 4.

Figure 4: 35MHz UL CBW Transmitter noise level in 35MHz DL CBW vs LCRB – Case C.

Tx noise level reaches a minimum for LCRB ranging from approximately 20 to 60 RB. At LCRB =25, the noise level is -11.2 dBm, i.e. approximately 25dB higher than in Case A.

The estimated MSD for each case is shown in Table 3 where no A-MPR is assumed, i.e., the PA total transmit power at 0dB back-off is 27dBm. For 35MHz CBW, NRB is assumed equal to 188,92,44RBs for SCS15,30 and 60kHz respectively.

Table 3: n71 35MHz MSD for case A,B,C

The MSD are approximately 3dB for Case A, 17dB for Case B and 22dB for Case C.

Observation 2: For 20MHz UL CBW, the 35MHz DL CBW MSD ranges from 3dB (best case) to 17dB in the worst case of UL channel located closest to the downlink band. For 35MHz UL CBW, the 35MHz DL CBW MSD is approximately 22dB.

Since MSD case A adds no further degradation to the baseline 20MHz UL / 20MHz DL operation, we propose to retain only 2 MSD test points, those corresponding to Case B and Case C. For the introduction of MSD test points, we propose that the support of 35 and 45 MHz Channel bandwidth is optional for band n71 and should use separate table for band specific requirements [3]. This applies to n71 35MHz REFSENS specifications and considering the optionality aspects of this configuration, we propose to minimize the impact of technical specifications by capturing MSD Test Points in a single table format – see Proposal 2.

Proposal 2:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for 35MHz Channel bandwidth

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n71 | 15 | 35 | 20 | 634.5 | 673 | 252 | -71.6 | FDD |
|  | 30 |  |  |  |  | 102 | -71.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| n71 | 15 | 35 | 35 | 634.5 | 680.5 | 252 | -66.5 | FDD |
|  | 30 |  |  |  |  | 102 | -66.6 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we make proposals n71 35MHz A-MPR and MSD test points with Proposal 1 and 2.

Proposal 1:

For an IMD3 falling in band 29, the proposed MPR curve coefficients versus RB bandwidth “B” are:

-50dBm/MHz A-MPRCA_IM3 =		14; 		0 ≤ B <1.08

13; 		1.08 ≤ B <2.16

12; 		2.16 ≤ B <3.24

11; 		3.24 ≤ B < 5.04

9.5; 		5.04 ≤ B < 10.08

7.5; 		10.08 ≤ B < 16.56

7; 		16.56 ≤ B

Proposal 2:

For aggregation of two or more downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-1: Intra-band contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7B | 15 | 52RB+216RB | 20 (RBstart = 32) | 25 (RBstart = 191) | [34] | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

# References

[1]		RP-201722 WID revision: Introduction of channel bandwidths 35MHz and 45 MHz Huawei, HiSilicon, RAN#89e.

[2] 	R4-2011793, WF on UE RF requirements, 	Skyworks Solutions Inc., ZTE, Huawei, Qualcomm, RAN4#96e.

[3] 	R4-2015800, Specification impact of additional 35 and 45MHz channel bandwidths, Skyworks Solutions, Inc., 3GPP TSG-RAN WG4 Meeting #97e.

[4] 	R4-1805749, UE-to-UE coexistence requirements for LTE band 71 and NR band n71 UE, LG Electronics, 3GPP TSG-RAN WG4 Meeting #86BIS.