3GPP TSG-RAN WG4 Meeting #97-e 	R4-2016395

# Electronic Meeting, 2nd – 13th November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1377 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE positioning measurements: UE Rx-Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incomplete requirements, incorrect references |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated requirements, corrected references |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements, incorrect references |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.9.4	UE Rx-Tx time difference measurements

#### 9.9.4.1 Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

#### 9.9.4.2 Requirements Applicability

The requirements in clause 9.9.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band,

-	SRS is configured on at least one of the PCell, PSCell and SCell.

The requirements in clause 9.9.4 apply, regardless of the frequency range in which the UE Rx-Tx time difference measurement is performed.

#### 9.9.4.3 Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

#### 9.9.4.4 Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported UE Rx-Tx time difference measurement values reported shall be based on the measurement report mapping requirements specified in clause 10.1.25.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25.3.

#### 9.9.4.5 Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=maxi⁡(TUERxTx,i) ,

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i

where

i is the index of PRS frequency layer,

TUERxTx,i is the measurement period for PRS frequency layer i as further defined in this clause,

Teffect,i

TUERxTx,i is the measurement period for PRS frequency layer i defined as follows:

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

Where

CSSFi is scaling factor for NR PRS-based positioning measurements as defined in clause 9.1.5.2.5, 9.1.5.2.6, and 9.1.5.2.7 for SA, NE-DC, and NR-DC operation mode, respectively,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,i is the size of the downlink PRS resource in the time domain defined in TS 38.211 [6] and indicated by the higher-layer parameter dl-PRS-NumSymbols specified in TS 37.355 [34],

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= [4],

Tlast	is the measurement duration for the last UE Rx-Tx sample, including the sampling time and processing time, Tlast = Ti + LPRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the DL PRS resource periodicity in frequency layer i,

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in frequency layer i.

If the frequency layer i has more than one DL PRS resource sets with different PRS periodicities, the maximum PRS periodicity among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer.

The time TUERxTx,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE shall discard the UE Rx-Tx time difference measurement if the NTA_offset changes during the UE Rx-Tx measurement period.

The UE shall discard the UE Rx-Tx time difference measurement if the uplink transmission timin, autonomous or based on network-configured TA, changes during the UE Rx-Tx measurement period.

TUERxTx,i can be extended if more than TBD PRS are dropped. The UE Rx-Tx measurement period can be extended if more than TBD SRS are dropped.

TUERxTx,Total can be extended if the SRS periodicity is longer than max(TPRS,i).

The UE Rx-Tx time difference measurement is restarted if the serving cell (PCell, PSCell, or SCell), which is configured with the SRS for positioning, changes during the measurement period. In this case, the UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete. Otherwise, the UE shall continue the on-going UE Rx-Tx time difference measurement after the serving cell change.