3GPP TSG-RAN WG4 Meeting #97-e                                R4-2016725

Online, 2nd Nov-13th Nov, 2020

Source:	KDDI

Title:	TP for TR 37.717-11-21: DC_3A_n28A-n41A

Agenda item:			10.7.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_3A_n28A-n41A according to the request in [1].

# 2. Reference

- RP-201477, New WID on Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	DC_3_n28-n41

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n28A-n41A | DC_3A_n28A DC_3A_n41A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Based on the co-existence studies of DC_3A-n28A and DC_3A-n41, impact on own Rx of 3rd band is as follows.

-    2nd and 3rd order IMD generated by dual uplink of Band 3 + Band n28 may fall into own Rx of band 41.

-    2nd and 3rd order IMDs generated by dual uplink of Band 3 + Band n41 may fall into own Rx of band n28.

### 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n3A-n41A are shown in the following tables.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n41 | 3 | 0.6 |
|  | n28 | 0.5 |
|  | n41 | 0.3 |

Table 6.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n41 | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n41 | 0 |

### 6.x.5	MSD

Table 6.x.5-1 shows the required MSD:

Table 6.x.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n28A-n41A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| DC_3A_n28A_n41A | DC_3A_n28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | n28 | 738 | 5 | 25 | 793 | 5 | N/A | FDD | N/A |
|  |  | n41 | 2518 | 5 | 25 | 2518 | 5 | 27.4 | TDD | IMD2 |
|  |  | 3 | 1715 | 5 | 25 | 1810 | 5 | N/A | FDD | N/A |
|  |  | n28 | 743 | 5 | 25 | 798 | 5 | N/A | FDD | N/A |
|  |  | n41 | 2687 | 5 | 25 | 2687 | 5 | 15.9 | TDD | IMD3 |
|  | DC_3A_n41A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | n41 | 2510 | 5 | 25 | 2510 | 5 | N/A | TDD | N/A |
|  |  | n28 | 735 | 5 | 25 | 790 | 5 | 26.0 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

<End of Text Proposal>