3GPP TSG-RAN WG4 Meeting # 97-e	R4-2016901

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	11.1.3.1

Source: 	Nokia, Nokia Shanghai Bell, ZTE

Title: 	TP to TR 38.921: Clarification of system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7] and RAN4#96-e [8].

An ambiguity has been identified in the agreed simulation assumptions for the coexistence study in TR 38.921 [9], where it is stated in table 4.2.1.1-1:

Table 4.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) 0.9 km (suburban) | Based on cell radius: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location [6] | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

And “cell radius” is illustrated in figure 4.2.1.1-2:

| Uncoordinated Operation: second network’s sites are located at the first network’s cell edge |  |
| --- | --- |

Figure 4.2.1.1-2: Uncoordinated operation

This is different than ITU definition of “cell radius”, from M.2292: A – cell radius, B – ISD.

Hence ITU “cell radius” is equivalent to RAN4 “cell range”. Therefore, it is proposed to use the term “cell range” instead of “cell radius” in table 4.2.1.1-1 to align with figure 4.2.1.1-2 and avoid the ambiguity.

Moreover, it was agreed in [8] that:

- Results with 0.4km ISD at 10GHz can also be provided

  - If simulation results show 5%-tile throughput cannot be achieved with 0.45km ISD at 10GHz.

It is also proposed to capture this agreement into the simulation assumptions as some companies provide the simulation results based on it.

The text proposal to TR 38.921 is provided below for approval.

# 2.	Text proposal

<Start of text proposal>

#### 4.2.1.1	Urban macro

Details on urban macro network layout model are listed in Tables 4.2.1.1-1 and 4.2.1.1-2.

Table 4.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location [6] | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Note 1: Results with 0.4 km ISD at 10GHz can also be provided if simulation results show 5%-tile throughput cannot be achieved with 0.45 km ISD at 10GHz. |  |  |  |

Table 4.2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) | RAN4 has long been using un-coordinated operation in below 6GHz coexistence simulation |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Figure 4.2.1.1-1: Coordinated operation

| Uncoordinated Operation: second network’s sites are located at the first network’s cell edge |  |
| --- | --- |

Figure 4.2.1.1-2: Uncoordinated operation

<End of text proposal>

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011827, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia.

[9]	R4-2015675, “Draft TR38.821 v0.2.0 Parameters for IMT studies on 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”, Huawei.