3GPP TSG-RAN WG4 Meeting #97-e	R4-2017306

Electronic Meeting, 2-13 Nov., 2020

Source: 	vivo, MediaTek

Title: 	Evaluation assumptions for R17 RLM/BFD relaxation

Agenda Item:	12.9.1

Document for:	Approval

Introduction

This contribution provides evaluation assumptions for R17 RLM/BFD relaxation

System level simulation assumptions for mobility impact analysis from RLM/BFD relaxation

The intention of this evaluation is to justify the mobility impact of RLM/BFD relaxation under different scenarios. For FR1 and FR2, the evaluation assumption is provided in Table I.

Table I Evaluation assumptions for mobility impact of RLM/BFD relaxation in FR1 and FR2

| Parameters | Dense urban (UMi) in FR1 | Dense urban (UMi) in FR2 |
| --- | --- | --- |
| UE speed | 3km/h; 30km/h |  |
| Relaxation factor (X) | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. |
| DRX cycle length | 20ms, 40ms |  |
| Layout | Single layer: Macro layer: Hex. Grid |  |
| Moving across Boarder | Option 1: Wrap around as defined in 36.839 [1] Option 2: Bouncing back as defined in 36.839 [1] |  |
| Inter-BS distance | 200m | 200m |
| Carrier frequency and SCS | 4GHz and 30kHz SCS for SSB | 30GHz and 120kHz SCS for SSB |
| Simulation bandwidth | 20MHz (for CORESET0 BW) | 80MHz (for CORESET0 BW) |
| N factor (# of RX beams for FR2) | N.A. | 8 for SSB, 1 for CSI-RS |
| SSB multiplex with Data | No |  |
| LoS probability | According to Table 7.4.2-1 in TR 38.901 [2] |  |
| Pathloss and shadow fading std. | Refer to UMi model in TR38.901 [2] |  |
| Shadow fading correlation distance | 10 for LOS and 13 for NLOS |  |
| BS Tx power (PA power) | 44dBm | 40dBm |
| UE receiver noise figure | 9dB | 13dB |
| UE distribution | 100% outdoor |  |
| BS antenna height | 10m |  |
| UE antenna height | 1.5m |  |
| BS antenna element gain + connector loss | Follow 38.802 [3] Table A.2.1-4 |  |
| UE antenna model | Follow 38.802 [3] Table A.2.1-4 | Follow 38.802 [3] Table A.2.1-4 (M,N,P,Mg,Ng) = Option 1: (1,2,2,1,2) Option 2: (2,2,2,1,2) Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) Note: only one of panels should be selected |
| UE beam sweeping | No | Yes. # of beams: N = 8 |
| BS antenna down-tilt | 15 degree |  |
| BS array configuration | (M,N,P,Mg,Ng) = Option 1: (8,8,2,1,1) Option 2: (2,4,2,1,1) (dH,dV) = Option 1: (0.5, 0.8)λ Option 2: (0.5, 0.9)λ +45°, -45° polarization | (M,N,P,Mg,Ng) = Option 1: (4,8,2,1,1) Option 2: (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| BS beam set | For direction of analog beam steering (in LCS): Azimuth angle φi = [-45, -15, 15, 45] degree Zenith angle θj = [90,115] degree Note: Other options are not precluded. | For direction of analog beam steering (in LCS): Azimuth angle φi = [-60, -42.86, -25.71, -8.57, 8.57, 25.71, 42.86, 60] degree Zenith angle θj = [90, 100, 110, 120] degree Note: Other options are not precluded. |

The UE performs signal and interference measurements based on RLM-RS (i.e. SSB or CSI-RS) to derive a downlink link quality metric, and SINR is considered here for simplicity.

The performance metric of this evaluation can be

- increased latency in RLF triggering (for RLM)

- increased latency in beam failure detection and the initiation of beam recovery procedure (for BFD)

- the CDF curves of Delta SINR comparison between R15 an R17.

- Delta SINR = (estimated SINR_R17– estimated SINR_R15) 	[dB].

As one reference, the definition of Delta SINR in R15 is

•	Delta SINR = (estimated SINR – ideal SINR) 	[dB].

Evaluation assumptions for power saving benefits from RLM/BFD relaxation

This evaluation is going to provide power consumption comparison between relaxed RLM/BFD and R15/R16 baseline. The evaluation is based on power model in annex of TR 38.840 [4] and some further evaluation assumptions are provided in Table II as follows.

Table II Evaluation assumptions for power saving gain of RLM/BFD relaxation

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| longDRXCycle | 40ms for VoIP traffic models; 160ms for FTP Model 3 Note: Other options are not precluded | 40ms for VoIP traffic models; 160ms for FTP Model 3 Note: Other options are not precluded |
| OnDurationTimer | 4ms for VoIP traffic models; 8ms for FTP Model 3 Note: Other options are not precluded | 4ms for VoIP traffic models; 8ms for FTP Model 3 Note: Other options are not precluded |
| Drx_InactiveTimer | 4ms for VoIP traffic models; 20ms for FTP Model 3 Note: Other options are not precluded | 4ms for VoIP traffic models; 20ms for FTP Model 3 Note: Other options are not precluded |
| Slot length | 0.5ms | 0.125ms |
| Relaxation factor (X) | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. |
| PDCCH-WUS configuration | Option 1: Yes WUS offset: 4 slot No wake-up if WUS is missed Option 2: No | Option 1: Yes WUS offset: 4 slot No wake-up if WUS is missed Option 2: No |
| Traffic Model | Option 1: FTP Model 3 0.1Mbytes packet size 200ms mean inter-arrival time (Note: Other options are not precluded) Option 2: VoIP traffic models in TR 38.840 | Option 1: FTP Model 3 0.1Mbytes packet size 200ms mean inter-arrival time (Note: Other options are not precluded) Option 2: VoIP traffic models in TR 38.840 |
| RLM-RS periodicity | SSB: 20 ms CSI-RS: TBD | SSB: 20 ms CSI-RS: TBD |
| RRM-RS periodicity | SSB(SMTC periodicity): 20ms Note: Other options are not precluded. | SSB(SMTC periodicity): 20ms Note: Other options are not precluded. |
| UL transmission | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| BW of RLM/BFD-RSs | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| RS for L1 measurements | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |

- References

[1]  TR 36.839	v11.1.0

[2]  TR 38.901	v15.1.0

[3]  TR 38.802	v14.2.0

[4]  TR 38.840	v16.0.0

[5]  TR 37.910	v16.1.0