3GPP TSG-RAN4 Meeting #97e	R4-2017380

# Electronic Meeting, 2 - 13 Nov, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1372 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on Active BWP switch and Active TCI State Switching requirements - Rel16. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-11-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently during RRC based active BWP switch and TCI state switch UE behavior for case when THARQ > TRRCProcessing is not captured. When THARQ > TRRCProcessing , UE might need additional time to send ACK/NACK due to ongoing switching procedure and the network might wait to switch BWP or TCI state after ACK is received. A longer switching delay is expected in this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add note that longer switching delay is expected if THARQ > TRRCProcessing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for RRC based switch is unclear when THARQ > TRRCProcessing |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2016374 |  |  |  |  |  |  |  |  |

Start of Change 1

### 8.6.3	RRC based BWP switch delay on a single CC

Editor Notes: More than one BWP configurations for RRC-based BWP switch on SCell is FFS.

The requirements in this clause only apply to the case that the BWP switch is performed on a single CC with one or more than one BWP configuration(s) configured.

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where

DL slot n is the last slot containing the RRC command, and

NR Slot length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves changing of SCS.

TRRCprocessingDelay is the length of the RRC procedure delay in ms as defined in clause 12 in TS 38.331 [2], and

TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs. When  THARQ> TRRCprocessingDelay a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [3].

End of Change 1

Unchanged sections omitted

Start of Change 2

### 8.10.5	RRC based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length, where TRRC_processing is the RRC processing delay, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3. The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

-	Tfirst-SSB is time to first SSB transmission after RRC processing by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state.

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ TRRC_processing  +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc) / NR slot length, where TRRC_processing is the RRC processing delay, and TOuk , TL1-RSRP are defined in clause 8.10.3. The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is time to first SSB transmission after RRC processing time at the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

The requirements for RRC based TCI state switch delay apply when only 1 TCI state is configured in RRC TCI state list. When  THARQ> TRRC_processing a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [3].

End of Change 2