3GPP TSG-RAN WG4 Meeting #97-e	R4-2017472

# Electronic Meeting, 2nd – 13th Nov., 2020

Title: 	Simulation assumptions for NR V2X multiple link test case

Source: 	Huawei, HiSilicon

Agenda item:	7.3.7.3

Document for:	Discussion

Introduction

In this contribution, we provide the simulation assumptions for NR V2X demodulation requirements. Companies are encouraged to submit the simulation results. The values with different options or [ ] need be updated in the next RAN4 meeting based on further discussion.

Simulation assumptions

Power imbalance

Methodology for power imbalance test:

  - The target requirement ICS = [-27] dBc

  - Select SINR2 from simulation results for SNR@10% BLER point.

  - Select SNR2 such that SNR2 >> SINR2 (e.g., 5dB higher)

  - Compute SNR1 from the relation: SINR2 = SNR2 – 10*log10(10^((SNR1 + ICS)/10)+1).

Companies are encouraged to provide analysis for ICS value and simulation results for SINR2 based on Table 1 and Table 2.

Table 2.1-1: Test assumptions

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | PRB pairs {0~9} |
|  | PSSCH RMC |  | QPSK (MCS 4), 10RB |
|  | PSCCH RB allocation |  | PRB pairs {0~9}, 2 symbols |
|  | Time offset Note 1 | s | 0 |
|  | Frequency offset Note 2 | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | Option 1: PRB pairs {20~29} Option 2: PRB pairs {40~49} |
|  | PSSCH RMC |  | QPSK (MCS 4), 10RB |
|  | PSCCH RB allocation |  | Option 1: PRB pairs {20~29} Option 2: PRB pairs {40~49}, 2 symbols |
|  | Time offset Note 1 | s | 0 |
|  | Frequency offset Note 2 | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |

Table 2.1-2: Test parameters and RMC to derive SINR2 (For information)

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Synchronization source |  |  | GNSS |
| Channel bandwidth |  | MHz | 20 |
| Allocated resource blocks |  | RB | 10 |
| Subcarrier spacing |  | kHz | 30 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| DMRS symbols for slot with PSFCH |  |  | [#1,#6,#11] |
| DMRS symbols for slot without PSFCH |  |  | [#3,#8] |
| Modulation order |  |  | 2 |
| MCS index |  |  | 4 (308/1024) |
| SCI format 2-A configuration | Payloads | Bits | [35] |
|  | α |  | [1] |
|  | βoffset |  | [3.5] |
| Transport Block Size for slot with PSFCH |  | Bits |  |
| Transport Block Size for slot without PSFCH |  | Bits |  |
| Transport block CRC |  | Bits | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH(Note 2) |  |  |  |
| Binary Channel Bits for slots without PSFCH |  | Bits |  |
| PSFCH resource period (Note 4) |  | Slot | [4] |
| MinTimeGapPSFCH |  | Slot | [2] |
| Antenna configuration |  |  | 1x2 Low |
| Performance metric(Note 3) |  |  | SNRPSSCH@10%BLER |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on every [4] slot. |  |  |  |

Soft buffer test

Table 2.2-1： Simulation assumptions for soft buffer test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Bandwidth |  | MHz | 20 |
| SCS |  | kHz | 30 |
| Subchannel size |  | RBs | Option 1:10 Option 2:15 |
| Active Sidelink UE(s) |  |  | - When A=16 or 24, 16 or 24 UEs transmit signal one by one circularly for every slot and in the first subchannel. - When A=32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The 32nd UE transmit signal in the same slot as the first UE but in the second subchannel. - When A=64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal in the same slot as the first two UEs but in the third subchannel. |
| Sidelink UE i, 0 ≤ i ≤ A | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Time gap between initial transmission and retransmission | Slots | 31 |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 |
| Test metric |  |  | 5% BLER of PSSCH |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: A sidelink UEs should transmit one by one circularly for every slot. Note 4: A depends on maximum number of HARQ processes supported by UE. |  |  |  |

Table 2.2-2： FRC for PSSCH

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Test |  |  | SCH_Test1 |
| Synchronization source |  |  | GNSS |
| Channel bandwidth |  | MHz | 20 |
| Allocated resource blocks |  | RB | Option 1: 10 Option 2: 15 |
| Subcarrier spacing |  | kHz | 30 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| DMRS symbols for slots with PSFCH |  |  | #3, #8 |
| DMRS symbols for slots without PSFCH |  |  | #3, #8 |
| MCS index |  |  | Option 1: 28 Option 3: 13 |
| SCI format 2-A configuration | Payloads | Bits | [35] |
|  | α |  | [1] |
|  | βoffset |  | [2.5] |
| Transport Block Size for slots with PSFCH |  | Bits |  |
| Transport Block Size for slots without PSFCH |  | Bits |  |
| Transport block CRC |  | Bits | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  | Bits |  |
| Binary Channel Bits for slots without PSFCH |  | Bits |  |
| Antenna configuration |  |  | 1x2 Low |
| PSFCH resource period |  | Slot | Option 1:0 Option 2:4 Option 3:1 |
| MinTimeGapPSFCH |  | Slot | [2] |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. |  |  |  |

PSCCH/PSSCH decoding capability test

Table 2.3-1 Simulation assumption for PSCCH decoding demodulation performance

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz/30kHz SCS | Option 1: 20 Option 2: 40 |
| Communication resource pool configuration |  |  | [5] sub-channels, [10] sub-channels |
| PSSCH/PSCCH transmission |  |  | Option 1: Per slots Option 2: Per 2 or 3 slots if PSFCH period=1 |
| DMRS symbol |  |  | 2 DM-RS: symbol #3, #8 |
| PSFCH period |  |  | Option 1:0 Option 2:1 |
| Sidelink UE i, 0 ≤ i ≤ [4 or 9] (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition without external noise |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | QPSK, MCS 4, 10RB |
|  | PSCCH RMC |  | The same as PSCCH decoding |
| Target requirement |  | SNR @ PSCCH BLER 1% |  |
| Note 1: Time offset of Sidelink UE received signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 4: Each UE occupies one sub-channel, hence all the sub-channels are filled by the 5 or 10 active SL UEs. |  |  |  |

PSFCH decoding capability test

This case is only for UEs supporting NACK/ACK feedback mode.

- Option 1:

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Active cell(s) |  |  | None |  |  |  |
| Bandwidth |  | MHz | 20 |  |  |  |
| SCS |  | kHz | 30 |  |  |  |
| PSFCH resource allocation(k) |  | RBs | Option 1: 50 (numSubchannel=5, PSFCH periodicity=1) Option 2: 51 (numSubchannel=1, PSFCH periodicity=1) |  |  |  |
| Sidelink UE i, 0 ≤ i ≤ A-1 | Sidelink Transmissions |  | PSFCH |  |  |  |
|  | Timing offset (Note 1) | s | 0 |  |  |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |  |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |  |  |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |  |  |  |
|  | at antenna port | dBm/15kHz | -85 |  |  |  |
|  | Antenna configuration |  | 1x2 |  |  |  |
|  | PSFCH resource allocation |  |  |  | RB index | CS pair index |
|  |  |  | A<64 |  | i | 0 |
|  |  |  | A=64 | 0≤i≤k-1 | i | 0 |
|  |  |  |  | k＜i≤63 | i-k | 3 |
|  | PSFCH transmission period | Slot | 1 |  |  |  |
|  | Test metric |  | Pr (ACK miss)=1%, Pr (DTX to ACK) <1% Pr (NACK to ACK) <0.1%. |  |  |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Candidate values of A are {5, 15, 25, 32, 35, 45, 50, 64} and A is equal to maximum number of PSFCHs received in a slot supported by UE. |  |  |  |  |  |  |

- Option 2:

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Active cell(s) |  |  | None |  |
| Bandwidth |  | MHz | Option 1:40 Option 2:30 |  |
| SCS |  | kHz | 30 |  |
| Sidelink UE i, 0 ≤ i ≤ A-1 | Sidelink Transmissions |  | PSFCH |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |  |
|  | at antenna port | dBm/15kHz | -85 |  |
|  | Antenna configuration |  | 1x2 |  |
|  | PSFCH resource allocation |  | RB index | CS pair index |
|  |  |  | i | 0 |
|  | PSFCH transmission period | Slot | 1 |  |
|  | Test metric |  | Pr (ACK miss)=1%, Pr (DTX to ACK) <1% Pr (NACK to ACK) <0.1%. |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Candidate values of A are {5, 15, 25, 32, 35, 45, 50, 64} and A is equal to maximum number of PSFCHs received in a slot supported by UE. |  |  |  |  |