3GPP TSG-RAN WG4 Meeting #97-e	R4-2017526

# Electronic Meeting, 2nd Nov - 13th Nov, 2020

Title: 	Simulation assumptions for NR URLLC BS FR2 performance requirements test cases

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2

Document for:	Approval

Introduction

This document provides the summary of the simulation assumptions for the NR URLLC BS FR2 performance requirements alignment for the RAN4 #97-e meeting, the simulation assumptions are based on the agreements from [1]:

Simulation assumptions

- PUSCH demodulation requirements for high reliability with higher BLER for FR2

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| SCS/BW |  | Both of 60 kHz and 120 kHz for both of 50 MHz and 100 MHz |
| Uplink-downlink allocation for TDD |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna configuration |  | 1x2, Low |
| Number of layers |  | 1 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0,3,0,3 (Note 1) |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Starting symbol | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n8 (Note 2) |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hoping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Propagation condition and correlation matrix |  | TDLA30-300 Low |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |
| Note 1: The effective RV sequence is {0,2,3,1} with slot aggregation Note 2: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary. |  |  |

- PUSCH demodulation requirements for PUSCH mapping Type B

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Transform precoding |  |  | Disabled |
| SCS/BW |  |  | Both of 60 kHz and 120 kHz for both of 50 MHz and 100 MHz |
| Uplink-downlink allocation for TDD |  |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna configuration |  |  | 1x2, Low |
| Number of layers |  |  | 1 |
| HARQ | Maximum number of HARQ transmissions |  | 1 |
|  | RV sequence |  | 0 |
| DM-RS | DM-RS configuration type |  | 1 |
|  | DM-RS duration |  | Single-symbol DM-RS |
|  | Additional DM-RS position |  | FFS |
|  | Number of DM-RS CDM group(s) without data |  | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  | DM-RS port(s) |  | {0}, {0, 1} |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type |  | B |
|  | Starting symbol |  | 0 |
|  | Allocation length |  | 2, 4 or 7 |
|  | PUSCH aggregation factor |  | 1 |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |
|  | Frequency hoping |  | Disabled |
| Code block group based PUSCH transmission |  |  | Disabled |
| Propagation condition and correlation matrix |  |  | TDLC300-100 Low |
| PT-RS configuration |  | Frequency density (KPT-RS) | Disabled |
|  |  | Time density (LPT-RS) | Disabled |
| Testing metric |  |  | 70% maximum throughput |

FRC

- PUSCH demodulation requirements for high reliability with higher BLER

| Reference channel | G-FR2-A3A-3 | G-FR2-A3A-3 | G-FR2-A3A-4 | G-FR2-A3A-4 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 99/1024 | 99/1024 | 99/1024 | 99/1024 |
| Payload size (bits) | 1224 | 2472 | 608 | 1224 |
| Transport block CRC (bits) | 16 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 1240 | 2488 | 624 | 1240 |
| Total number of bits per slot | 12672 | 25344 | 6144 | 12672 |
| Total symbols per slot | 6336 | 12672 | 3072 | 6336 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0= 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |

- PUSCH demodulation requirements for PUSCH mapping Type B

| Reference channel | G-FR2-A3A-x |  |  | G-FR2-A3A-x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 60 | 60 | 60 | 60 | 60 | 60 |
| Allocated resource blocks | 66 | 66 | 66 | 132 | 132 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 1 | 3 | 6 | 1 | 3 | 6 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 308/1024 | 99/1024 | 99/1024 | 308/1024 | 99/1024 | 99/1024 |
| Payload size (bits) | 480 | 456 | 928 | 984 | 928 | 1864 |
| Transport block CRC (bits) | 16 | 16 | 16 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 496 | 472 | 944 | 1000 | 944 | 1880 |
| Total number of bits per slot | 1584 | 4752 | 9504 | 3168 | 9504 | 19008 |
| Total symbols per slot | 792 | 2376 | 4752 | 1584 | 4752 | 9504 |

| Reference channel | G-FR2-A3A-x |  |  | G-FR2-A3A-x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks | 32 | 32 | 32 | 66 | 66 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 1 | 3 | 6 | 1 | 3 | 6 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 308/1024 | 99/1024 | 99/1024 | 308/1024 | 99/1024 | 99/1024 |
| Payload size (bits) | 224 | 192 | 456 | 480 | 456 | 928 |
| Transport block CRC (bits) | 16 | 16 | 16 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 240 | 208 | 472 | 496 | 472 | 944 |
| Total number of bits per slot | 768 | 2304 | 4608 | 1584 | 4752 | 9504 |
| Total symbols per slot | 384 | 1152 | 2304 | 792 | 2376 | 4752 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0= 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |  |

Reference

[1] R4-2017525 WF on URLLC BS performance requirement with higher BLER, Huawei