3GPP TSG-RAN4 Meeting #98-e	R4-2100091

# Electronic Meeting, 25th Jan 2021 – 5th Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CA_n1-n3, CA_n1-n78, CA_n3-n78 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R17_2BDL_xBUL |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introducing various CA combinaions including CA_n1-n3, CA_n1-n78, CA_n3-n78 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following CA combinations will be included: CA_n1B-n3A CA_n1A-n3(2A) CA_n1A-n78A CA_n1A-n78C CA_n3A-n78A CA_n3A-n78C CA_n3A-n78(2A) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above combinations won’t be included in specification for operator deployment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5A.3.1, 7.3A.4, 7.3A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

#### 5.5A.3.1	Configurations for inter-band CA (two bands)

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
| CA_n1B-n3A | CA_n1A-n3A | n1 | See CA_n1B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n1 | See CA_n1B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n1A-n3(2A) | CA_n1A-n3A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | See CA_n3(2A) bandwidth combination set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n3 | See CA_n3(2A) bandwidth combination set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n7A | CA_n1A-n7A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n1A-n7B | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A | CA_n1A-n8A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A | CA_n1A-n28A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n40A | CA_n1A-n40A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n1A-n41A | CA_n1A-n41A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n77A | CA_n1A-n77A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n78A | CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 2 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 2 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78C | CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n79A | CA_n1A-n79A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n1A-n79C | CA_n1A-n79A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A | CA_n2A-n5A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48A | CA_n2A-n48A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 501 | 601 |  | 801 | 901 | 1001 |  |
| CA_n2A-n48C | CA_n2A-n48A CA_n48C | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48(2A) | CA_n2A-n48A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48(A-C) | CA_n2A-n48A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(A-C) Bandwidth Combination Set 0 in Table 5.5A.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n2A-n66A | CA_n2A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n77A | CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n77(2A) | CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n78A | CA_n2A-n78A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n2A-n78(2A) | CA_n2A-n78A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7A | CA_n3A-n7A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n3A-n7A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n3A-n7B | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A | CA_n3A-n8A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A | CA_n3A-n28A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n38A | CA_n3A-n38A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n38 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n3A-n40A | CA_n3A-n40A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n3A-n41A | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n3A-n41C | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n41(2A) | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A | CA_n3A-n77A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n77(2A) | CA_n77(2A) CA_n3A-n77A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A | CA_n3A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n78C | CA_n3A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78(2A) | CA_n78(2A) | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n79A | CA_n3A-n79A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n3A-n79C | CA_n3A-n79A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n7A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n5A-n7B | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n25(2A) | CA_n5A-n25A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_48(2A) in Table 5.5A.2-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48C | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C in Table 5.5A.1-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A | CA_n5A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n5A-n66(2A) | CA_n5A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n77A | CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n77(2A) | CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n78A | CA_n5A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n78C | CA_n5A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n79A | CA_n5A-n79A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n5A-n79C | CA_n5A-n79A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A | CA_n7A-n25A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n7A-n25(2A) | CA_n7A-n25A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25A | CA_n7A-n25A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A) | CA_n7A-n25A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n28A | CA_n7A-n28A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n7B-n28A | - | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 |  | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  |  |
| CA_n7A-n66(2A) | CA_n7A-n66A | n7 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66 | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A) | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n78A | CA_n7A-n78A | n7 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n7A-n78(2A) | CA_n7A-n78A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n78A | CA_n7A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n7(2A)-n78(2A) | CA_n7A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n28A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n8A-n39A | CA_n8A-n39A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n8A-n40A | CA_n8A-n40A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n8A-n41A | CA_n8A-n41A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n8A-n75A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n78A | CA_n8A-n78A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n79A | CA_n8A-n79A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n20A-n28A | CA_n20A-n28A | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n20A-n75A | - | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n20A-n78A | CA_n20A-n78A | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n38A | CA_n25A-n38A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25(2A)-n38A | CA_n25A-n38A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n41A | CA_n25A-n41A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25(2A)-n41A | CA_n25A-n41A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n41C | CA_n25A-n41A CA_n41C | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A) | CA_n25A-n41A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n48(2A) | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48C | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A | CA_n25A-n66A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n66(2A) | CA_n25A-n66A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66A | CA_n25A-n66A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 |  | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66(2A) | CA_n25A-n66A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71A | CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71(2A) | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n77A | CA_n25A-n77A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n78A | CA_n25A-n78A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n78(2A) | CA_n25A-n78A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n78A | CA_n25A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25(2A)-n78(2A) | CA_n25A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n46A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n28A-n40A | CA_n28A-n40A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n28A-n41A | CA_n28A-n41A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n28A-n50A | CA_n28A-n50A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n50 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 801 |  |  |  |
| CA_n28A-n75A | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n75A | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n75 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n28A-n77A | CA_n28A-n77A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n28A-n77(2A) | CA_n77(2A) CA_n28A-n77A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A | CA_n28A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n78(2A) | CA_n78(2A) CA_n28A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66A | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n29A-n66B | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66(2A) | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n70A | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n38A-n66A | CA_n38A-n66A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n38A-n66(2A) | CA_n38A-n66A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n38A-n78A | CA_n38A-n78A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n38A-n78(2A) | CA_n38A-n78A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n79A | CA_n28A-n79A | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n34A-n79A | CA_n34A-n79A | n34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n39A-n40A | CA_n39A-n40A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n39A-n41C | CA_n39A-n41A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n41(2A) | CA_n39A-n41A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n79A | CA_n39A-n79A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n40A-n41A | CA_n40A-n41A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n40A-n41C | CA_n41C CA_n40A-n41A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n40 | Refer to CA_n41C Bandwidth combination set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n40A-n78A | CA_n40A-n78A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n40A-n78(2A) | CA_n40A-n78A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n40A-n79A | CA_n40A-n79A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n41A-n50A | CA_n41A-n50A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n50 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 801 |  |  |  |
| CA_n41A-n66A | CA_n41A-n66A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n41(2A)-n66A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 inTable 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n41C-n66A | - | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n41C-n66A | CA_n41C CA_n41A-n66A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n41A-n71A | CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71B | - | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71(2A) | - | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n71A | CA_n41C CA_n41A-n71A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n71A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n71B | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n71B | - | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n77A | CA_n41A-n77A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41(2A)-n77A | CA_n41A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41C-n77A | CA_n41A-n77A CA_n41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n77(2A) | CA_n41A-n77A | n41 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n41A-n78(2A) | - | n41 |  | 10 | 15 | 50 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n79A | CA_n41A-n79A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  | 1 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n41C-n79A | CA_n41A-n79A CA_n41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n46A-n48A | CA_n46A-n48A | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48A | CA_n46A-n48A | n46 | See CA_n46B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48A | CA_n46A-n48A | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48A | CA_n46A-n48A | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46E-n48A | CA_n46A-n48A | n46 | See CA_n46E Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n66A | - | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n48A-n66A | CA_n48A-n66A | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 501 | 601 |  | 801 | 901 | 1001 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48C-n66A | CA_n48A-n66A | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48(2A)-n66A | CA_n48A-n66A | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48(A-C)-n66A | CA_n48A-n66A | n48 | See CA_n48(A-C) Bandwidth Combination Set 0 in Table 5.5A.2-2 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n50A-n78A | CA_n50A-n78A | n50 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 801 |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n66A-n70A | - | n66 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A | - | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A | - | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A | CA_n66A-n71A | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71(2A) | - | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A | CA_n66A-n71A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n66B-n71A | CA_n66A-n71A | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77A | CA_n66A-n77A | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66(2A)-n77A | CA_n66A-n77A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66A-n77(2A) | CA_n66A-n77A | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77(2A) | CA_n66A-n77A CA_n77(2A) | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n77(2A) | CA_n66A-n77A CA_n77(2A) | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n78A | CA_n66A-n78A | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66A-n78(2A) | CA_n66A-n78A | n66 | 5 | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n78A | CA_n66A-n78A | n66 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66(2A)-n78(2A) | CA_n66A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n70A-n71A | CA_n70A-n71A | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n77A | CA_n71A-n77A | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n71A-n78A | CA_n71A-n78A | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n71A-n78(2A) | CA_n71A-n78A | n71 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n75A-n78A | - | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n75A-n78(2A) | - | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n76A-n78A | - | n76 | 5 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n77A-n78A2 |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n77A-n79A | CA_n77A-n79A | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n78A-n79A | CA_n78A-n79A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n78A-n92A | CA_n78A-n92A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n92 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n78(2A)-n92A | CA_n78A-n92A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n92 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE 2: The minimum requirements for intra-band contiguous or non-contiguous CA apply. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Next changes >>

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for a band in frequency range 1 if it is impacted by UL harmonic interference from another band in frequency range 1 of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-1 with uplink configuration specified in Table 7.3A.4-2.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n1 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n2 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n2 | n771, 2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 15.5 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| 2 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n3 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
|  | n781,2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.1 | 17.9 | 16.9 | 16.1 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n5 | n774, 5 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 |  | 2.8 | 2.3 | 2.1 |
| n5 | n776,7 |  | 10.4 | 8.9 | 7.8 | 7.4 | 6.5 | 4.7 | 3.7 | 3 |  | 2.35 | 1.7 | 1.2 |
| n5 | n784,5 |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| n8 | n311 | N/A | N/A | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |
|  | n418,9 |  | 13.0 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
|  | n784,5 |  | 10.8 | 9.1 | 8.0 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
|  | n796,7 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| n20 | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n25 | n481,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n25 | n771,2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 15.5 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| 25 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n28 | n18,9 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
|  | n501,2 |  | 19.8 | 18.0 | 16.8 |  |  | 13.8 | 12.8 | 12.0 |  | 10.8 |  |  |
|  | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 | 19.7 | 18.7 |  |  |  |  |  |
|  | n776,7 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
|  | n786,7 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n66 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | n771, 2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 15.3 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| n66 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n71 | n2510 | 10 | 7.5 | 6 | 5.1 |  |  |  |  |  |  |  |  |  |
|  | n414,5 |  | 10.8 | 9.1 | 8.0 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
|  | n708,9 | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |  |
| n92 | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n77 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n2 | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n2 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n2 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n3 | n77 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n3 | n78 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n5 | n77 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n5 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 | n78 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 | n79 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| n20 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n25 | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n25 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | n2 |
| n25 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n28 | n1 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n28 | n50 |  | 25 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 |  |  |
| n28 | n75 | 12 | 25 | 36 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |
| n28 | n77 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n28 | n78 |  | 10 | 15 | 20 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| n66 | n48 | 12 | 25 | 36 | 50 |  |  | 100 | 128 | 160 |  | 200 | 200 | 200 |
| n66 | n77 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n66 | n78 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n71 | n25 | 84 | 84 | 84 | 84 |  |  |  |  |  |  |  |  |  |
| n71 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n71 | n70 | 8 | 16 | 20 | 20 | 20 |  |  |  |  |  |  |  |  |
| n92 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Next changes >>

7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same NR CA configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3A.6-1 with uplink configuration of the agressor band specified in Table 7.3A.6-2.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n1 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | 1.4 |  |  |  |  |  |  |
| n1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 |  |  |
| n1 | n41 |  | 6.1 | 6.1 | 6.1 |  |  | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 |
| n3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| n38 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 |  | 4.0 | 3.9 | 3.8 |
| n40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | n1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |
| n7 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n41 | n25 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n38 | n25 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n411 | n66 | 3.5 | 3.5 | 3.5 | 3.5 |  |  | 3.5 |  |  |  |  |  |  |
| n41 | n77 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n41 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n78 | n71 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |
| n78 | n38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | n401 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 |  |  |
| n78 | n411 |  | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 |
| n783 | n79 |  |  |  |  |  |  | 2 | 2 | 2 |  | 2 |  | 2 |
| n79 | n783 |  | 2.6 | 2.6 | 2.6 |  |  | 2.6 | 2.6 | 2.6 |  | 2.6 | 2.6 | 2.6 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.6.2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| n1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 |  |  |
| n1 | n41 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n3 | n41 | 15 |  | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n38 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n40 | n1 | 30 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n41 | n1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | n3 | 30 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |
| n7 | n3 | 15 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |
| n41 | n25 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n38 | n25 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n41 | n66 | 30 | 128 | 128 | 128 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |
| n41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n41 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n7 | 30 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |
| n78 | n38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 30 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  | 270 |  |  |
| n78 | n41 | 30 |  | 270 | 270 | 270 |  | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |
| n783 | n79 | 30 |  |  |  |  |  | 270 | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n783 | 30 |  | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>