3GPP RAN WG4 Meeting #98e		R4-2100094

Electronic Meeting, January 25-February 5, 2021

Agenda item:	9.25.1.1

Source:	Murata Manufacturing Co., Ltd.

Title:	Multi-band relaxation for band n262

WI/SI:	NR_47GHz_Band-Core

Release:	Rel-17

Document for:	Discussion

1	Introduction

The work item introducing the new FR2 47GHz band was approved in RAN #88-e [1]. It was discussed during RAN4 #97-e [2] and a WF was approved [3] where companies are encouraged to provide the values for MBR for n262. In this contribution we provide our inputs to MBR value for n262 band.

2	Discussion

In order to estimate the multi-band relaxation value, we need to consider the impact of antenna performance when band n262 is added. Then we investigated the following three type antennas for this discussion.

- 39/47GHz antenna which supports n260/n259/n262

- 47GHz antenna which supports n262

- 39GHz antenna which supports n260/n259

2.1	Analysis model of antenna

Antenna structure and placement are assumed as the Figure 1.

Figure 1. Antenna size and placement in UE.

In case of 47GHz single band, we can shrink antenna size, but from antenna gain point of view, large antenna size makes antenna gain and directivity higher, so that we set the same antenna size for each type antenna.

2.2	Peak gain and coverage

The following result shows antenna gain at peak and 50%-tile CDF.

Table 1. Peak and 50%-tile CDF gain of 39/47GHz antenna

|  | 39/47GHz antenna |  |  | 47GHz antenna | 39GHz antenna |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | n260 | n259 | n262 | n262 | n260 | n259 |
| Peak | 11.3dBi | 10.8dBi | 11.5dBi | 12.3dBi | 11.4dBi | 11.2dBi |
| 50%-tile | 5.4dBi | 4.5dBi | 4.3dBi | 4.8dBi | 5.2dBi | 4.7dBi |

| 39/47GHz antenna | 47GHz antenna | 39GHz antenna |
| --- | --- | --- |

Figure 2. CDF plots of each antennas

Based on our investigation, we summarized antenna gain degradation of multi-band antenna as below.

Table 2. Gain degradation of multi-band antenna

| NR band | Peak | 50%-tile CDF | Note |
| --- | --- | --- | --- |
| n262 | 0.8dB | 0.5dB | For UE that supports n262 + n260/n259 |
| n260 | 0.1dB | 0.0dB | For UE that exclusively supports n260/n259 + n262 |
| n259 | 0.4dB | 0.2dB |  |

For n260 and n259, gain degradation is smaller than current multi-band relaxation specifications (MBP,n=0.5dB, MBS,n=0.4dB).

Observation 1:	For n260 and n259, gain degradation is smaller than current multi-band relaxation specifications (MBP,n=0.5dB, MBS,n=0.4dB).

Proposal 1:	MBP,n for n262 is 0.8dB, MBS,n for n262 is 0.5dB.

3	Conclusions

This contribution provides our view on the multi-band relaxation for n262:

Observation 1:	For n260 and n259, gain degradation is smaller than current multi-band relaxation specifications (MBP,n=0.5dB, MBS,n=0.4dB).

Proposal 1:	MBP,n for n262 is 0.8dB, MBS,n for n262 is 0.5dB.

4	References

RP-201129 "New WID on introduction of NR 47 GHz band”, T-Mobile

R4-2016971 "Email discussion summary for [97e][130] NR_47GHz_Band”, Nokia

R4-2016880 "WF on multi-band relaxation of n262”, Apple