3GPP TSG-RAN4 Meeting #98-e	R4-2100192

# Online, 25 Jan. – 5 Feb., 2021

Agenda item:	11.4.2.1

Source:	Apple

Title:	On SRS antenna port switching

Document for:	Discussion

# Introduction

In the WID of R17 RRM further enhancement [1], the working scope of SRS antenna port switching is as below,

- SRS antenna port switching [RAN4]

  - Specify RRM requirement for SRS antenna port switching, e.g.

    - Interruption requirement

    - Potential impact on other RRM requirements

In last RAN4 meeting, the work plan has been agreed for this WID, as duplicated below,

  - 3GPP RAN4 #98e meeting (February, 2021, 1.5TU, Core part)

- SRS antenna port switching [RAN4]

  - Initial discussion on Interruption requirement

  - Initial discussion on the impact(s) to other RRM requirements

In this contribution we discuss the preliminary RRM requirements for SRS antenna port switching.

# Discussion on R17 working scope

Since in the WID discussion only NR SRS antenna switching scenario is considered, in the R17 FeRRM WI only RRM requirement of NR SRS antenna port switching would be discussed, that is, no any LTE side SRS switching would be discussed.

Proposal 1: In the R17 FeRRM WI only RRM requirement of NR SRS antenna port switching would be discussed, including following cases:

- NR SRS antenna port switching impacting LTE CC

- NR SRS antenna port switching impacting NR CC

In current RAN4 TS38.133, we have defined interruption requirement for SRS carrier based switching and some of the clarifications are added for the impacting to other RRM requirements, which is like the following wording:

Regarding the SRS antenna port switching, we think the same methodology could be used,

Proposal 2: like carrier based SRS switching, RAN4 would specify the interruption requirements for SRS antenna switching and would add clarification in other RRM requirement for application conditions.

# Interruption requirement for SRS antenna port switching

In RAN2 TS38.306 spec, it was defined that,

When UE support SRS antenna port switching for DL CSI acquisition, it could indicate the capability of the impact to other Rx. And in RAN4 RF spec TS38.101-1/2, the SRS time mask has been defined, in which the uplink transmission on other uplink CCs would also be impacted since the SRS antenna switching without involving carrier frequency change needs to conduct the PA ramp down/up and antenna switching from one port to the other port. In RAN4 RF, only FR1 has the time mask for SRS antenna switching (For sounding different antenna ports, RAN4 has agreed that antenna switching time is 15 us in R4-1710048), but in RAN1 design, the guard period is defined for both FR1 and FR2.

The followings are SRS time mask from RAN4: TS38.101-1

The guard period in RAN1 TS38.214 is defined as below,

In our understanding, RAN1 has chosen to use the most conservative way to define the guard period and no signals would be expected to transmit in guard period. We propose to use the RAN1 guard period as baseline to determine the interruption requirement. However, similar as other RRM interruption requirements, even though the GP is at symbol level, but the interruption requirement shall still be defined in unit of slot for NR and in unit of subframe for LTE.

Proposal 3: use RAN1 defined guard period Y as a baseline for interruption requirement design. And the interruption requirement shall be defined in unit of slot for NR and in unit of subframe for LTE respectively.

With this guard period we also to consider the TA between DL and UL as well as the time difference between CCs in CA. With the TA, the GP might be collided with two DL slots if the victim cell is NR, as shown in figure 1.

Figure 1. interruption to DL due to TA

Regarding the different CA scenarios, in R16 carrier based SRS switching, we have conclusion in R4-2012155 that,

- Interruption requirements for CA case 1, case 2 and case 3

  - The same interruption requirements as for async case in the spec shall apply

Where,

  - Case 1: CA is co-location deployed

  - Case 2: Single TAG CA, or carriers in the same TAG for multiple TAG CA

  - Case 3: uplink time difference does not exceed a threshold X

    - X = [5] us

So here, we think the same principle shall be applied. With those assumptions, we think 2 slots/subframes of interruption could be sufficient for all the SCS cases on victim cell. In addition, the per-FR MG UE capability still needs to be taken into account. In different MR-DC mode, the interruption could be summarized as below,

In EN-DC and NE-DC operation,

- For UE capable of per-FR gap,

  - Interruptions on E-UTRA are allowed due to NR SRS antenna port switching in FR1, but NOT allowed due to NR SRS antenna port switching in FR2.

  - Additional delay can be expected on E-UTRA measurement when UE is configured to perform NR SRS antenna port switching in FR1.

- For UE not capable of per-FR gap,

  - Interruptions on E-UTRA are allowed due to NR SRS antenna port switching in both FR1 and FR2.

  - Additional delay can be expected on E-UTRA measurement when UE is configured to perform NR SRS antenna port switching in FR1 and/or in FR2.

In NR-DC and NR SA operation,

- For UE capable of per-FR gap,

  - Interruptions on NR are allowed if NR SRS antenna port switching happens in the same FR as victim NR CC, otherwise NOT allowed.

  - Additional delay can be expected on a target NR measurement when UE is configured to perform NR SRS antenna port switching in the same FR as this measurement.

- For UE not capable of per-FR gap,

  - Interruptions on NR are allowed due to NR SRS antenna port switching in both FR1 and FR2.

  - Additional delay can be expected on NR measurement when UE is configured to perform NR SRS antenna port switching in FR1 and/or in FR2.

Proposal 4:

For UE capable of per-FR MG, the interruption is only allowed to a victim serving CC if NR SRS antenna port switching happens in the same FR as this victim serving CC.

For UE not capable of per-FR MG, the interruption is always allowed to a victim serving CC regardless of whether NR SRS antenna port switching happens in the same FR as this victim serving CC.

Proposal 5: if NR SRS antenna switching causes interruption to LTE serving CC, the interruption is 2 subframes; while if NR SRS antenna switching causes interruption to NR serving CC, the interruption is 2 slots based on the SCS of the victim CC.

# Conclusion

In this contribution we discuss the preliminary RRM requirements for SRS antenna port switching.

Proposal 1: In the R17 FeRRM WI only RRM requirement of NR SRS antenna port switching would be discussed, including following cases:

- NR SRS antenna port switching impacting LTE CC

- NR SRS antenna port switching impacting NR CC

Proposal 2: like carrier based SRS switching, RAN4 would specify the interruption requirements for SRS antenna switching and would add clarification in other RRM requirement for application conditions.

Proposal 3: use RAN1 defined guard period Y as a baseline for interruption requirement design. And the interruption requirement shall be defined in unit of slot for NR and in unit of subframe for LTE respectively.

Proposal 4:

For UE capable of per-FR MG, the interruption is only allowed to a victim serving CC if NR SRS antenna port switching happens in the same FR as this victim serving CC.

For UE not capable of per-FR MG, the interruption is always allowed to a victim serving CC regardless of whether NR SRS antenna port switching happens in the same FR as this victim serving CC.

Proposal 5: if NR SRS antenna switching causes interruption to LTE serving CC, the interruption is 2 subframes; while if NR SRS antenna switching causes interruption to NR serving CC, the interruption is 2 slots based on the SCS of the victim CC.

# References

[1] RP-202874, Revised	WID of REL-17 NR RRM further enhancement, Apple, RAN #90